|  | Simulation of 6in Bottle Endcap-FEA Date: Tuesday, March 15, 2016 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 9 Study Results 10 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: 6in Bottle Endcap-FEA Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. M4x0.7 Tapped Hole1; Solid Body; Mass:3.80942 kg Volume:0.000844662 m^3 Density:4510 kg/m^3 Weight:37.3324 N; X:\368000_DMO_AUV\Drawings\AUV-3 Reson System\6in Bottle Endcap-FEA.SLDPRT Mar 15 11:01:12 2016.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; On. Inertial Relief:; On. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (X:\368000_DMO_AUV\Drawings\AUV-3 Reson System).

# Units Unit system:; SI (MKS). Length/Displacement; m. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; Commercially Pure CP-Ti UNS R50700 Grade 4 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 5e+008 N/m^2. Tensile strength:; 5.5e+008 N/m^2. Elastic modulus:; 1.05e+011 N/m^2. Poisson's ratio:; 0.37. Mass density:; 4510 kg/m^3. Shear modulus:; 4.5e+010 N/m^2. Thermal expansion coefficient:; 9e-006 /Kelvin.; SolidBody 1(M4x0.7 Tapped Hole1)(6in Bottle Endcap). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Reference Geometry-1; Entities:; 1 face(s). Type:; Use reference geometry. Translation:; ---, ---, 0. Units:; m.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0; 1.09788; 0; 1.09788. Reaction Moment(N.m); 0; 0; 0; 0.. On Cylindrical Faces-1; Entities:; 3 face(s). Type:; On Cylindrical Faces. Translation:; 0, ---, ---. Units:; m.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 9.15527e-005; -159266; 0.00216675; 159266. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 30 face(s). Type:; Normal to selected face. Value:; 9000. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.372246 in. Tolerance; 0.0186123 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 32156. Total Elements; 20706. Maximum Aspect Ratio; 15.385. % of elements with Aspect Ratio < 3; 89.2. % of elements with Aspect Ratio > 10; 0.116. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:07. Computer name:; HUNGA-TONGA.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.00105054; 1.09788; 0.00182243; 1.09788. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 95.4436 psi Node: 5491; 20306 psi Node: 27179. 6in Bottle Endcap-FEA-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 1.39261e-005 m Node: 20615; 2.75089e-005 m Node: 29515. 6in Bottle Endcap-FEA-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.39019e-005 Element: 8924; 0.000988144 Element: 9891. 6in Bottle Endcap-FEA-Static 1-Strain-Strain1.

# Conclusion