|  | Simulation of Cradle Arm Rev AB Date: Thursday, May 16, 2019 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Cradle Arm Rev AB Current Configuration: Default<As Machined>. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Template RECTANGULAR TUBE 5X2X.25(1)[2]; Solid Body; Mass:5.06748 kg Volume:0.000645541 m^3 Density:7,849.98 kg/m^3 Weight:49.6613 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019. Template RECTANGULAR TUBE 5X2X.25(1)[3]; Solid Body; Mass:4.76857 kg Volume:0.000607461 m^3 Density:7,850 kg/m^3 Weight:46.732 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019. Boss-Extrude2; Solid Body; Mass:36.0056 kg Volume:0.0045867 m^3 Density:7,850 kg/m^3 Weight:352.855 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019. Gusset1; Solid Body; Mass:1.15775 kg Volume:0.000147484 m^3 Density:7,850 kg/m^3 Weight:11.3459 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019. Gusset2; Solid Body; Mass:1.15775 kg Volume:0.000147484 m^3 Density:7,850 kg/m^3 Weight:11.3459 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019. End cap1; Solid Body; Mass:0.267327 kg Volume:3.40543e-05 m^3 Density:7,850 kg/m^3 Weight:2.6198 N; C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson\Cradle Arm Rev AB.SLDPRT May 16 15:39:44 2019.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Direct sparse solver. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\PDM_Vault\DMO\Capture Head Boom Crutch Rachel Carson).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; ASTM A36 Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.5e+08 N/m^2. Tensile strength:; 4e+08 N/m^2. Elastic modulus:; 2e+11 N/m^2. Poisson's ratio:; 0.26. Mass density:; 7,850 kg/m^3. Shear modulus:; 7.93e+10 N/m^2.; SolidBody 1(Template RECTANGULAR TUBE 5X2X.25(1)[2])(Cradle Arm Rev AB), SolidBody 3(Template RECTANGULAR TUBE 5X2X.25(1)[3])(Cradle Arm Rev AB), SolidBody 4(Boss-Extrude2)(Cradle Arm Rev AB), SolidBody 5(Gusset1)(Cradle Arm Rev AB), SolidBody 6(Gusset2)(Cradle Arm Rev AB), SolidBody 10(End cap1)(Cradle Arm Rev AB). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-8; Entities:; 8 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.000329018; -0.000343323; -40,034; 40,034. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 9,000 lbf..

# Connector Definitions No Data

# Contact Information Contact; Contact Image; Contact Properties. Global Contact; Type:; Bonded. Components:; 1 component(s). Options:; Incompatible mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.548383 in. Tolerance; 0.0274192 in. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 61824. Total Elements; 30776. Maximum Aspect Ratio; 10.744. % of elements with Aspect Ratio < 3; 72.4. % of elements with Aspect Ratio > 10; 0.0162. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:09. Computer name:; ORIGINATOR.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.000329018; -0.000343323; -40,034; 40,034. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Displacement1; UZ: Z Displacement; -1.925e-04in Node: 51461; 1.957e-02 in Node: 2460. Cradle Arm Rev AB-Static 1-Displacement-Displacement1. Name; Type. Displacement1{1}; Deformed shape. Cradle Arm Rev AB-Static 1-Displacement-Displacement1{1}. Name; Type; Min; Max. Stress1; VON: von Mises Stress; 3.792e+04 N/m^2 Node: 54845; 2.207e+08 N/m^2 Node: 58925. Cradle Arm Rev AB-Static 1-Stress-Stress1. Name; Type. Design Insight1; Design Insight. Cradle Arm Rev AB-Static 1-Design Insight-Design Insight1. Image-1. Image-2. Image-3. Image-4. Image-5. Image-6.

# Conclusion