|  | Simulation of 20190610 Crane Crutch Assembly Date: Monday, June 10, 2019 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: 20190610 Crane Crutch Assembly Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Combine1; Solid Body; Mass:24.6528 kg Volume:0.00314048 m^3 Density:7,850 kg/m^3 Weight:241.598 N; C:\Users\enolasco\Desktop\RC Boom Crutch Atached to ROV Winch Chassis\Crane Parts\Crane Cradle Arm Tubing.SLDPRT Jun 10 16:02:06 2019. Boss-Extrude2[2]; Solid Body; Mass:0.385915 kg Volume:4.91612e-05 m^3 Density:7,850 kg/m^3 Weight:3.78197 N; C:\Users\enolasco\Desktop\RC Boom Crutch Atached to ROV Winch Chassis\Crane Parts\Crutch Base Mount.SLDPRT Jun 10 10:35:49 2019. Boss-Extrude2[1]; Solid Body; Mass:0.385915 kg Volume:4.91612e-05 m^3 Density:7,850 kg/m^3 Weight:3.78197 N; C:\Users\enolasco\Desktop\RC Boom Crutch Atached to ROV Winch Chassis\Crane Parts\Crutch Base Mount.SLDPRT Jun 10 10:35:49 2019. Fillet1; Solid Body; Mass:0.975889 kg Volume:0.00069212 m^3 Density:1,410 kg/m^3 Weight:9.56371 N; C:\Users\enolasco\Desktop\RC Boom Crutch Atached to ROV Winch Chassis\Crane Parts\UHMW Cradle.SLDPRT Jun 10 11:23:52 2019.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Mixed Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Direct sparse solver. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\enolasco\Desktop\RC Boom Crutch Atached to ROV Winch Chassis\Crane Parts).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; ASTM A36 Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.5e+08 N/m^2. Tensile strength:; 4e+08 N/m^2. Elastic modulus:; 2e+11 N/m^2. Poisson's ratio:; 0.26. Mass density:; 7,850 kg/m^3. Shear modulus:; 7.93e+10 N/m^2.; SolidBody 1(Combine1)(Crane Cradle Arm Tubing-1), SolidBody 1(Convert-Solid1)(Crane Foot Attachment-1), SolidBody 1(Fillet1)(Crutch Base Mount-1), SolidBody 2(Boss-Extrude2[2])(Crutch Base Mount-1), SolidBody 3(Boss-Extrude2[1])(Crutch Base Mount-1). Curve Data:N/A. Name:; Delrin 2700 NC010, Low Viscosity Acetal Copolymer (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 6.3e+07 N/m^2. Tensile strength:; 4.07e+07 N/m^2. Elastic modulus:; 2.9e+09 N/m^2. Poisson's ratio:; 0.3. Mass density:; 1,410 kg/m^3.; SolidBody 1(Fillet1)(UHMW Cradle-1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-2; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -33,025.8; 59,167.6; 0.00978063; 67,760.7. Reaction Moment(N.m); 0; 0; 0; 1e-33.. Load name; Load Image; Load Details. Force-1; Entities:; 2 face(s). Type:; Apply normal force. Value:; 16,683 lbf..

# Connector Definitions No Data

# Contact Information Contact; Contact Image; Contact Properties. Global Contact; Type:; Bonded. Components:; 1 component(s). Options:; Compatible mesh..

# Mesh information Mesh type; Mixed Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Jacobian check for shell; On. Element Size; 0.555272 in. Tolerance; 0.0277636 in. Mesh Quality Plot; High. Remesh failed parts with incompatible mesh; Off. ## Mesh information - Details Total Nodes; 38925. Total Elements; 19586. Time to complete mesh(hh;mm;ss):; 00:00:11. Computer name:; ORIGINATOR.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -33,025.8; 59,167.6; 0.00978063; 67,760.7. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.327e-08 N/mm^2 (MPa) Node: 22907; 6.522e+02 N/mm^2 (MPa) Node: 28546. 20190610 Crane Crutch Assembly-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; UZ: Z Displacement; -9.346e-03in Node: 35148; 1.454e-02 in Node: 35617. 20190610 Crane Crutch Assembly-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 6.950e-14 Element: 11288; 7.325e-03 Element: 18515. 20190610 Crane Crutch Assembly-Static 1-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. 20190610 Crane Crutch Assembly-Static 1-Displacement-Displacement1{1}. Name; Type; Min; Max. Factor of Safety1; Automatic; 3.833e-01 Node: 28546; 1.884e+10 Node: 22907. 20190610 Crane Crutch Assembly-Static 1-Factor of Safety-Factor of Safety1. Name; Type. Displacement2; Deformed shape. 20190610 Crane Crutch Assembly-Static 1-Displacement-Displacement2. Name; Type. Design Insight1; Design Insight. 20190610 Crane Crutch Assembly-Static 1-Design Insight-Design Insight1. Name; Type; Min; Max. Stress2; P1: 1st Principal Stress; -5.688e+07N/m^2 Node: 28541; 3.498e+08 N/m^2 Node: 3859. 20190610 Crane Crutch Assembly-Static 1-Stress-Stress2. Name; Type. Design Insight2; Design Insight. 20190610 Crane Crutch Assembly-Static 1-Design Insight-Design Insight2.

# Conclusion