- Overview

This README is intended to establish and inform, the specifics of how to organize the AUV SolidWorks Models and Drawings.

- How to Name

  - Description Title

If it is a single part, it should be short description, basic name.

    - Example:  MHDL-16 CCP to NetG CCP, CABLE

  - If you’re making a distinct assembly and have different components as part of it

    - AUV TAG LINE HANDLE

      - AUV TOW LINE HANDLE, HANDLE

      - AUV TOW LINE HANDLE, BASE

    - EK80 ECHOSOUNDER, BRACKET

    - PMEL, BRACKET

      - PMEL, HOUSING

    - 30 IN MIDSECTION TOP, RAILING

    - 30 IN MIDSECTION BOTTOM, RAILING

    - Examples of unacceptable names:

      - I2MAP, BRACKET

      - MIDSECTION, BRACKET

  - SolidWorks Model

    - H-100221350 AUV TOW LINE HANDLE

    - H-100221351 AUV TOW LINE HANDLE, HANDLE

  - Drawings

    - H-100221351 AUV TOW LINE HANDLE, HANDLE_RevB.PDF

  - MasterCam File

- How to File

\\atlas\ProjectLibrary\368000DMO_AUV\documents\AUV_SOLIDWORKS_LIBRARY\

  - ASSEM AND PART LIBRARY

  - GULPER

  - I2MAP

  - MAPPER

  - TAILCONE

  - If the part is altered specifically for the vehicle, add it to Configurations under the vehicle name that is altered

- Symbols

| Symbol | Category | Examples |
| --- | --- | --- |
| C | Circuit Boards | MVC, Battery Board, |
| CE | Batteries and Cables | Batteries, Bus, |
| F | Fairing-Related Hardware | Fairings, Fairing Handles, Lifting, Railing |
| H | In-House Hardware | Brackets, Custom Housings, Guards, Tail Cone Railings, |
| M | Motors, Actuators, Props | Prop, Prop Guard, Actuator, |
| N | Navigation | Antenna, DVL, |
| O | Off the Shelf Hardware | Connectors, Bolts, Nuts, Couplings, Housings, Glass Spheres |
| S | Science Sensors | Transducers, CTD, |
| SH | Science Mechanisms | Gulpers |
| T | Tooling and Jigs | Fairing Hole Tool, Gulper Cocker |

- MBARI Numbers

  - How to Obtain a number

  - Go to the MBARI Canyon head (mww.mbari.org) and click on “Engineering Drawings”

  - Click “Get a New Drawing Number”

  - Fill out “GET A NEW DRAWING NUMBER” form. If applicable, start with Highest Assembly first,

    - Drawing TITLE

    - Designer Name

    - Project Number

    - Revision number A to Z

    - Software Program

      - The options are

        - Solidworks

        - AutoCAD

    - File Extension Type:

    - Location of Software Program File Path

    - Description Line 1 (Limit 50 Characters):

    - Description Line 2 (Limit 50 Characters):

    - Next Higher Assembly (Limit 50 Characters):

    - Review and click “Submit”

  - Get

- Library of common DESCRIPTIONS