Overview

Currently, the tag line handles are made out of aluminum and are surface mounted. During the R/V Kilo Moana Trip 2018, during a launch one of the handles broke. During a later 2018 R/V Falkor mission multiple tag handles also failed in a small vessel tow recovery in addition to one of the front bale. All handles and bales are aluminum.  All failures are due to material stress failure.

Further constraints and knowns

- The tag line handles need to be rounded to prevent tears in lines and back-gating in clips.

- The vehicles weigh about 2 tons.

- These tow handles and bale are not to be lifting points for the vehicle.

- The tag line handles should have the same hole pattern unless the change is necessary

- The current hardware is ¼-20 and the handle holds the thread.

- The hardware cannot stick out into the line area-this will prevent snags.

- Aluminum is flexible, soft, and light however

New Design

While the tag line handles could be incredibly strong but the weakest point will be the ABS frame.

Will be made with Grade 2 Titanium, has great weldability and formability. It’s necessary that we can bend the rod.