|  | Simulation of LiftingBailPullTestMount Date: Wednesday, January 22, 2014 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 4 Mesh Information 5 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: LiftingBailPullTestMount Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet1; Solid Body; Mass:2.00512 kg Volume:0.000260405 m^3 Density:7700 kg/m^3 Weight:19.6502 N; Y:\Solid Models\LiftingBailPullTestMount.SLDPRT Jan 22 10:57:55 2014.

# Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 6.20422e+008 N/m^2. Tensile strength:; 7.23826e+008 N/m^2.; SolidBody 1(Fillet1)(LiftingBailPullTestMount).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 4 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Top Plane. Type:; Apply force. Values:; ---, ---, 3000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.251479 in. Tolerance; 0.012574 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 15150. Total Elements; 9439. Maximum Aspect Ratio; 4.2454. % of elements with Aspect Ratio < 3; 99.5. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 10944.2 N/m^2 Node: 12712; 3.34453e+007 N/m^2 Node: 13677. LiftingBailPullTestMount-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 1; 0.00623244 mm Node: 888. LiftingBailPullTestMount-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. LiftingBailPullTestMount-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 18.5503 Node: 13677; 56689.5 Node: 12712. LiftingBailPullTestMount-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion