|  | Simulation of Fugro_hook Date: Wednesday, October 31, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Fugro_hook Current Configuration: Long Version. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet3; Solid Body; Mass:0.490564 kg Volume:0.000110767 m^3 Density:4428.78 kg/m^3 Weight:4.80753 N; Y:\Solid Models\Mapper Recovery Hook\Fugro_hook.SLDPRT Oct 31 07:43:57 2012.

# Material Properties Model Reference; Properties; Components. Name:; Ti-6Al-4VSolution treated and aged (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 8.27371e+008 N/m^2. Tensile strength:; 1.05e+009 N/m^2.; SolidBody 1(Fillet3)(Fugro_hook).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Type:; Apply normal force. Value:; 2000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.189088 in. Tolerance; 0.00945438 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 12256. Total Elements; 7364. Maximum Aspect Ratio; 10.923. % of elements with Aspect Ratio < 3; 99.7. % of elements with Aspect Ratio > 10; 0.0679. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 2.61581 N/m^2 Node: 8550; 2.67608e+008 N/m^2 Node: 12093. Fugro_hook-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 1; 0.937684 mm Node: 110. Fugro_hook-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. Fugro_hook-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 3.09172 Node: 12093; 3.16296e+008 Node: 8550. Fugro_hook-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion