|  | Simulation of Fugro_hook_with_snapgate Date: Tuesday, November 13, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Fugro_hook_with_snapgate Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line3; Solid Body; Mass:1.18824 lb Volume:7.4265 in^3 Density:0.16 lb/in^3 Weight:1.18743 lbf; Y:\Solid Models\Mapper Recovery Hook\Fugro_hook_with_snapgate.SLDPRT Nov 13 11:48:05 2012.

# Material Properties Model Reference; Properties; Components. Name:; Ti-6Al-4VSolution treated and aged (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 120000 psi. Tensile strength:; 152290 psi.; SolidBody 1(Split Line3)(Fugro_hook_with_snapgate).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Plane2. Type:; Apply force. Values:; ---, ---, 2000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.195159 in. Tolerance; 0.00975797 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 14565. Total Elements; 8811. Maximum Aspect Ratio; 68.652. % of elements with Aspect Ratio < 3; 97.6. % of elements with Aspect Ratio > 10; 0.193. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 0.000227263 psi Node: 6885; 23990.9 psi Node: 123. Fugro_hook_with_snapgate-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 108; 0.533422 mm Node: 84. Fugro_hook_with_snapgate-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. Fugro_hook_with_snapgate-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 5.0019 Node: 123; 5.28023e+008 Node: 6885. Fugro_hook_with_snapgate-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion