|  | Simulation of SeaCatchCopy3 Date: Wednesday, October 31, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: SeaCatchCopy3 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet3; Solid Body; Mass:1.78493 lb Volume:11.1558 in^3 Density:0.16 lb/in^3 Weight:1.78372 lbf; Y:\Solid Models\Mapper Recovery Hook\SeaCatchCopy3-Thick.SLDPRT Oct 31 10:33:51 2012.

# Material Properties Model Reference; Properties; Components. Name:; Ti-6Al-4VSolution treated and aged (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 120000 psi. Tensile strength:; 152290 psi.; SolidBody 1(Fillet3)(SeaCatchCopy3).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-3; Entities:; 1 face(s), 1 plane(s). Reference:; Top Plane. Type:; Apply force. Values:; ---, ---, -2000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.169793 in. Tolerance; 0.00848964 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 25369. Total Elements; 16244. Maximum Aspect Ratio; 31.662. % of elements with Aspect Ratio < 3; 99.7. % of elements with Aspect Ratio > 10; 0.0554. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 0.000863328 psi Node: 19109; 23767.6 psi Node: 23372. SeaCatchCopy3-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 151; 0.529241 mm Node: 19450. SeaCatchCopy3-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. SeaCatchCopy3-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 5.0489 Node: 23372; 1.38997e+008 Node: 19109. SeaCatchCopy3-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion