|  | Simulation of SeaCatchCopy3 Date: Wednesday, October 31, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: SeaCatchCopy3 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:1.9289 lb Volume:12.0556 in^3 Density:0.16 lb/in^3 Weight:1.92759 lbf; Y:\Solid Models\Mapper Recovery Hook\SeaCatchCopy3.SLDPRT Oct 31 10:25:24 2012.

# Material Properties Model Reference; Properties; Components. Name:; Ti-6Al-4VSolution treated and aged (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 120000 psi. Tensile strength:; 152290 psi.; SolidBody 1(Split Line1)(SeaCatchCopy3).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-3; Entities:; 1 face(s), 1 plane(s). Reference:; Top Plane. Type:; Apply force. Values:; ---, ---, -2000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.169793 in. Tolerance; 0.00848964 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 26823. Total Elements; 17270. Maximum Aspect Ratio; 4.283. % of elements with Aspect Ratio < 3; 99.7. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 0.000427675 psi Node: 513; 22367.7 psi Node: 24649. SeaCatchCopy3-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 153; 0.499313 mm Node: 23687. SeaCatchCopy3-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. SeaCatchCopy3-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 5.36488 Node: 24649; 2.80587e+008 Node: 513. SeaCatchCopy3-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion