|  | Simulation of SnapGateHook1 Date: Thursday, November 08, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: SnapGateHook1 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Extrude2; Solid Body; Mass:2.8342 lb Volume:17.7138 in^3 Density:0.16 lb/in^3 Weight:2.83228 lbf; Y:\Solid Models\Mapper Recovery Hook\SnapGateHook1.SLDPRT Nov 08 12:34:55 2012.

# Material Properties Model Reference; Properties; Components. Name:; Ti-6Al-4VSolution treated and aged (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 120000 psi. Tensile strength:; 152290 psi.; SolidBody 1(Cut-Extrude2)(SnapGateHook1).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Top Plane. Type:; Apply force. Values:; ---, ---, -2000 lbf..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.263339 in. Tolerance; 0.013167 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 12139. Total Elements; 7291. Maximum Aspect Ratio; 4.4257. % of elements with Aspect Ratio < 3; 99.6. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; DEUTSCHLAND.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 0.000295694 psi Node: 10342; 23160.3 psi Node: 11571. SnapGateHook1-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 113; 1.01509 mm Node: 727. SnapGateHook1-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. SnapGateHook1-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 5.18127 Node: 11571; 4.05826e+008 Node: 10342. SnapGateHook1-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion