5/17/2019 AUV Group Meeting

Upcoming Dives

May 21/22 – I2MAP (scrubbed due to crane failure)

June 3/4 – I2MAP at MARS (still possible if crane part delivery best case)

June 6/7 – I2MAP day run

June 13 – TRN Testing

June 25-28 – Ocean Imaging on Western Flyer (INS, Multibeam, possibly Edgetech SBP)

Wind Farm Cruise Last Week

- 4 dives completed between mapper 1 and mapper 2

- SVP logging, high speed sound velocity updates, direct edgetech control worked properly

- Mapper 1

  - No major issues

- Mapper 2

  - DVL generating erroneous velocities at high altitude during descent caused navigation jumps (compare DVL settings between vehicles)

  - Normalized backscatter didn’t seem to be working

CTD

- No runs planned

Vehicle Status

- Start charging up all batteries today (mapper 2 in B137, mapper 1 & CTD in Bay 4)

- I2MAP

  - Ethernet connection still a problem (talk to simrad about this)

  - Noise issue now present when lights are turned off. Need to troubleshoot with camera open and resolve. Noise baseline taken on Tuesday

  - 4K video looks sharp, still evaluating

  - Camera is off vehicle and opened up for switch back to standard resolution

- Mapper 2

  - Tear down and inspect, get projector serial number

  - Dump DVL settings to try and resolve INS behavior

  - Bring INS up to latest firmware revision

  - Make up spare  cable between edgetech & battery (mcil8f to mcil8m)

  - Make up spare cable between reson and battery (lpdil13M to mcil13f)

  - Move pressure sensor, change pressure sensor to 4000m unit

- Mapper 1

  - Tear down and inspect, get projector serial number

  - Adjust fastners in mapper 1 to eliminate cable chafe issues

  - Pack 49 in Forward battery pack not discharging

  - Bring INS up to latest firmware revision

  - Move pressure sensor, change pressure sensor to 4000m unit

- CTD vehicle

  - Wire in avtrak beacon to mvc sphere for comms and power

- All Tails

  - Change out iridium beacons for SBD warriors

- I2MAP

  - Troubleshoot light related noise

  - Improve mechanical arrangement for cable routing

  - Change camera back to standard configuration

- Other projects

  - Boom crutch – status

  - INS isolation board (duane)

    - Board ready for fab, 5 day turn at PCBWAY due in 5/28 or 5/29, built up by June 7

    - Work on harness drawing after board out fab

  - Picking up vehicle with other crane

    - Take spare rigging with us on the Carson