# EdgeTech Sonar Interface Protocol

Ref.: Sonar-AUV-SDK\Docs\SonarMessages.h

# Introduction

EdgeTech Sonar can control up to three subsystems as shown in Figure 1: Side Scan Low Frequency (SSL), Side Scan High Frequency (SSH), and Sub-Bottom (SB).   Not all subsystems may be present within a particular configuration.  Each subsystem can be controlled individually (i.e. independently) of the others.  (Note:  Synchronized ping timing may be controlled within a Master-Slave relationship).

# Sonar Communications

- Two channels of communication via Ethernet connection (TCP/IP):
          Command channel - Send commands & receive replies
          Data channel - Receive sonar data and auxilliary (navigation/sensor) data

- EdgeTech Sonar acts as the server and its IP address is 192.9.0.101

- The port numbers are 1700 for command and 1701 for data.

Figure 1 - Block diagram of EdgeTech Sonar with Ethernet connection to Sonar Control

# Sonar Message Protocol

## 16-byte Message Header

All messages on both the command and data channels begin with a 16 byte message header.  This header describes the type and length of the message along with other information as shown in Table 1.

| Byte Offsets | Size | Description | Reference |
| --- | --- | --- | --- |
| 0-1 | UINT16 | Marker for start of header (always 0x1601) |  |
| 2 | UINT8 | Version number of the EdgeTech sonar protocol used |  |
| 3 | UINT8 | Session Identifier |  |
| 4-5 | UINT16 | Message Type (e.g. 80 => sonar trace data) | Table 2 |
| 6 | UINT8 | Command type (e.g. Get, Set, or Reply) | Table 3 |
| 7 | UINT8 | Subsystem ID 0 = Sub-Bottom 20 = Single Frequency or Lower Frequency SideScan 21 = Higher Frequency SideScan |  |
| 8 | UINT8 | Channel Number for multi-channel subsystems For sidescan subsystems: 0=Port, 1=Starboard For serial ports: Port Number |  |
| 9 | UINT8 | Sequence Number |  |
| 10-11 | UINT16 | Reserved |  |
| 12-15 | UINT32 | Size of the following message in bytes |  |

Table 1 - Layout of the 16-byte Message Header

### Detailed description of message header entries

Byte offsets 0-1: Marker for start of header

The marker for the start of header is a 2-byte integer and is always equal to 0x1601.  This marker helps in the identification and verification of a message header.

Byte offset 2: Version number of the EdgeTech sonar protocol used

The value of this field identifies the version number of the sonar that is in use and is used for backward compatibility.  Messages of differing protocol levels may be interspersed in the same file.

Byte offset 3: Session Identifier

This field is used for internal routing and can be ignored.  When composing (i.e.  sending) a message, a value of zero should be used here.

Byte offsets 4-5: Message Type

The value within this field is a 2-byte unsigned integer that identifies the type of message that follows the header.  See Table 2 for a description of the types of messages that are available.  If when receiving data an unrecognized value (i.e.  a value that is not listed in table 2) is encountered, then the remainder of the message (for which the length is specified in bytes 12-15) should be ignored.