Procedure:

- Bring vehicle on board

- Proceed with shutdown procedure on AUV computer

  - Check to see if mission is still running

$ ps -ef   //this checks to see if the mission is still running

//if you see a process called supervisor still running, then you do the following

$ slay supervisor

  - Check if recordings give indication of being successful

Check on browser 134.89.32.54 and click on "All Clips", makes sure you have 14 videos 6-8 minutes long

  - Shut down camera system gracefully

$ carlTest

6  //Shut off recorder

3  //Shut off camera

4  //Check status of Camera to make sure it is off, 0 indicates off, 1 indicates on

7  //Check status of the recorder to make sre it is off

1 //This exits the carlTest

  - Check environmental monitoring

$ fanTest

r

  - Check and turn off battery

telnet 134.89.32.121 10003

@99AF //turns off battery

- Connect vehicle to tether

  - Check if power supplies are off and that the voltage and current are setup correctly V<36 & I>10A

  - Attach Tether to vehicle, then turn power supply on

- Log back onto the vehicle

$ telnet mvc-dmo1

- Check Camera system

$ carlTest

//check on browser 134.89.32.54 and click on "All Clips", makes ure you have 14 videos 6-8 mintues long

6 //turn recorder off

3 // turn off camera (should be off already)

7 //check recorder, =0

4 //check camera, = 0

1 //exit

- Download log files

Note: Color indicates different Cygwin Windows

$ echo $AUV/logs //enter, this shows you the path of the log files

Open a second Cygwin terminal

//go to the directory that you want the logs to be downloaded into

$ ncftp -u dorado1 mvc-dmo1

ncftp /home/dorado1 > cd //insert directory that came from "echo $AUV.."command

ls //find the latest directory Ex:2018.065.02/

get -r //insert the latest directory

Close out

- Center Tail cone

$ td newTailCone //we need to center the tail cone duct

e 0

r 0

Close Out

- Turn off Tracking on Ranger 2

  - This prevents the system from just sitting and pinging in the harbor until someone notices.

  - Note: When your done in the control room please shut down power to the racks, turn off the overhead lights and close the control room door.