| Date: | Aug 20, 2020 |
| --- | --- |
| Vehicle: | CTD |
| Location: | Between C1 & M1 |
| Major Objective: | VN100 Calibration & Eval Dive |
| Dive Plan: | Calibration dive, followed by overnight dive |

Accomplishments:

VN100 Calibration dive was successful, with acceptable calibration values returned.

VN100 Overnight dive was also successful, running an overnight diamond pattern with 50m depth legs of 3.5 and 5 km, followed by 10-90m yoyo legs of 3.5 and 5 km.

VN100 Specific report-back is in the folder named `20200820`

Vehicle Issues:

Need to create Ahrs taskIF that accepts ‘startCalibration()’ requests.

Note(s):

Copied Summary slide from VN100 report-back:

- VN100 performance within acceptable bounds.

  - Cross track error in the range of 2-11 %DT without water current velocity estimation, or DVL-VN100 alignment calibration (room for improvement)

- More stable heading during yoyo compared to MicroStrain.

  - Should lead to less rudder actuation effort and “wobble”

- Calibration routine provides decent figures of merit, as confirmed by norm of magnetic vector

- Remaining work:

  - Implement Start/stop calibration in AHRS taskIF

  - Perform DVL/VN100 alignment run.