| Date: | October 8, 2020 |
| --- | --- |
| Vehicle: | CTD |
| Location: | Monterey Bay |
| Major Objective: | MBTS Diamond |
| Dive Plan: | MBTS Diamond Pattern, gulp locations provided by M. Blum |

Accomplishments:

New Brushless DC (BLDC) tailcone was run successfully on this mission.

No notable oil loss from tailcone comp on recovery

Mission completed

Vehicle Issues:

Vehicle speed appeared slightly slow (2 kn), when it should have been closer to 3 kn.

Hans adjusted propServoRate parameter in bldcTailCone.cfg from 40 to 127, hopefully resolving this issue.

Note(s):

Vehicle likely saw some kelp entanglement near Santa Cruz, as indicated by extended surface interval and slow southward progress.