| Date: | 22-23Sep 2020 |
| --- | --- |
| Vehicle: | I2Map |
| Location: | 36.6969, -122.0470 |
| Major Objective: | Overnight Transect Mission |
| Dive Plan: | Deimos_Transect_202000922.cfg |

Accomplishments:

- Vehicle started night dive set at 22:55L (22:30L was desired night transect timeout).

- No timeouts or spurious log entries from CARL Server.

- No CTD dropouts, and PAR data contained fully in seabird25p.log

Vehicle Issues:

- Required second mission launch, as exception was thrown from DVL driver, with the following syslog error, 

“: dvl - caught Exception: Buffer Full”

Note(s):

- Vehicle was rinsed and main batteries charged while on deck.

- Oil comp was empty on recovery.