- Vehicle Operations Checklist

  - Pre-mission Checklist

  - Charge batteries (see battery charge directions in: \\atlas\368000_DMO_AUV\manuals\002_OPS_MANUALS\InspEngBatts\

  - All tether connections are made and all ends capped with Dummy Plugs

  - Inspect tailcone hardware

    - Actuator bracket bolts tight

    - Duct bolts tight

    - Actuator cotter pins installed

    - Oil comp charged

  - Foam installed

  - Prop tight

  - Dropweight present

  - Fairings secured

  - Rings secured

  - Dunk in Test Tank to check to ballast

  - Strobe tight

  - Magnet removed

  - Pressure switch activated

  - RDF beacon audible on the receiver

  - Strobe flashing

  - Launch Checklist

| Power AUV on deck; telnet in | telnet mvc-dmo1 OR telnet mvc-dmo1-rf (if on freewave) |
| --- | --- |
| Run dropweight server from a laptop | resetdw |
| Ensure mission .profile -> .profile_auvdev | cd ~ rm .profile ln –s .profile_auvdev .profile ls –l .pro* (to confirm link) |
| Confirm Battery names match the Config file | cd $AUV_CONFIG_DIR vi devices.cfg ( opens up vi editor) |
| Get MVC sphere environmentals | fanTest <r> |
| Check paroscientific function | td parosci |
| Check compass function | td compass |
| Check GPS function (the vehicle’s antenna needs to be outside ) | td gps |
| Check DVL function | td dvl |
| Check Tailcone function | td newTailCone |
| Check Free Space on MVC | df –h |
| Check battery comms (FWD & AFT) | telnet 134.89.32.XXX @99RQ <cr> |
| Enable battery discharge | @99AD <cr> |
| Turn on BioAcoustics Sphere |  |
| Run carlTest | carlTest MAKE SURE BATTERIES ARE DISCHARGING BEFORE TESTING LIGHTS |
| Check Iridium comms | qtalk –m /dev/ser4 –b 2400,8,n,1 cmd:1 <cr> (resets counter) Watch for GPS message <ctrl-A> exit |
| Camera Cover Off |  |
| CTD Tubing Off |  |

  - Run Mission

| Check Mission variable times |  |
| --- | --- |
| Ask Captain for heading and input into Mission |  |
| Save program |  |
| Notify Captain before running mission |  |
| Send mission from deployment laptop | cd /cygdrive/c/Dorado/missions run_mission <missionName.cfg> |
| Watch syslog for confirmation | telnet mvc-dmo1-rf tail cd $AUV/logs/latest tail -f syslog |
| Wait for RDF to fade |  |
| (If applicable) Have the boat follow the vehicle for a bit to ensure that it’s running properly |  |

Emergency abort sss on any Cygwin window

  - Recovery Checklist

| Run dropweight server from a laptop | resetdw |
| --- | --- |
| Slay supervisor | sss |
| Telnet onto the vehicle and verify supervisor has been slayed | telnet mvc-dmo1-rf |
| Get vehicle onto the boat |  |
| Magnet In/CTD Tubing On/ Camera Cover on |  |
| Open Vehicle to have the Science Party Extract their data |  |
| Download Vehicle Data and deliver to Science Party | cygwin cd /cygdrive/c/dorado/data ncftp -u dorado1 mvc-dmo1 cd auv-qnx/auv/altex/onboard/logs get -r (data) (eg: data 2015.148.01) |
| Review vehicle data | matlab cd \dorado\data\(data) loadData plotSwSci =2 review |
| Install Light Bar couplers |  |
| Once all data has been downloaded, turn off power supplies |  |
| Remove deck tethers |  |

  - Post Dive Checklist

| Reset Dropweight | resetdw |
| --- | --- |
| Rinse |  |
| Rinse CTD tubes and install hose |  |
| Inspect tail cone |  |
| Inspect sphere and check the pressure |  |
| Charge Batteries |  |

## Operations Schedules for I2MAP

  - Day Trip

| 2 Day | 1 Day | 0 Day | -1 Day |
| --- | --- | --- | --- |
| Charge Batteries (~5 hours) | - Swap over Tail Cone and mvc from Gulper Vehicle - Pre Mission - Test Tank - Ready for the Rachel Carson to Crane onto Boat - Set up Laptop and FreeWave | - Pre-Dive - Launch - Recovery | - Post Dive |

  - Overnight Trip

| 2 Day | 1 Day | 0 Day | -1 Day | -2 Day |
| --- | --- | --- | --- | --- |
| Charge Batteries (~5 hours) | - Swap over Tail Cone and mvc from Gulper Vehicle - Pre Mission - Test Tank - Ready for the Rachel Carson to Crane onto Boat - Set up Laptop and FreeWave | - Pre-Dive - Launch | - Recovery - Post Dive | Post Dive (if not already completed) |

- Tools, Lab, and Test Equipment

  - Comps

  - Before adding any oil to comps, make sure there is no water in the Quick Disconnect, and no air in the line of the comp pump.

  - Tailcone Comp reservoir plug should extend 2 ½ to 3 inches.

  - Comp fluid is Tellus 22

  - Motors

- The motor oil compensator is located on the starbord side of the tail cone, accessible through the access door.

- The compensator should have the piston extended about 3 1/2 fingers, close to but not touching the fairing.

- Motor Comp fluid is Tellus 22.

- Make sure the following tight:

  - Actuator bracket bolts tight

<Insert Picture Here>

  - Duct bolts tight

<Insert Picture Here>

  - Actuator cotter pins installed

<Insert Picture Here>

  - Oil comp charged

<Insert Picture Here>

  - Dummy Plugs

- Rubber: Pull plugs straight out of jacks.  Use Molykote 44 medium before re-inserting connections.

- Metal: The only plug that is installed and removed regularly is the tether (no spray)

  - Floatation

- Camera

- Mid Body

- Aft Block

- Aft Half Round

- Tail Cone Bottom

- Tail Cone Top

<Insert Picture Here>

  - Sphere Vacuum

- Check sphere with fairings off for damage

- Pressure gauge inside sphere should read 3+ psi

- Make sure that all penetrators that are not being used are dummied off!!

  - Instrument nose

Visually inspect instruments

  - Check Ballast

Refer to the spreadsheet:

\\atlas\ProjectLibrary\368000_DMO_AUV\manuals\Ballasting\AUV_Ballasting_Calcs.xls

  - Batteries Charged

See the battery section.

  - Test Tank

    - Ensure the vehicle has all foam pieces and all connections are either connected or dummied off.

    - Make sure the vehicle's fairings are fully buttoned down.

    - Strap the AUV to the cart and bring it around to the test tank door

    - Lift with 6’ lifting strap and pear on lifting bail, single point lift.

    - Check to make sure the tether is dummied off place before putting AUV in the tank.

    - Put AUV in the tank with slack on the lifting strap; should float level in the water.

    - Tie line to lifting bail, put on proof weight bag (approx 18 pounds in tank weight)

    - Position AUV near the bridge, let out enough line for AUV to be suspended about 8 feet beneath the surface.

    - Use a pole to “burp” out as much air as possible from the AUV.

    - Top: Suspend the line from the fish scale and record the weight, subtracted from the reading of the proof weight.

    - Next, position the AUV close to the bottom of the tank while keeping the weight off the bottom.

    - Bottom: Suspend the line from the fish scale and record the weight, subtracted from the reading of the proof weight.

    - Multiply the difference between the two calculated weights by 2, and subtract from the “top” calculated weight.

    - The answer should be about 5.75 to 7.75 pounds.

    - Remove AUV from the tank and rinse with fresh water.

    - Record Measurements on Ballast Spreadsheet

- Loading and Setup

## Bring AUV to the Rachel Carson.

Use three straps to tie AUV to cradle

Be careful about the bumps crossing the street; AUV is top-heavy.

## Lifting Slings and tag lines set up

6 foot sling and pear in lifting bail.

<Insert Picture Here>

## Lift AUV with cradle

Transport cradle is the deck cradle

## Crane to Rachel Carson

Dock manager, Erik Fitzgerald or Carson crew will handle crane

## Tie AUV down to the deck

    - Set up deck eye bolts

    - Put cloth strap guards on each side of the cart

    - Tie down with two straps FWD and two straps AFT

    - <Insert Picture Here>

## Setup laptop in Lab

- Set up “mission control” laptop ( HILLARY ) by connecting battery and Ethernet to AUV

- Set up FreeWave

- <Insert Cabling Diagram Here>

## Tether on AUV

- Handle connectors carefully.  Be sure O-rings are intact.

- Carefully dry off endcaps and cables before making connections/disconnections

## Deck Checkout

Refer to the spreadsheet:

\\atlas\ProjectLibrary\368000_DMO_AUV\manuals\002_OPS_MANUALS\I2MAP_AUV OPS\I2MAP Mission Checklist.xlsx

## Telnet Commands

  - To Switch Profiles Between auv-qnx (CTD Gulper mode) and auv-qnx-dev (I2Map)

In /home/dorado1 rm .profile to remove the current profile, then choose on from below

ln –s .profile_auv .profile   to link the CTD Gulper profile

ls –l .profile   check to confirm

If the profile is changed, you should restart the vehicle for new settings to take full effect.

### Change directory:

cd $AUV_CONFIG_DIR

Check devices.cfg

(vi devices.cfg if necessary to make changes)

## Check abortMplan.cfg

cd $AUV_PLAN_DIR

more abortMplan.cfg

unattended	2h	=	7200 sec

unattended	1h	= 	3600 sec

attended	15m	=	 900 sec

### td newTailCone

Make sure no one is working near the tailcone.

r 10  r -10  r 0		(rudder)

e 10 e -10  e 0		(elevator)

p 100  p 0		            (prop)

Ctr+c  			(exit)

### td compass  (Microstrain AHRS Compass)

m3dmgx1Test –dev /dev/ser5

Examine values (should be near level, temp is for inside MVC Sphere)

### td parosci	      (Paroscientific pressure sensor)

Check pressure and temperature

IF Externally Triggered, run command:

trigger -dev /dev/ser1

Before starting td dvl

### td dvl	 (Doppler Velocity Log)

Check range, pitch, and temp

### td gps         (Global Positioning System)

Run for a while,  do several Ctr+c, then date in rapid succession

If gps info keeps scrolling, type “slay ashtechDriver” <cr>

Check date time against gps time

If time is off:

Type “su” to enter superuser mode

Type “settime” enter time

Type “exit” to exit superuser mode

### fanTest	(Environmental Fan Board in MVC sphere)

<r> option reads values

( If fanTest will not run:

su	(become root)

cd $AUV/bin

chown root fan*

chmod +s fan*

exit	)

  - Test Iridium

qtalk –m /dev/ser4 –b 2400,8,n,1

cmd:1 <cr>  (resets counter)

Watch for GPS message

### Test Drop Weight

qtalk –m /dev/ser13

!d	drop the weight

!r	reset dropweight

### How to drop the weight

### To talk to the PIC

qtalk –m /dev/ser11

### To check SBD

Check your email

### To Send SBD (Short Data Burst) from Vehicle

qtalk –m /dev/ser1

AT+SBDI

--or—

Cd $AUV/bin

iridiumDriver –dev /dev/ser1

<ctrl-C> to end

## Launch

Check Tailcone Comp

Make sure fairings and rings are secure

Make sure all doors are secure

See that the Strobe is On

Check RDF

See that the PTT Magnet is removed

Check to lift bail latch for free play

Straighten out Tail Cone <td newTailCone>

Examine ping window to make sure comms are up and working

Unstrap AUV from cradle

Boat crew will get AUV in the water.

Let it soak, then do fanTest

Kill fanTest

Do a “ps” to make sure no test programs are running

## Run Mission

With the Captain’s Permission:

From a cygwin window

cd /cygdrive/c/Dorado/Missions

run_mission <mission_name.cfg>

Alternative method:

Use ncftp to copy the compiled mission file into:

/home/dorado1/auv-qnx/auv/altex/onboard/missions

Then give command:

nohup supervisor –plan <mission_name.cfg.pp> &

In another cygwin window:

cd $AUV/logs/latest

tail –f syslog

run_mission script notes:

- sed ( stream editor ) strips comments

- cpp ( macro replacement )  The .cfg file compiles to a .cfg.pp file.  The macro replacement (via the C pre-processor) happens on the laptop, not on the MVC. This pre-processed output is stored in the .cfg.pp file, which is then transferred to the MVC by an ncFTP session.

- rsh ( remote shell ) runs the .cfg.pp file

- -.rhosts does authentication

## Recovery

  - Recovery Methods

    - Iridium

    - Radio Frequency Detector

    - Yagi

    - Flight

    - Open the front two sections for the scientists to access and download their data from the Bio Acoustics and CARL equipment.

    - Rinse out instraments with freshwater

    - Replace the lens cap

    - Install the CTD tubing

  - Recovery Instruction

- Ensure that the dropweight reset is on resertdw

- Once the FreeWave connects (Lights turn green) sss or telnet onto the vehicle and slay supervisor

- Confirm supervisor has been slayed ps –ef

$ ps -ef   //this checks to see if the mission is still running

//if you see a process called supervisor still running, then you do the following

$ slay supervisor

- Straighten out tail cone by commanding td newTailCone

- Notify the bridgethat the vehicle is ready to be recovered

- Once the vehicle has been craned into the cradle, gripe down the vehicle

- Open the front two sections of the AUV for the scientists to access and download their data from the Bio Acoustics and CARL equipment

- Install Camera Lens Cap

- Install the two CTD Hoses

- Check if power supplies are off and that the voltage and current are setup correctly V<36 and I>10A

- Attached Tether to the vehicle, then turn the power supply on

- Shut down the camera gracefully

$ carlTest

6  //Shut off the recorder

3  //Shut off camera

4  //Check status of Camera to make sure it is off, 0 indicates off, 1 indicates on

7  //Check status of the recorder to make sre it is off

1 //This exits the carlTest

- Check if recordings indicate being successful

Check on browser 134.89.32.54 and click on "All Clips", makes sure you have 14 videos 6-8 minutes long

- Check environmental monitoring

$ fanTest

r

- Check and turn off batteries

telnet 134.89.32.130 10003

@99AF //turns off AFT battery

telnet 134.89.32.128 10003

@99AF //turns off MID battery

telnet 134.89.32.121 10003

@99AF //turns off FWD battery

- Download log files, and deliver to science party

Note: Color indicates different Cygwin Windows

$ echo $AUV/logs //enter, this shows you the path of the log files

Open a second Cygwin terminal

//go to the directory that you want the logs to be downloaded into

$ ncftp -u dorado1 mvc-dmo1

ncftp /home/dorado1 > cd //insert directory that came from "echo $AUV.."command

ls //find the latest directory Ex:2018.065.02/

get -r //insert the latest directory

Close out

- Turn off Tracking on Ranger 2

  - This prevents the system from just sitting and pinging in the harbor until someone notices.

  - Note: When your done in the control room please shut down power to the racks, turn off the overhead lights and close the control room door.

- Once data has been downloaded off CARL, it is fine to power down the vehicle.

  - Emergency abort sss on any Cygwin window

## Post Dive Checklist

  - Ensure batteries are charged

  - Vehicle is freshwater rinsed

  - Rinse out the CTD components with freshwater

## Inspired Energy Batteries

## IP Addresses

134.89.32.130 (AFT)

134.89.32.128 (MID)

134.89.32.121 (FWD)

## Charging

## Charging off shore

## Charging on shore

## Time until Charged (From Empty  Full ~= 5hrs)

z [mAh]-y[mAh]z [mAh]=Time until Charged [hours]

Where,

y =  Current Charge

z = Full charge Capacity

## Commands

@99RQ	( status returned == comms up )

@99AD	( Main Discharge Mode )

@99AF	( Main off mode)

@99AC	( Charge Mode )

@99BD	(AUX1 Discharge enable)

@99BF	(AUX1 Discharge disable)

WDOFF	(Charge watchdog disable)

WDACT	(Charge watchdog active)

## Downloading Data

- In a new, freshly opened cygwin window, give these commands:

cd /cygdrive/c/dorado/data

ncftp –u dorado1 mvc-dmo1

ncftp cd /home/dorado1/auv-qnx/auv/altex/onboard/logs

ncftp>  get –r  (data) (data eg: 2005.021.05)

ncftp>  exit

Data is downloaded off the mvc and into “cd /cygdrive/c/dorado/data”

- Run MATLAB

cd \dorado\data

cd data

loaddata

plotSwSci =2

review

To close data windows:

close all

clear all

## Componenets

| Pressure Sensor, DigiQuartz Parosci | Model 8CB4000-I-263 part# 1789-103-0 serial#123917 4000m |
| --- | --- |
| Nova Tech Flasher / Pressure Switch | ST-400AR Serial# A04-028 7300m |
| Sonardyne Deep HF Marker Beacon | Type:7835-000-01 Serial #212664-01, 4000m ID - 23 |
| Sonardyne AVTrack Beacon (Tx/Rx) | 6G MF-Omni address 2409 serial#302765-006 3000m |
| DVL | Model:WHN1200 Serial #2036 1200khz |
| Seabird Subsea Pump | Model:5T part#05T-22120 Serial#05-7920 10500m |
| Seabird Dissolved Oxygen 1 |  |
| Seabird Dissolved Oxygen 2 |  |

## Directories, files, useful commands

| $AUV | = /home/dorado1/auv/altex/onboard (ex. cd $AUV) |
| --- | --- |
| $AUV_CONFIG_DIR | current vehicle configuration directory vehicle.cfg, devices.cfg, worksite.cfg |
| $AUV/bin | dir containing vehicle drivers |
| $AUV/logs/latest | most recent mission log file |
| syslog | file found in the mission log directory; useful for finding mission problems |
| grep –i abort syslog | did the mission abort?) |
| date +”%j” | day of the year, used in mission name |
| su ( become root ) shutdown –f | To reset the mvc |
| chksys –u | To check system files |
| df –h | To view disk space used/available |
| qtalk –dev /dev/ser6 ctrl-A to pull up qtalk menu ctrl-B to turn off DVL | To turn off the DVL |
| As root /usr/bin/ntpupdate –p 8 (IP of ntp server) On the Rachel Carson: 134.89.32.117 Or 134.89.22.5 cd /usr/local/bin more settime (settime script) | To Set Time |
| logtoMatlab gps.log > gpslog.txt | To convert GPS log file to Matlab file |
| ls –ld | to list directories |
| su | Super user |
| chmod a+rwx | to make a script executable |
| exit |  |
| telnet –e 0 134.89.32.XXX | Telnet within another telnet The –e allows you to enter an escape character then. Th escape character allows you to exit the telnet session without fully exiting out of the original telnet session. The standard escape character is “0”. |
| Ctrl+c |  |
| Ctrl+] |  |
| ncftp -u (insert username) | A way to transfer files between teo interconnected systems Use username |

## VI Editing commands

- i - Insert at cursor (goes into insert mode)

- a - Write after cursor (goes into insert mode)

- A - Write at the end of line (goes into insert mode)

- ESC - Terminate insert mode

- u - Undo last change

- U - Undo all changes to the entire line

- o - Open a new line (goes into insert mode)

- dd - Delete line

- 3dd - Delete 3 lines.

- D - Delete contents of line after the cursor

- C - Delete contents of a line after the cursor and insert new text. Press ESC key to end insertion.

- dw - Delete word

- 4dw - Delete 4 words

- cw - Change word

- x - Delete character at the cursor

- r - Replace character

- R - Overwrite characters from cursor onward

- s - Substitute one character under cursor continue to insert

- S - Substitute entire line and begin to insert at the beginning of the line

- ~ - Change case of individual character

Note: You should be in the "command mode" to execute these commands. VI editor is case-sensitive so make sure you type the commands in the right letter-case.

Make sure you press the right command otherwise you will end up making undesirable changes to the file. You can also enter the insert mode by pressing a, A, o, as required.

## Moving within a file

- k - Move cursor up

- j - Move cursor down

- h - Move cursor left

- l - Move cursor right

You need to be in the command mode to move within a file. The default keys for navigation are mentioned below else; You can also use the arrow keys on the keyboard.

## Saving and Closing the file

- Shift+zz - Save the file and quit

- :w - Save the file but keep it open

- :q - Quit without saving

- :wq - Save the file and quit

.