Power Supplies

| Supply | IP 134.89.32.xxx |
| --- | --- |
| LC1 | 209 |
| LC2 | 210 |
| 2 | 211 |
| 3 | 212 |
| 5 | 213 |
| 6 | 214 |

telnet 134.89.32.xxx 8003

outp:curr?, outp:volt?, outp:state?             -Request output current, voltage, state

outp:curr x, outp:volt x, outp:state x         -Set output current volt, and state

meas:curr?, meas:volt?                              -Request measured current, voltage

Mapper Operations

Mapper Layout: hostnames, can be used for pinging

| Mapper 1 | Mapper 2 |
| --- | --- |
| Edgetech3 (fwd bat) | Edgetech4 (fwd bat) |
| Reson4 (aft bat) | Reson1 (aft bat) |
| USBL Address: 1711 | USBL Address: 1712 |

- Use "alias", "check-" followed by two tab inputs, or "td " followed by two tab inputs to see available commands

- check alias use ~ as the esc key

- "check-help" will also list out currently used commands for each sensor

- "AUV_HOST=mvc-mb2-rf resetdw"                       reset dropweight server over freewave for mapper2

- "AUV_HOST=mvc-mb2-rf run_mission <mission_file>"    run mission (execute command within directory containing mission file)

- check devices.cfg within AUV_CONFIG_DIR to get serial ports of all devices

- Ewing User: mappingauv, Pass: buenavista. IP = TODO: get ewing IP

- "export" to get all terminal constants

- In cygwin, can get to C drive with either "cd C:" or "cd cygdrive/c/"

- Ctrl-R to search past commands in terminal

- Older versions of vim: use x to delete characters

Batteries

@99RQ, @99AF, @99AD, @99AC                     --Request status, off, discharge, charge (33.65V, 45A)

@99BD, @99BF, @99CD, @99CF                     --Turs AUX B and AUX C on/off

STATS

@<pack#>PF                                                       --turns a pack off

Reson

Password: reppe

- Reson timing checks require kearfott to be running!

- Requires Aux B output from aft battery

-Can use Deckmode when pulling data to prevent overheating

Edgetech

Username: administrator

Password: admin

- Requires Aux B output from fwd battery

- Use remote desktop for status checks

- User: administrator

- Pass: admin

- File -> Show Status -> POST gets general status

- Use "check-edgetech" for remote controls

- r for range, 1 for LFSS, set to 200

- w for pulse width, 0 for SBP, set to 300ms (3 Hz desired frequency)

- m for mode, n for on, 3 for ALL

- this should audibly ping at 3 Hz

- confirm on remote desktop

Avtrack

check-avtrack

cs to display configuration

vs to show battery info

Acomm

check-acomms

+++ to enter command mode, then type ATR-Enter to squack or ATS? for config

ATO to go back to online mode then crtl a q to exit

Timing

check-ntp on mvc

on Reson check sync'd to GPS in diagnostics (remote desktop)

Dimension 4 on Edgetech (remote desktop)

Parosci

on map1 need to qtalk and Ctrl-M, Ctrl-J to get streaming again

Kearfott

- kearfottTest restart, then align if you need to restart so run this early!

- mode: 0x6 and monitor 0x40 mean fully aligned

- if needed: kearfottTest shutdown -dev dev/ser10/

-also: kearfottTest restart -dev dev/ser10/ to realign

Logs/Syslog

cd $AUV/logs/latest

tail -f syslog

WinFrog

-Configure - Vehicles -Config Vehicle Devices

-Output AUV Data Output only one at a time

-Configure I/O Devices - Config

-Output AUV Data Output, change beacon code

-Map1=beacon2, Map2=beacon 3

-Configure Geodetics -UTM Zone 10

AUVMon

- Set Device Manager so Winfrog=COM1

- PAN=COM2

- ADDR 1711 for Map1, 1712 for Map2

- Then connect to all 3

- Start NAV packets (20sec)

-Connect to AUV is COM? on USB to Serial (check COM port in Device Manager)  -don't need if using SMS

-only use one or the other (Acomms or SMS)

Log Transfer

1. Navigate cygwin to where you want data on North

- In cygwin, one can get to C drive with either "cd C:" or "cd cygdrive/c/" -> "dorado/data/mapper1"

2. ncftp -u dorado1 mvc-mb1

3. get to log folder

- Using a different terminal with telnet connection to dorado, "cd $AUV_LOG_DIR" (or $AUV/logs)

- Transfer using "get -r <foldername>" (r = recursive)

- Open Matlab -> folder containing log/data

- Run 2nd command from Matlab Script Commands file on desktop

- If edgetech download isn't immediately working:

- Remote desktop into edgetech

- Data disk properties -> sharing -> advanced -> allow all