CPU BOARD ECOs

27 Feb 2019

PAROSCI:

Do not install U1 "upside down"; we will not be using this IC

TS4100:

Remove D15

Remove R139

Add 1uF 0603 Caps in parallel with C159, C160, C156, C155

SUPERCAP:

Add 22awg wire from (F3,D17,C66, TP160)-pad to positive side of C49

( to reduce resistance )

Remove Q6

Remove Q5

Remove R135

Remove R134

Add blue wire connection: U31 pin 7 to D15 - Anode

( EXT_RESET_N )

Add 22 AWG wire from U31 pin 1 to positive junction of C159 and C160

( to reduce resistance )

Add 100uF, 100nF and 10nF Cap in parallel with C50

Add 100uF Cap in parallel with C200

R26 is now 909k 0603 1%

R28 is now 274k 0603 1%

R21 is now 237k 0603 1%

Add 0.1uF 0603 cap in parallel with R24

FREEWAVE:

Add 10k resistor between pin 1 and pin 2 on U52 ( Freewave Connector )

Cut trace to gate of Q1, bluewire gate to CPU GPIO connector J12, pin 13 ( CN1-21 )

( Pulling the line to ground is required to get the radio into programming mode for setup. )

Add blue wire between TP90 and TP93, restoring the "OFF_BD_RESET_N" net to the UARTs.

GNSS5-CLICK:

Swap A3 pins 13 and 14 ( Rx and Tx lines into GNSS-5-CLICK )

Cut trace from A3 pin 2 ( GNSS_5_CLICK "RESET" ) to signal "OFF_BD_RESET"

CONSOLE RS232:

U39 is not installed:

Add blue wire from CN2-93 to J25-2

( Same as blue wire U39 pin 9 to 11 )

Add blue wire from CN2-95 to J25-3

( Same as blue wire U39 pin 7 to 6 )

VECTORNAV VN100:

Remove Q4

USB OTG:

remove R18