| P/N #: | 52362 | Effective Date: | September 24, 2019 |
| --- | --- | --- | --- |
| Title: | Replacing obsolete M100 modem |  |  |

## Purpose / Summary

The M100 Ethernet Modem Board (De-cased) (VR PN: 50153) is obsolete and a new modem assembly, RAK Modem G100 Topside Assembly (72246), has been introduced to replace the outgoing M100 modem used inside IP65 Rated Pro4 Panel Assembly (52362).

## Parts Affected

| Part Number | Notes | Removed | Added | Modified | REV |
| --- | --- | --- | --- | --- | --- |
| 52362 | IP65 Rated Pro4 Panel Assembly | ☐ | ☐ | ☒ | C |
| 50153 | M100 Ethernet Modem Board (De-cased) | ☒ | ☐ | ☐ | N/A |
| 72246 | RAK Modem G100 Topside Assembly | ☐ | ☒ | ☐ | A |

## Detail

The M100 modem is used on IP65 Rated Pro4 Panel Assembly to convert 4 wire Ethernet to 2 wire Ethernet and vice versa for Ethernet communication between the ROV and the top side ICB panel (to communicate with Blueview Sonar). The M100 has become obsolete and a new modem assembly based on LX200V30 (RAK) Modem has been designed to replace the M100 modem. This ECO deals with all changes involved is the replacement of M100 modem with new RAK modem assembly.

## Disposition of Material

The remaining (If any) stock of M100 should be used in production.

## Action Items

| Action | Department | Acknowledgement Initials |
| --- | --- | --- |
| UPDATE 52362-IP65 Rated Pro4 Panel BOM | ENGR | GD |
| UPDATE EPICOR BOM | ENGR | RN |
| UPDATE WI-52362-IP65 Rated Pro4 Panel Assembly Instructions | ENGR | GD |
| CREATE 52362.PDF- IP65 Rated Pro4 Panel Assembly Drawing | ENGR | JV |

## Modified / Referenced Documents:

| Document | Current Revision |
| --- | --- |
| WI-52362-IP65 Rated Pro4 Panel Assembly Instructions | B |
| 74426.PDF - RAK modem G100 Topside Assembly Drawing | A |
| 71095.PDF- IP65 Rated Pro4 Panel Assembly Drawing | A |

## Revision History

| Date Revised (mm-dd-yyyy) | Location of Change (section, paragraph no., step no., etc.) | Description of Change Made | Name of Person making the Change |
| --- | --- | --- | --- |
| 9/24/2019 |  | Initial release | GD |