| P/N #: | PN4-003 | Effective Date: | ### September 11, 2018 |
| --- | --- | --- | --- |
| Title: | Modification to accommodate new 384V to 12V DC-DC converter changes |  |  |

## Purpose / Summary

The 384V to 12 DC-DC went EOL

The new 384V to 12V DC-DC converter on Pro4 ICB panel power PCB (started to install in February 2018) has slight different characteristics than the obsolete BCM converter.

As a result, the tether voltage may not come up when the ROV is connected to ICB without a tether. To rectify this change minor modification to the main solid-state relay is needed.

Pro4 power supply shipped individually to customers will need to include the “68uF Capacitor Assembly for Pro4 SS Relay” PN 71573 and Work instructions “Pro4 Relay Capacitor Modification”.

This ECO addresses the implementation of the modification required.

## Parts Affected

| Part Number | Notes | Removed | Added | Modified |
| --- | --- | --- | --- | --- |
| PN4-003 | Circuit Board Panel Power Supply 5/12/24/74 V DC (Pro 4) | ☐ | ☐ | ☒ |
| 52334 | Loctite 596 Red High Temperature RTV Silicone | ☐ | ☒ | ☐ |
| 71573 | 68uF Capacitor Assembly for Pro4 SS Relay | ☐ | ☒ | ☐ |

## Detail

The Pro4 Panel ICB power PCB electronics uses a 384V to 12V DC-DC converter to generate the required 12V power at 300W for various uses. The specific DC-DC converter used for this purpose became obsolete (P/N B384F120T30 from Vicor) and as a result the Pro4 ICB Panel Power PCB (PN4-003) BOM was updated with a new placement (P/N BCM384F120T300A00 from Vicor) under ECO-PN4-003-006.

The new converter has slightly different electrical characteristics compared to the obsolete converter. These minor changes have caused the tether voltage to not switch on correctly when the ROV is connected to the ICB panel without a tether.

To rectify this issue a 68uF/100V Aluminum electrolytic capacitor is added across the output of the main solid-state relay which controls the switch ON/OFF function of tether voltage. The capacitor is retained with Red RTV to breaking off due to vibrations during shipping as shown in the photo below (next page). The retaining method may be improved via another ECO near future.

Effective from date stated above, all Pro4 power supplies shipped individually to customers will have to include the “68uF Capacitor Assembly for Pro4 SS Relay” PN 71573 and Work instructions “Pro4 Relay Capacitor Modification”.

This ECO records new changes together with subsequent follow up actions in related departments.

Note that as PRO4 ICB work instructions are out of date and uncontrolled, this document serves as the work instruction addendum.

## Disposition of Material

Rework of ALL stock.  (completed at time of ECO publication)

## Action Items

| Action | Department | Acknowledgement Initials |
| --- | --- | --- |
| Update the remaining ICB panels in stock with this modification | Engineering | GD |
| Update SharePoint Records | Engineering | AEG |
| Inform subcontractors of modification and supply new Panel BOM (PPI) | Engineering | CH |
| Provide work instructions “Pro4 Relay Capacitor Modification” | Engineering | CH |
| - Include 68uF capacitor assembly PN 71753 and “Pro4 Relay Capacitor Modification” work instructions with all Pro4 power supplies shipped individually - Complete 68uF Capacitor Assembly for Pro4 SS Relay PN 71573 for inventory | Production | KK |
| Find location for “68uF Capacitor Assembly for Pro4 SS Relay” PN 71573 in inventory | Inventory | SL |

## Modified / Referenced Documents:

| Document | Current Revision |
| --- | --- |
| Pro4 Relay Capacitor Modification | A |

## Revision History

| Date Revised (mm-dd-yyyy) | Location of Change (section, paragraph no., step no., etc.) | Description of Change Made | Name of Person making the Change |
| --- | --- | --- | --- |
| 9/11/2018 | Rev A: new release | Initial release | GD |
| 9/27/2018 | Rev. B: changes to Summary, paragraph 4. Details paragraph 4. Reference Documents | Added individual power supply shipping and work instructions | CH |