## Purpose / Summary

CPU boards with the latest KCF compass would have a delay in startup and would not calibrate. A CPU firmware update was made to correct the problem.

## Parts Affected

| Part Number | Notes | Removed | Added | New | Modified |
| --- | --- | --- | --- | --- | --- |
| PRO4-SUB-CPU19 | Only Firmware modification | ☐ | ☐ | ☐ | ☒ |
| 50175 | Only Firmware modification | ☐ | ☐ | ☐ | ☒ |

## Detail

Comms timeouts and delays on the latest batch of KCF compass boards appear to have changed, or the original firmware was not robust enough to handle the seen variance in timing.   The firmware was modified to improve communication between the cpu and compass when reading the compass type and calibration data.

This firmware was tested on 20 board sets with no failures.   The firmware was tested on 3 ROV, all that were available.  One of the ROV still had issues.  Additional ROV will be tested when available.

## Action Items

| Action | Department | Acknowledgement Initials |
| --- | --- | --- |
| Test firmware on 20 board sets and 3 ROV | Engineering | SL, AEG |
| Ensure new firmware is being used in production | Production | KK |
| Inform Support of change | Engineering | AEG |
| Coordinate internal distribution of firmware | Engineering | SL |
| Update all new compass board sets to latest firmware | Engineering | SL |
| Push new firmware to subcontractor suppliers | Engineering | AEG |

## Modified / Referenced Documents:

| Document | Current Revision |
| --- | --- |
| Pro4 firmware (pro4_2_7_12.hex) | 2.7.12 |

## Revision History

| Date Revised (mm-dd-yyyy) | Location of Change (section, paragraph no., step no., etc.) | Description of Change Made | Name of Person making the Change |
| --- | --- | --- | --- |
| 11/09/2017 | All sections | New release, Rev. A | AEG |