bmorenobr

The Global Leader in MicroROV Technology

Pottstown, PA

Engineering Change Proposal

| Document Number: | ECP-70273-005 |  |  |  | Document Revision: |  | A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Revision Date: | 9/8/2023 |  | New Product Revision: |  |  | H |  |

Xsens Mti-30 AHRS EOL

| ECP Class: | ☐1 Form Fit Function | ☒1.5 Minor Change | ☐2 Doc/Informative |
| --- | --- | --- | --- |

| Priority: | ☐1 EMERGENCY | ☐2 Urgent | ☒3 Routine |
| --- | --- | --- | --- |

| Need Date: | 1/15/2024 |
| --- | --- |

| Backwards compatible: | ☒ YES | ☐ NO |
| --- | --- | --- |

| External Approval Needed: | ☐ YES | ☒ NO | CONACT: |
| --- | --- | --- | --- |

Revision History

| Rev. | Date | Description | Requester | Engineering Approval |
| --- | --- | --- | --- | --- |
| A | 9/7/2023 | Initial release | PUR | AEG |

Abbreviations

| Abbr. | Definition |
| --- | --- |
| AHRS | Attitude heading and reference system |
| IMU | Inertial measurement unit |

Xsens Mti-30 AHRS EOL

Details:

The xsens sensor used in the mission specialist AHRS has gone obsolete.

The xsense Mti-630 is a new improved sensor for the same manufacturer.

The Mti-630 can output exactly the same data sentence as the Mti-30, so no software changes are required.

The new revision is fully backwards compatible with all fielded tethers and systems.

There will be no change to operation at all.

There should be no impact on usage, maintenance, or training.

There will be no change to external interfaces.

There should be improved performance of the AHRS and potentially new features.

There will be internal changes to the mounting of the sensor.

We also plan to make the internals compatible with other AHRS sensors, in particular the Sparton M2.  This will allow us to evaluate a potentially more performant sensor.   Any sensor change beyond the Mti-630 documented here would be a separate change proposal.

Any impacts to product cost are to be determined but should be minimal.

Reason

| ☐ Improvement | ☒ EOL | ☐ Flaw | ☐ Safety | ☐ Cost | ☐ Production Optimization |
| --- | --- | --- | --- | --- | --- |

Anticipated Parts Effected

| PN: | Description/Notes: |
| --- | --- |
| 70273 | AHRS module |
| 70555 | Xsens adapter pcb |
| 70371 | IMU board skid |

Compatibility / Interface Issues:

None.

Affected Stakeholders

| ☒ Purchasing | ☒ Production | ☐ QC | ☐ Sales / Marketing | ☐ User Docs | ☒ Customer Success |
| --- | --- | --- | --- | --- | --- |
| ☐ Dealers | ☐ New Customers | ☐ Existing Customers | ☐ Specific Customer(s) | ☐ Integration partners |  |

Details:

General awareness and education.

No change to documentation or published specifications will be required.

No recall of previous revisions.

Current stock can be used to exhaustion.

Cost Estimate (reference estimated budget when available)

| Non-recurring: | $10,000 |
| --- | --- |
| Recurring: | <=$500 |

References

| Document Number | Description |
| --- | --- |
| https://base.movella.com/s/article/Migration-MTi-10-100-series-to-MTi-600-series-1605870241697?language=en_US | XSens Migration document ling |