bmorenobr

The Global Leader in MicroROV Technology

Pottstown, PA

Engineering Change Proposal

| Document Number: | ECP-73755-001 |  |  |  | Document Revision: |  | A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Revision Date: | 10/16/2023 |  | New Product Revision: |  |  | B |  |

Replacing 4 Holes on Top Plate of Combo Skid

| ECP Class: | ☐1 Form Fit Function | ☒1.5 Minor Change | ☐2 Doc/Informative |
| --- | --- | --- | --- |

| Priority: | ☐1 EMERGENCY | ☐2 Urgent | ☒3 Routine |
| --- | --- | --- | --- |

| Need Date: | 10/16/2023 |
| --- | --- |

| Backwards compatible: | ☒ YES | ☐ NO |
| --- | --- | --- |

| External Approval Needed: | ☒ YES | ☐ NO | CONTACT: Gaghan, Frederick E CIV (USA) <frederick.e.gaghan.civ@us.navy.mil> |
| --- | --- | --- | --- |

Revision History

| Rev. | Date | Description | Requester | Engineering Approval |
| --- | --- | --- | --- | --- |
| A | 10/16/2023 | Initial release | MW | AEG |

Abbreviations

| Abbr. | Definition |
| --- | --- |
| MSS | Mission Specialist Series |
| SHCS | Socket Head Cap Screws |

Replacing 4 Holes on Top Plate of Combo Skid

Details:

Top Plate 73754 has 2 holes on both starboard and port side (4 total) that are counterbored for attaching Titan Disruptor parts. During testing, it was found that the bolts going through these holes shear through the top plate. As a remedy, it was found that switching these counterbored holes to countersink will fix the problem. We are requesting to make future skids with a top plate that has countersunk holes.

Reason

| ☒ Improvement | ☐ EOL | ☐ Flaw | ☐ Safety | ☐ Cost | ☐ Production Optimization |
| --- | --- | --- | --- | --- | --- |

Anticipated Parts Effected

| Part Number: | Description/Notes: |
| --- | --- |
| 73754 | Top Plate – EOD Combo Skid |
| 73755 | EOD Combo Skid Assembly |

Compatibility / Interface Issues:

None

Proposed Test Plan:

None

Affected Stakeholders

| ☒ Purchasing | ☐ Production | ☒ QC | ☐ Sales / Marketing | ☐ User Docs | ☐ Customer Success |
| --- | --- | --- | --- | --- | --- |
| ☒ Dealers | ☒ New Customers | ☐ Existing Customers | ☒ Specific Customer(s) | ☐ Integration partners |  |

Details:

Engineering to update the 73754 dwg to updated countersunk holes. See pics below on what holes and the new dimensions.

Cost Estimate (reference estimated budget when available)

| Non-recurring: | None |
| --- | --- |
| Recurring: | Same price as original top plate |

References

| Document Number | Description |
| --- | --- |
| 73755 | EOD Combo Skid Assembly |