| P/N #: | 70160 | Effective Date: | March 15, 2022 |
| --- | --- | --- | --- |
| Title: | RS485 communication reliability improvements |  |  |

## Purpose / Summary

Resistor values of RS485 communication chip are changed to accomplish reliable communication regardless the chip manufacturer.

## Parts Affected

| Part Number | Notes | Removed | Added | Modified | REV |
| --- | --- | --- | --- | --- | --- |
| RC0603FR-071KL | RESISTOR SMD 1K OHM 1% 1/10W 0603 | ☒ | ☐ | ☐ | - |
| RC0603FR-074K7L | RES SMD 4.7K OHM 1% 1/10W 0604 | ☒ | ☐ | ☐ | - |
| RMCF0603FG464R | RES 464 OHM 1% 1/10W 0603 | ☐ | ☒ | ☐ | - |
| 70060 | SPAM | ☐ | ☐ | ☒ | B |
| 70155 | Power Module | ☐ | ☐ | ☒ | C |
| 70192 | Comms Module | ☐ | ☐ | ☒ | D |

## Detail

Serial communication tests would fail in during QC inspection.  This was traced to a tolerance change in the biasing requirements on a batch of transceiver chips.  New biasing resistors were selected to ensure the biasing was more robust.

Resistor networks (R4, R8, R11, R15, R18, R22) associated with RS485 communication chip on board (U1, U2, U3) needed to be modified to accommodate slight changes in tolerances with these chips depending on the chip manufacturer.

Resistor Locations

The next revision of the power board will include the updated BOM.

## Disposition of Material

Old stock should be reworked on occurrence of Q/C failures.

Change should be implemented on all new production runs.

## Action Items

| Action | Department | Acknowledgement Initials |
| --- | --- | --- |
| Updated power converter module BOM reflecting the new modification | Engineering | GD |
| Update SharePoint Records | Engineering | AEG |
| Inform board assembly vendors about updated BOM | Engineering | GD |

## Modified / Referenced Documents:

| Document | Current Revision |
| --- | --- |
| Click here to enter text. | Click here to enter text. |

## Revision History

| Date Revised (mm-dd-yyyy) | Location of Change (section, paragraph no., step no., etc.) | Description of Change Made | Name of Person making the Change |
| --- | --- | --- | --- |
| 3/15/2022 | All | Init | GD |