| P/N #: | 70191 | Effective Date: | April 20, 2022 |
| --- | --- | --- | --- |
| Title: | Depth Rating Improvements |  |  |

## Purpose / Summary

Add up-armor to crystals to improve depth rating

## Parts Affected

| Part Number | Notes | Removed | Added | Modified | REV |
| --- | --- | --- | --- | --- | --- |
| 70191 |  | ☐ | ☐ | ☒ | E |
| 70188 |  | ☐ | ☐ | ☒ | G |
| 72247 |  | ☐ | ☐ | ☒ | C |

## Detail

During a deep (>1km) dive communications were lost to the ROV.

This was traced to crushing of metal cased crystals on the PCBs.  Notable on the ethernet hub and long haul modem board.

Adding a stiffing member on top of the crystals protects them from stress induced failures.   A small laminate (PCB substrate) was fabricated and epoxied on top of the crystals.

In tank pressure tests validated the improvement (3000psi).  Multiple deep dive operations in excess of 1km have been accomplished with reworked comms modules.

## Disposition of Material

Rework failed and FA stock as needed

Implement change in all WIP

## Action Items

| Action | Department | Acknowledgement Initials |
| --- | --- | --- |
| Update BOM in Epicor | Engineering | JRK |
| Update Part and Assembly Revisions in Epicor | Engineering | JRK |

## Modified / Referenced Documents:

| Document | Current Revision |
| --- | --- |
| WI-70191 | D |
| WI-72247 | C |

## Revision History

| Date Revised (mm-dd-yyyy) | Location of Change (section, paragraph no., step no., etc.) | Description of Change Made | Name of Person making the Change |
| --- | --- | --- | --- |
| 03/03/2022 | - | Initial Release | aeg |