EODROV Change Configuration Log

# Current Modifications

| ## Item | ## NIWC Name | ## Description | ## p/n | ## REV | ## ECO | ## Compatibility ISsue | ## DATE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Mission Specialist Tool Kit Updates | 70910 | G | ECO-70910-003 | No | 6/17/2021 |
| 2 |  | Mounting support for local autonomy engine | 70741 | B | ECO-70741-001 | No | 10/1/2021 |
| 3 |  | Material change to meet flatness tolerance | 71326 | B | ECO-71326-001 | No | 10/20/2021 |
| 4 |  | EOL Lens replacement with new wide-angle lens | 70044 | G | ECO-70044-5 | No* | 12/23/2021 |
| 5 | Manipulator head | Rotating Manipulator quick change head | 70824 | D | ECO-70824-001 | Yes | 1/12/2022 |
| 6 | Thruster refill attachment | Temporary thruster fill kit coupler replacement | 72854 | B | ECO-72854-001 | No | 1/13/2022 |
| 7 |  | Improved rs485 bias resistors | 70060,70155,70192 | B,C,D | ECO-70060-70155-70192-001 | No | 3/15/2022 |
| 9 |  | Depth rating improvements | 70191 | E | ECO-70191-005 | No | 3/20/2022 |
| 9 | Adding the strain relief block | 8 pin whip retention | 70745 | E | ECO-70745- | No* | 5/04/2022 |
| 10 | Adding the DAGR Cable | DAGR cable addition | 73233 | B | ECO-73233-001 | No | 5/04/2022 |

# Pending Modifications with current delivery order

| ## Item | ## NIWC NAME | ## Description | ## p/n | ## REV | ## ECO | ## Compatibility ISsue | ## DATE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | OCS CPU change due to EOL | New OCC computer and 24V aux | 70218 | J | ECO-70218-003 | No | TBD |

DETAILS: 1 Mission Specialist Tool Kit Updates Minor changes to tool kit, to cover EOL sog tool and mag sig reduction. This had been made prior to previous delivery order and was captured in last configuration audit. Verification/Validation: N/A 2 Mounting support for local autonomy engine Minor change to single frame piece, a few holes to allow for addition of the local autonomy engine. Note that we are still using up old stock Verification/Validation: Fitment of local autonomy engine 3 Material Change to meet flatness tolerance

This is to ease the manufacturing for requirements necessary to improve the robustness of the OCC monitor and prevent delamination.   Note that VideoRay had been scrapping material that did not meet tolerance specification, supplier requested change to different grade of aluminum for the monitor bezel.

Verification/Validation: Standard Q/C flatness tests and monitor usage tests in demo equipment

4 EOL Lens replacement with new wide angle lens

Lens went EOL without notification and was unobtainable.  A cross was selected.

It has a wider angle, so customers who require exact lens intrinsic parameters would have a compatibility issue.

Verification/Validation: Standard Q/C flatness tests, FOV tests, internal field tests, evaluation by NIWC personnel.

5 Rotating Manipulator quick change head

New quick release heads.  Older heads/bodies are not cross compatible but can be upgraded.

New p/n’s were giving to the body and heads.

Verification/Validation: Standard Q/C operational tests, 10kg lift tests, internal field trials, external (NIWC) field trials

6 Temporary thruster fill kit coupler replacement

Nylon coupler was unobtainable due to supply chain.   Replaced with brass coupler and o-ring.  We anticipate this to be temporary and plan to switch back.

Verification/Validation: Leak checks with brass coupler and o-ring.  Operational verification.

7 Improved rs485 bias resistors

Minor resistor change to accommodate new batch of rs485 transceivers.

This was required on multiple PCBs.   Prior to change PCBs would fail internal Q/C tests.

Verification/Validation: Standard Q/C operational tests, bandwidth test, internal field trials.

8 Depth rating improvements

OEM component revisions that were not caught caused failures at depth >= ~1000m.

Addition of reinforcement to crystals on comms hub printed circuit board

Verification/Validation: Standard Q/C operational tests, pressure test in tank ~3000psi, field trials on San Clemente Island to > 1km depth.

9 pin whip retention

In an attempt to prevent damage to the power module connector additional retention of the 8 pin whip was added.

The rear frame member [] needs holes drilled into it.  Currently that is being done as post operation during frame production (rather than a modification to the part drawing). A drill jig was designed for this operation and can be done in the field.   The frame member will eventually get a revision once the need for the retention block long term is determined.

Verification/Validation: Fitment tests, internal field tests, prototype evaluation by NIWC personnel

10 DAGR cable addition

A new custom cable for the DAGR was added to the standard EODROV package

Verification/Validation: Fitment tests both AC/400 and standard OCC, lab test with GFE, prototype evaluation by NIWC personnel

11 New OCC computer and 24V aux (still in progress)

Updates of EOL computer.   Simple move to newer version of same model.  Also, some additional minor changes worked into the ECO

Verification/Validation: Fitment build ups, lab eval for compute performance and accessory compatibility.  Plan for field test prior to full release

# Pending Future Modification. ITEM; NIWC NAME; DESCRIPTION; P/N; REV; ECO; COMPATIBILLITY ISSUE; DATE. 12; Some kind of external hardening of the power module; Power Module bulkhead; Yes; TBD. 13; Rear facing camera mount; No; TBD. 14; New monitor supplier; No; TBD. 15; Defender strain relief mag sig mods; No; TBD. 16; Thruster Guards; No* (multiple part replacement per thruster); TBD. 17; Thruster diaphragm; No; TBD. 18; New PPT; No; TBD. 19; Deep USBL mount; No* (needs drill jig); TBD. 20; New Auxiliary camera; No; TBD. 21; Replacing the camera (not the lens) due to EOL; New Main camera; No; TBD. 22; Frame mods to support Tritech; No* (significant frame mod); TBD. 23; Switch to XGT power modules; No; TBD. 24; Tablet sunshade; No; TBD. 25; PPT reel w/ level wind; No; TBD. 26; Refactored CDS/Titan Skid; No; TBD. 27; GPS antenna potting improvements; No; TBD.