Mesobot camera notes

- Aurora, 700 tvl, 61 degree, 965 pixels horizontal

  - @1m: 1.22 mm/pixel

  - @1.5m: 1.83 mm/pixels

- DSPL HD 85 degree FOV, 1920 pixels horizontal

  - @1m: 0.93mm/pixel

  - @1.5m: 1.43mm/pixel

- DSPL HD 145 degree FOV

  - @1m: 3.33mm/pixel

  - @1.5m: 4.95mm/pixel

- 4k 145 degree FOV

  - @1m: 1.65mm/pixel

  - @1.5m: 2.48/pixel

August 29 ROV tests

- Use Mike’s Aurora stereo setup

- Move working area farther out

  - Center a 1.2m working volume 1.5m out

    - Limited by the Aurora FOV

    - Lower camera toed up 10 degrees

      - Adjusted camera toe in and recalibrated on the 16th

  - Mount two of the existing lights

    - 250 watt ROS block lights, 85 degree FOV

    - on frame outside of cameras pointed forward, toed in 17 degrees.  This shouldn’t flood the nearfield with light