Prototype Power Buoy Configuration 3 – Un-instrumented Flat Plate

This configuration represents the most reduced system envisioned and includes an instrumented surface expression with power take-off system and an un-instrumented submerged flat plate.

Functional Requirements:

- The device shall be engineered as economically as possible to operate for short tests over the one-year duration of the test program.

- The device should be as small as possible to quantify the desired effects, but large enough to extrapolate the desired effects to a full-sized production model.

- The structure shall be sufficient to float in water without significant deformation and support the required subsurface elements without separating in sea states up to state TBD.

- The surface expression shall support the power take-off device described in the PTO functional requirement section.

- The surface expression must support the load of a significant quantity of batteries, representing the power storage for a production model. For the prototype, the batteries may be represented by comparable ballast.

- The surface expression shall have a minimum of structure above the still waterline to minimize wind loading.

- As an engineering prototype, the device does not need to be engineered for subsequent transition to another group (such as operations or support engineering); it is fine if the engineers who design the various systems are the only ones who can operate them.

- As an engineering prototype, the device may contain additional sensors and systems that would not be present on a production model.

- The device does not need to be self-powering; for example, it is completely acceptable that there are auxiliary batteries designed into the system to power any needed equipment.

- COTS hardware shall be used wherever possible.

- Surface communications shall be sufficient for transmitting system status to shore or support vessel via line-of-site radio or satellite system

- Data from all sensors must be logged at a rate sufficient to prevent aliasing of the measured quantity, such that data deviations between samples don’t exceed 1% of full scale.

Design Requirements:

- Surface Element

  - The surface expression shall be designed to support unattended deployments of up to 2 days, with a service lifetime of 1 year. [FR 1]

  - The surface expression shall have a circular water plane area of 10 square meters (diameter = 3.6 meters). [FR 2]

  - The surface expression shall have a displacement of 5.3 m^3 when supporting the static weight of the submerged foil (8400 N / 1880 lbs). [FR 2]

  - The surface expression shall have a reserve buoyancy of 5.0 m^3. [FR 2]

  - The surface expression shall support at least a 25 kN downward force at the attachment point. [FR 3]

  - The surface expression shall support at least 1500 kg of lead-acid batteries, or their ballast equivalent. [FR 5]

  - The exterior shape of the surface expression shall be axi-symmetric around a vertical axis. [FR 1]

  - Surface Element Controller

    - GPS: Log and timestamp GPS measurements no slower than a rate of 0.5 Hz. [FR 12]

    - IMU: Log and timestamp 6 DOF IMU measurements at the rate the sensor produces measurements (TBD). [FR 12]

    - Compass: Log and timestamp compass measurements no slower than a rate of 5 Hz. Accurate to 1 degree, suitable for use in magnetic environment dictated by the surface expression structure construction (which may be steel), and at tilt angles of TBD degrees. [FR 12]

    - Load Cell: Log and timestamp load cell measurements no slower than a rate of 5 Hz. (0 – 10,000 lb, accurate to +/- 5 lbs, or 0.05% FS). [FR 12]

    - Power Take-Off Device: Computing infrastructure shall be sufficient to manage data logging and control algorithms required by the power take-off system. [FR 4]

    - All timestamp measurements shall be accurate relative to 10% of sample rate. The absolute timestamp accuracy shall be TBD sec. [FR 12]

    - Radio communications shall support a data rate of TBD bps with a latency of no more than TBD sec and a payload of TBD kbytes per day. [FR 11]

- Subsurface Element

  - The subsurface expression shall be designed to support unattended deployments of up to 2 days, with a service lifetime of 1 year. [FR 1]

  - The subsurface structure shall support 20 m^2 of foil area. [FR 2]

  - The subsurface element shall weigh 8,400 N (1,880 lbs) in seawater. [FR 2]

  - The structure and attachment points shall be strong enough to allow at minimum a 25 kN (5,600 lbs) force to be applied vertically upwards. [FR 3]

  - The structure shall hang in a stable horizontal orientation from a single attachment point (with the use of a bridle if required). [FR 1]

  - The subsurface device shall be suitable for use up to 50 m water depth in seawater. [FR 2]