EyeRIS Dive Operations

Pre Dive

- Check hardware visually

  - Clean camera and light ports with wipes

  - Check for loose fittings

- Power on ROV and check Camera Ethernet comms (internal sensors, etc)

- Plug all four coax connectors into PHT4 FG box and confirm it as power.

- Turn on camera power and confirm PHT4 FG box shows no flashing lights

  - Flashing red means no light on that channel

  - Flashing green means weak signal and data recoding is not possible

- Open up RxLive and wait for the “Updating” message in the bottom right corner to change to showing GPU memory

- Once the GUI has updated, click on camera on open

- Confirm camera is working as expected

- Click on Export and select sequence output, set file path and file prefix as desired

- Turn on light power and confirm all lights are working up to 100%. Turn off lights until vehicle is in the water

Dive

- For MiniROV, Turn on O2 sensor

- Turn on lights at 100% and check GUI shows 100% output

- Check exposure time, fps and set sequence recording for mission

- Periodically check the following during dive:

  - Lights at 100% output

  - Available disk space on E: (Storage)

  - Camera settings

Post Dive (During Recovery)

- Turn off lights before the vehicle leaves the water

- Start copying data from Storage to external USB drives

- Close RxLive and turn off power to camera

- Unplug 4 coax connectors from PHT4 FG module.