EyeRIS (Remote Imaging System)

MBARI Project 800139

- Functional Requirements (General)

  - EyeRIS shall be configurable for a range of particle sizes, particle concentrations, flow rates, and platform profiling speeds for a specific use case.

  - EyeRIS shall measure particle sizes in the range 250-10000 microns with a resolution of 100 um

  - EyeRIS shall be able to bin particle sizes with 10% accuracy (min 100 particles per bin)

  - EyeRIS shall provide images sufficient for morphological identification of objects in the range 1 mm to 20 cm

  - EyeRIS shall provide a quantifiable imaged volume

  - EyeRIS shall provide direct color LED illumination, one for minimal animal disturbance [Red or Near IR]

  - EyeRIS shall provide adjustment of illumination direction as needed for application

  - EyeRIS shall be  underwater with targets and particle fields

- Functional Requirements (Cabled Instrument):

  - EyeRIS shall stream live video to a surface vessel

  - EyeRIS shall record data on an external high-resolution recorder on a surface vessel

  - EyeRIS shall operate to a depth of 4000 meters

  - EyeRIS shall be compatible with existing ROV power and communications

  - EyeRIS shall be deployable on a manipulator arm of an ROV

- Functional Requirements (Autonomous Instrument)

  - EyeRIS shall record image and video data locally

  - EyeRIS shall operate to a depth of 4000 meters

  - EyeRIS shall operate autonomously for the duration of an LRAUV mission

  - Shall record continuously or be able to go in a low power sleep mode

  - Shall have internal control of camera from a low-power MCU

  - Shall support vehicle ground fault detection

  - Shall be controllable with vehicle mission exec

- Functional Requirements (3DPIV):

  - EyeRIS shall be able to resolve objects in the size range from 1-20 cm (x/y dimension)

  - EyeRIS shall be able to resolve objects in the size range 1-20 cm (z dimension)

  - EyeRIS shall measure fluid velocities from 1mm/s to 1 m/s

- Functional Requirements (3D(dt)R):

  - EyeRIS shall resolve features on the order of 1% of size on objects in the size range 1-20 cm (x/y/z).

  - EyeRIS shall resolve mucus and gelatinous structures (> 50% opacity)

  - EyeRIS shall be able to resolve objects in the size range from 1-20 cm (x/y dimension)

  - EyeRIS shall size targets with accuracy better than 5% x/y/z

  - EyeRIS may use auxiliary/multiple cameras to verify target measurements.

  - EyeRIS may use structured illumination to enhance surface reconstruction

- Functional Requirements (Plankton Imaging, CeNCOOS):

  - EyeRIS shall provide data that can be integrated into CeNCOOS workflows

  - EyeRIS shall provide raw camera data for still image capture for posterity