Comparison of other 3D imaging methods

Camera Arrays

Combine multiple cameras together along with point correspondence matching and reconstruction of a 3D scene with high-resolution texture.

Structured illumination

Project a known pattern into the scene such that its appearance in the image varies uniquely with depth. Typically combine this with a color camera. Many modern systems now combine stereo cameras with structured illumination.

Depth Camera or Time-of-Flight Camera

Two common types:

- Phase Modulation: The camera exposes synchronously with a modulated light source at 10s of MHz. The camera samples at 2 or 4 different phases of the modulation to recover the magnitude and phase at each pixel. The phase shift by the 3D scene is recovered.

- Time of Flight: The camera sensor has the ability digitize the time between emission and reception of a pulsed light source (flash-lidar). The time delay is converted to range and combined with the intensity to make a point cloud.

Plenoptic Cameras

A microlens array is placed in front of the camera sensor. Lighfield 1.0 places the microlenses one focal length away from the sensor such that pixels subtended by each microlens measure the angular variation in the field and the microlenses sample the spatial variation. Lightfield 2.0 places the microlenses farther than 1 focal length from the sensor such that each microlens and subtended pixels act like a miniature camera in a dense camera array.

|  | Camera Arrays | Structured Illumination | Depth Camera / Flash Lidar | Plenoptic |
| --- | --- | --- | --- | --- |
| Method | Parallax | Parallax | Phase modulation or Time of Flight | Parallax |
| Pros | High-resolution Scalable Many geometries possible Can cover full 4 pi around object | High-resolution Single camera Provides texture to featureless objects | Low computational complexity No correspondence problem | High-resolution Very good light collection efficiency Single camera Simpler correspondence problem than stereo |
| Cons | Correspondence problem High computational complexity Multi-camera synchronization | Sensitive to IOPs of seawater Mostly IR only in COTS products | Low-resolution Limited perspective Low-power IR | Limited perspective Loss of spatial resolution from native sensor resolution High computational complexity |
| Example | Stanford Multi-Camera Array | Kinect 2010 | Basler ToF camera | Raytrix R26 |

Table 1: Comparison of several types of 3D imaging methods that can acquire a 3D image from a single exposure.