# Potential impacts of THE MONTEREY Accelerated Research System (MARS) Cable on the seabed and benthic faunal assemblages

# 2011

# J.P. Barry, L.A. Kuhnz, K. Buck, C. Lovera, P.J. Whaling

Monterey Bay Aquarium Research Institute

SUMMARY

A geological and biological sampling program to assess the condition of the MARS cable and its potential effects on seabed geology and biology was performed in late 2007-2008 and again in 2010.  The most recent study was concluded 37 months after the cable was installed.  The sampling program was designed to:

- Observe the condition of the cable or cable trench along the cable route (51 km),

- Assess the potential impacts of the MARS cable on geological characteristics and biological assemblages on a local scale (0-100m from cable) and a regional scale (km), using remotely operated vehicle (ROV) video transects and sediment samples.

The major conclusion of the study is that the MARS cable has had little detectable impact on seabed geomorphology, sediment conditions, or biological assemblages.  Specific conclusions include the following:

- Over most of its length, the cable remains buried, with little evidence of change since installation

  - The burial trench remains intact along deeper areas of the cable route,

  - Sediment is filling the cable trench, which is now nearly invisible in many locations.

  - In the limited areas where the cable could not be buried, only minor suspensions of the cable are present

- No detectable changes in sediment characteristics (mean grain size, or percent organic carbon) can be attributed to the MARS cable.

- Local variation in benthic megafaunal communities near (within 50-100 m) the MARS cable is minor or undetectable.

  - The abundances of most animals observed did not differ between the area over the cable route and 50 m away.

  - In 2008, before the cable was powered, Longnose skates (Rajarhina) were significantly more abundant along a 350 m long section at ~300 m depth, near minor (2-10 cm) suspensions of the cable above the seabed. R. rhinamay have responded to mild electromagnetic fields generated by components of the cable.  In 2010, when the cable was powered, there was no significant difference in abundance of skates near the cable compared to the number of skates 50 m away.

- The MARS cable has little or no detectable effect on the distribution and abundance of macrofaunal and megafaunal assemblages on a regional scale (e.g. kilometers).

  - Megafauna and macrofauna compared before and after cable installation among 3 control stations and 1 cable station at each of 3 depth regions (Shelf - <200 m, Neck – 200-500 m, Slope - >500 m) indicated very few potential changes in benthic biological patterns due to the MARS cable.

  - Natural spatial and temporal variation in the abundance and distribution of benthic macrofauna and megafauna appears to be greater than any detectable effects of the MARS cable.

INTRODUCTION

The Monterey Accelerated Research System (MARS) is an undersea cable from the Monterey Bay Aquarium Research Institute (MBARI) to a science ‘node’ at a depth of 891 m on the continental slope just outside Monterey Bay, California.  The system provides power and high data bandwidth for science instruments connected to the node via thin ‘extension’ cables deployed on the seabed by remotely operated vehicles (ROVs).  MARS is one of a few cabled ocean-observing systems that enable continuous, long-term science capabilities for ocean science with real-time communication, control, and data capture from offshore subsea sensor systems.

Figure 1.   View of the MARS cable, node, and potential science instruments over exaggerated bathymetry of Monterey Bay, Monterey Canyon, and the continental slope.  The science node is indicated as “MARS site”.

The main MARS cable was installed in March 2007 from the cable-laying ship Global Sentinel.  It stretches 51 km from shore to the science node, which is positioned in 891 m depth roughly 35 km from shore.  The cable was plowed in for most of its length, and jetted near the benthic node at the MARS Site (Figure 1) and around the HDD (horizontal directional drilling) conduit section where it dives under the shoreline.  Burial was not possible just below the continental shelf break (200 – 400 m depth) where carbonate and bedrock outcrops prevent complete burial of the cable.  The MARS science node was installed and powered briefly in February 2008, but failed due to a subsea connector.  The failed parts were recovered, repaired, and reinstalled in November 2008.  MARS has been fully operational since that time.

Figure 2.  Map of MARS cable (black line) environmental studies sites.  Colored circles represent regional sites for impact studies at Shelf (green), Neck (yellow), and Slope (blue) depths.  Red box indicates location for Skate transects.  Brown plus symbols (A-J) indicate transect locations for localized megafaunal studies.

Prior to MARS cable installation, an environmental impact report was prepared, including characterization of seabed biological communities along the cable route and initial sampling for future environmental impact assessment (2004).  This survey included characterization of the megafaunal animals (organisms identifiable in video recordings) and macrofaunal organisms (worms, crustaceans, etc., captured from sieved sediment samples) along the cable route. Subsequent to MARS cable installation, a Post-Lay Inspection and Burial (PLIB) survey of the entire route was conducted (March 16 to March 22, 2007 and June 7, 2007) and environmental impact assessmentsare required at ~18 month to 5-year intervals, including observations of the condition of the cable and potential effects on biological communities.   In this report, we present

data from environmental impact assessment surveys performed prior to cable installation, in 2007-2008following cable installation, and again in 2010.

# METHODS

# Cable Condition Survey

The position and condition of the cable was assessed by a ‘fly-over’ survey of the cable and cable route using the ROV Ventana.  The ROV was flown over the cable or cable route approximately 1-3 m over the seabed, using a cable-sensing system attached to the ROV toaccurately determine the position of the cable over the buried portion of the cable route.  Annotations of video observations included;

- megafauna present

- superficial condition of the seabed

- damage to the seabed related to cable installation

- burial of the cable

- condition of the burial trench (i.e. exposed, filled with mud, etc.), and

- if not buried, the condition of the cable (lying on seabed or suspended between seabed objects

# Seabed Observations and Sediment Sampling

Observations of the seabed and sample collection were performed using the ROV Ventana, supported by the R/V Point Lobosand ROVDoc Ricketts, supported by the R/VWestern Flyer. The main camera on each of the ROVs is an Ikegama high definition camera with a HA10Xt.2 Fujinon Lens mounted on a 3-axis pan and tilt capable of +/- 45o of tilt.  Two manipulator arms and a sample drawer provide space and manipulation capabilities for sediment sampling.  All available recorded video was annotated using MBARI’s computer annotation system, Video Information and Reference System (VARS).

# Video Transects

Figure 3.  Sampling protocol for cable and control sites at each depth.  Circle – 200 m diameter site.  Blue lines – 100 m long video transects.  Green circles – sediment core samples for macrofaunal assemblage characterization.  Red circles = cores for sediment characteristic measurements.

Quantitative estimates of the densities of seabed organisms and objects were obtained from the analysis of video transects.  For each video transect, the ROV camera was tilted toward the seabed and zoomed to provide a 1 m wide swath visible at the base of the image frame.  Each transect was run at ~0.1-0.2 ms-1 over a distance of 100 m.  Paired lasers (~29 cm apart) provided a reference scale for estimating the spatial dimensions of the video image. Voucher specimens were collected as needed for additional taxonomic study.

Video transects were annotated using the VARS annotation system in a quantitative manner to provide estimates of the density (# 100 m2) of identifiable objects and organisms.  Taxonomic identification of all megafauna was performed to the lowest practical taxonomic level.  Because identification of organisms from video can be difficult, we were conservative in assignment of taxonomic names.  All objects within a 1-1.5 m wide swath (width determined repeatedly during each transect using the paired laser system) were annotated (counted) along the length of the transect.  To avoid bias in counts of the number of organisms due to field of view distortion, animals in the upper third of the image were not counted.  Thus only those organisms passing through a 1 m wide swath in the lower 2/3 of the image were used for counts.  The density of objects and organisms over a single transect was used as a sample unit for further analyses.

Figure 4.  Collection of sediment core.

Sediment Samples

Samples of seabed sediments for faunal and geologic characteristics were collected using 6.9 cm diameter tube cores (area = 37.39 cm2), which penetrated the sediment to a depth of ~20 cm.  The top 5 cm of each core sample was washed gently through a 0.3 mm sieve using cold seawater.

Collected organisms were relaxed using a 7% solution of magnesium chloride (MgCl2), then preserved in a 4% formaldehyde (10% formalin) solution for several days. Samples were then rinsed with de-ionized water and stored in 70% ethanol for subsequent sorting and identification under a dissecting microscope.

Biological Communities

Local Effects of Cable Installation

Are there detectable differences in the abundances of animals living directly on or over the cable path compared to nearby areas not on the cable path?  This was evaluated using video transects at stations positioned at 5 km intervals along the entire cable route, comprising 10 cable stations (Figure 2).  At each station, a 100m long video transect was run over the cable route (impact), and a second 100 m long transect (control) was performed parallel to the cable, but 50 m away from it. For each transect, all identifiable organisms were identified and counted.   Data from impact and control treatments were then compared to assess differences in megafaunal assemblages potentially caused by the cable.

Regional Effects of Cable Installation

Geological and biological impacts potentially associated with the installation and presence of the MARS cable were investigated at three depths selected within three major habitat types or “depthregions” occupied by the cable (Figure 2).  These include 1) the continental shelf (Shelf), 2) the continental shelf break and upper slope (Neck), and 3) the continental slope region near the MARS benthic node (Slope). These depths represent the principal habitat types along the cable path.  Within each depthregion, a single ‘cable’ site was selected over the cable route, and three control sites were selected in the region, each ranging from 1-16 km from the cable route.

Each sampling site was considered as a ~200 m diameter circular area within which three replicate 100 m long ROV video transects were performedat randomly selected compass headings (Figure 3). All animals, as well as rocks, debris, and other items, were identified to the lowest possible taxon, and counted. In addition, replicate sediment cores (6.9 cm diameter) werecollected (Figure 4) at random locations along video transects to characterize macrofauna (n= 6 cores per site) and sediment characteristics (n=3 per site; %C, %N, grain size composition).

# Skate Abundance at Neck Cable Region

Early observations of aggregations of Longnose skatessuggested that they may associate with the cable, particularly in a localized area where small scarps and topographic depressions on the seafloor resulted in mild suspensions (2 -10 cm) of the MARS Cable.  To test the

Figure 5.  Exposure and depth of burial of the MARS cable after installation.  Upper map – buried versus exposed condition of cable.  Note exposure in Neck Cable region.  Lower panel – depth of burial of cable.  These observations were made during the 2007 survey.

hypothesis that this species (and perhaps others) were more (or less) abundant near suspended portions of the MARS cable, three replicate ROV video transects (100 m long) were performed along this portion of the cable in the Neck cable region, and compared to three similar control transects performed ~50 m from the cable-transects.

# Analytical Methods

Detection of differences in geologic and biologic parameters between cable and control sites with data available for both before and after cable installation were made using a BACI (Before-After, Control-Impact) analytical design (Stewart-Oaten et al. 1986; Underwood et al. 1994; Hewitt et al. 2001).  Using this design, individual 2-factor (Period (before, after), Treatment (cable, control)) comparisons were performed using permutation statistics available with PRIMER-6 and PERMANOVA+ (v.1.03; www.primer-e.com). This design was used for both multivariate and univariate data sets of biological or geological parameters. In most cases, raw counts of abundance (# 100 m2 megafauna; # m2 macrofauna) were transformed (4th root) prior to analysis to increase homogeneity of variances among groups and to reduce the influence of very abundant species.  Similarity matrices for Permanova were calculated using Bray-Curtis for multivariate tests, univariate tests and Multidimensional Scaling (MDS) analysis. Monte Carlo P-values (Anderson and Robinson 2003) were used to assess statistical significance.

Univariate tests were run for sediment characteristics (mean grain size and % organic carbon), with no transformation of raw data and Euclidean Distance as an overlap measure.

Faunal assemblage data were analyzed on a level of individual species and faunal groups (e.g. family, class).  For multivariate tests, all species or all taxonomic groups were analyzed together.  For univariate tests, only the most abundant species (~>1% of total faunal abundance) or faunal groups (~>3% of total faunal abundance) were analyzed.

Because of the large number of individual tests run, the likelihood of a type 1 error increases, and α (the likelihood of type 1 error) is often adjusted downward from 0.05 to a lower proportion based on the number of tests performed (Cabin and Mitchell 2000).  Unfortunately, decreasing the likelihood of type 1 errors also increases the likelihood of type II errors (accepting a false null hypothesis).  Thus, the use of such adjustments is questionable (Perneger 1998; Cabin and Mitchell 2000), and we have not used them in analyses of cable impact data.

# RESULTS& DISCUSSION

# ROV Dive Series

A total of 16 days of ROV dives using the R/V Point Lobos andROV Ventana were used to complete the first MARS cable environmental studies from November 30, 2007 to April 1, 2008; Appendix 1).  Three additional sea days were cancelled or postponed due to weather.  In the most recent study, we surveyed between January 8, 2010 and April 9, 2010 using the R/V Point Lobos andROV Ventana.  Deeper stations were survey usingR/V Western Flyer andthe ROVDoc Ricketts(Appendix 1).  We went to sea 16 days (34 ROV dives), three of which were aborted due to severe weather conditions. Additionally, some dives had to be cancelled due to poor visibility, and some sites were revisited 2-3 times checking for better conditions.

Observations including ROV surveys, transects, and sediment collections (tube cores) were completed during these cruises. In addition to surveying the length (~51 km) of the cable routefour times, the field and analysis team completed 158 quantitative video transects, and collected and analyzed 210 sediment cores (120 for macrofaunal analyses, 72 for sediment characteristics, 18 spare cores).

Figure 6.  Images of the seabed along the cable route.  A.  Sand ripples at 31 m depth, cable not visible (2008).  B. Cable on seabed at 226 m depth (2010).  C.  Skate (Raja rhina) aggregation at 303 m depth (2008).  D. Far fewer skates were present in 2010.  E.  Cable trench sediment infilling at 447 m depth (2010).  F.  Cable trench is filled near the MARS node at 867 m depth (2010).

Condition of Cable

# Inspection of the cable and underlying seabed indicated that there was no apparent change in the position of the cable 13 months and again at about 36 months (survey dates vary), after installation. Survey observations verified that the cable is still buried from 0-1 m beneath the surface of the seabed, as measured during the PLIB survey (2007; Figure 5). No evidence of trawling impact was observed during initial surveys, however the 2010 survey reveal trawling activity near the MARS node at 891 m (Figure 2). The main cable has not moved, however instrumentation attached to the MARS node has been damaged.  Three trawling incidents were documented between April 2009 and February 2010.There were no apparent changes along exposed sections, including portions of the cablewith minor suspensions due to small-scale topographic highs and lows (Figure 6), which are largely confined to the Neck depth (Figure 4). There is no evidence of strumming or other movement of the cable. As in the PLIB and 2008 surveys, the cable remains buried with no trace of habitat disturbance in shallow sandy areas (Figure 6a), which comprise roughly 50 percent of the cable route. In 2010, due to continuous storms, the water column in this shallow (< 50 m water depth) area of the cable route was so sediment-laden, that we used sonar to verify that the cable was not present on the seabed. An approximately 12 km long section was not surveyed between stations; we verified that the cable was fully buried at stations F and G (Figure 2).  In most other areas, habitat disturbance is less evident compared with the PLIB and 2008 surveys and the burial trench continues to fill in with time (Figure 6e), as indicated by the accumulation of soft, light-colored sediment in the trench.In 2008 there appeared to have been some minor redistribution of the tension in the cable.  From 120 – 300 m depth, sections of the surface-laid cable are sinking into the surficial sediments; these continue to sink and be covered with sediment in 2010.  As observed during the PLIB inspection, minor suspensions of the cable associated with topographic features between 136 and 450 m depth occurred, with the majority between 300 and 450 m.  Of the 23 suspensions, most are less than 15 cm in length and none are more than 30 cm above the seabed.  There were also 21 minor point suspensions.  These are areas where the cable is suspended < 10 cm above the seabed for a distance of <1 m each.  Most suspensions are less than 10 cm above the seabed.  None were higher than 30 cm above the seafloor and they all appear to be essentially unchanged in 2010.

Figure 7.  Mean sediment grain size (+/- SD) at control and cable (impact) stations for cable depth regions. *= significant difference between control and impact areas.

# Sediment Characteristics

In 2008 and 2010, the grain size and percentage carbon content of sediments varied significantly between treatments and sites for all three cable depth regions.  However, because no “Before” data were available, any effects of the cable on sediment grain size could only be inferred from comparisons between impact and control sites after installation of the cable.

# Sediment Grain Size

Mean sediment grain size continued to be highly variable among and within cable depth regions ranging from9 to almost 300 µm, with the coarsest sediments at Slope stations in 2010 (mean = 166 µm).  These

samples contained up to 38% coarse sand and very coarse sand, substantially more than in the 2008 samples (slope mean = 49 µm). Finer mean sizes characterized the Shelf (44 µm in 2008, 32 µm in 2010) and Neck(82.4 µm in 2008, 88 µm in 2010) depths.

Grain size at the shelf control sites were significantly smaller than at the impact site (p<0.001; Figure 7) in 2008, which might be expected in association with the installation of the cable. But because grain size also varied significantly between sites, irrespective of treatment (e.g. SC-1 < SC-2, SC-3; p<0.05), it is possible that the difference among stations was due to natural variability rather than any effects of the cable.In 2010, mean grain size was not significantly differentbetween the impact and control sites (p = 0.10; Figure 7).

The pattern of significant variation between treatments and among control sites was also observed at Neck and Slope depths (Figure 7).  Within the Neck cable depth region, control sitesdiffered significantly from the impactsitein 2008 (p<0.01); in contrast to Shelf depths, grain size at the Neck cable region was finer at the impact site than the control sites.In 2010, there was no significant difference at the neck region (p = 0.09).

Figure 8.  Mean percent organic carbon in surficial sediments for control and cable sites at three cable depth regions.   Comparisons of control vs. cable (impact) sites within each depth were nonsignificant (Shelf, p>0.6, Slope (p>0.3) and marginally significant at the Neck depth (p<0.05).  *= significant difference between control and impact areas.

The intersite variation in grain size observed at shallower depth also occurred at the Slope sites with both impact and two of the control sites showing a major change toward larger grain size in 2010; there was no significant difference between impact and treatment areas for either survey (p = 0.38 in 2008 and 2010).

Together, these results suggest strongly that variation in mean sediment grain size varies greatly among stations both within and between depths.  Although significant variation in grain size was detected between control and cable stations, it is not possible to determine if this difference is due to the effects of the cable or simply to naturally high station-to-station variation.

# Sediment Percent Carbon Content

In 2008, the mean organic carbon content of sediments ranged among sites from 0.39 to 1.11 percent (Figure 8), with the more organic-rich sediments generally found in the finer grain sizes (R2 = 0.28, p<0.06).  Analysis of the percentage carbon content of sediments at control and cable sites for each cable depth regionindicated marginally significant variation in carbon content between cable and control stations at the Neck regionin 2008 and again in 2010 and at the Slope region in 2010 (p<0.05). No significant variation between treatments at the Shelf (p>0.6 2008, p>0.4 2010) and Slope in 2008 (p>0.3) depths.  Variation among stations within each depth was often significant. These results indicate that there was little variation in the carbon content of sediments related to the installation of the cable.

Biological Characteristics

Megafaunal Assemblage

The seabed megafaunal assemblage along the cable route included 116 taxa observed in 156 ROV video transects (Appendix 1,2). The overall average density of megafauna averaged 158 ind./100 m2(Table 1). By phyla andtaxonomic groups, cnidarianswere most abundant,comprising just over 44 percent of the total megafaunal abundance andrepresented mainly by sea pens (Pennatulacea) and anemones (Actinaria). Echinoderms ranked second among phyla with almost 29 percent of the total abundance, particularly sea stars (Asteroidea), urchins (Echinoidea), and sea cucumbers (Holothuroidea) (Table 1). Fishes (Chordata) were the third most abundance phylum with over eight percent of the total abundance.

The top seven ranking megafaunal species accounted for nearly 60 percent of the total abundance.  Three common taxa (Figure 9a-c, Appendix 2) comprised 35 percent of the total megafauna.  Funiculina sp., a common sea pen at slope depths near 500 – 1000 m was the most abundant species (32 ind./100 m2), with 20.2 percent of the total megafaunal abundance.  Small anemones (Isosicyonis sp.), common at the deeper depths of the study, and Stronglylocentrotus fragilis, a common urchin at upper slope depths, ranked second and third, with 7.3 percent each.

Eighteen months after installation, there were 59 organisms attached to the cable in the neck region where the cable is on the seafloor (anemones Liponemabrevicornis, 39 individuals, and Metridiumfarcimen, 13, and the crinoidFlorometraserratisima,7).  We also observed four sea slug eggcases (Pleurobranchaeacalifornica) attached to the cable.After 36 months, there were 683 animals on the cable; 66 percent of them are semi-mobile and may be using the cable as temporary habitat (L. brevicornis345, F. serratissima 108, actinostolid anemones 189, M. farcimen 37, hydroids 4).

Table 1.  Summary of megafaunal abundance by phyla and groups. Abundance is listed as a mean (# ind. 100 m2, standard deviation (SD), and percent of total abundance (%).

Phylum / Group	Mean	SE	%

Cnidaria	69.89	12.96	44.17

Pennatulacea	50.28	9.46	31.78

Actiniaria	17.10	2.98	10.81

Ceriantharia	1.91	0.27	1.21

Gorgonacea	0.24	0.12	0.15

Corallimorphidae	0.13	0.05	0.08

Alcyonacea	0.09	0.04	0.06

Trachymedusae	0.11	0.02	0.06

Anthozoa	0.03	0.02	0.02

Echinodermata	45.59	14.87	28.81	Asteroidea	15.58	3.10	9.85

Echinoidea	11.60	3.31	7.33

Holothuroidea	11.06	5.17	6.99

Ophiuroidea	7.14	3.16	4.51

Crinoidea	0.21	0.13	0.13

Vertebrata	14.62	6.29	8.34

Scorpaenidae	5.58	0.92	3.53

Pleuronectiformes	4.52	0.61	2.86

Zoarcidae	1.69	3.09	0.33

Agonidae	0.44	0.13	0.28

Stichaeidae	0.42	0.21	0.27

Phylum / Group	Mean	SE	%

Merlucciidae	0.37	0.15	0.24

Osteichthyes	0.29	0.11	0.18

Macrouridae	0.19	0.09	0.12

Rajiformes	0.21	0.06	0.13

Myxinidae	0.14	0.52	0.09

Zaniolepididae	0.12	0.05	0.07

Moridae	0.08	0.04	0.05

Liparidae	0.05	0.02	0.03

Scyliorhinidae	0.05	0.03	0.03

Anoplopomatidae	0.04	0.02	0.02

Embiotocidae	0.04	0.03	0.02

Ophidiidae	0.04	0.02	0.02

Squalidae	0.03	0.03	0.02	Alepocephalidae	0.01	0.01	0.01

Chimaeridae	0.01	0.01	0.01

Hexagrammidae	0.14	0.06	0.01

Osmeridae	0.15	0.07	0.01

Torpedinidae	0.01	0.01	0.01

Detritus (plant/kelp)	10.92	1.69	6.9

Mollusca	12.48	3.06	7.89

Gastropoda	12.01	2.90	7.59

Cephalopoda	0.47	0.16	0.30

Arthropoda	2.86	0.88	1.81

Decapoda	2.28	0.67	1.44

Anomura	0.50	0.17	0.32

Caridea	0.08	0.04	0.05

Porifera	1.58	0.50	1.00

Urochordata	0.10	0.08	0.07

Brachipoda	0.06	0.06	0.04

Echiura	0.05	0.03	0.03

Annelida (Polychaeta)	0.05	0.02	0.03

Grand Total	158.22

Local Effects of Cable Installation – Megafauna

Figure 9.  Common megafaunal and macrofaunal animals along the MARS Cable route.  A-C: Megafauna:  A. Rathbunastercalifornicus (sea star).  B.  Funiculina sp. (sea pen).  C.  Stronglylocentrotus fragilis, (urchin).  D-F: Macrofauna: D. Cossura sp. (polychaete).  E.  Oligochaeta.  F.  Prionospio sp. (polychaete).  These taxa are some of the most abundant organisms observed in video (megafauna) or collected in sediment cores (macrofauna).

Little variation in the megafaunal assemblage was detected between video transects directly over the cable route and parallel transects 50 m from the cable (Figure 10, 11).  Multivariate tests comparing cable and control treatments for all species or all groups were non-significant (p = 0.96).  Likewise, univariate tests evaluating the abundance of faunal groups or individual species indicated no significant variation in the megafaunal assemblage from directly over the cable to 50 m away.  Thus, local variation in the megafaunal assemblage very near the cable was largely undetectable.

One important exception to this pattern was observed near 300 m depth in 2008 in the Neck region, where the cable is occasionally suspended 2-10 cm above the seabed between rocks for short distances.

Figure 10.  Variation in abundance of megafaunal groups along the cable route.  Paired stations show little variation in densities of megafauna over 10 stations along the cable (p = 0.96). Cable = directly over the cable route.  Control = 50 m away from and parallel to the cable.

Figure 11.   MDS plot based on Bray-Curtis Similarity Index for megafaunal assemblages at local stations sampled in 2010, showing high similarity between biological communities at control and impact sites along the cable route and 50 m away.  Letters refer to stations along the cable route (A = deepest station at 890 m, J = shallowest station at 19 m; Figure 2).

Within this 300 long segment of the cable route, the density of Longnose skates (Raja rhina) was anomalously high (Figure 6c).  The mean density of R. rhina was 33 per 100 m2 over the cable, but only 0.3 per 100 m2 at nearby control areas (p<0.027). The densest aggregations were concentrated along a 75 - 100 m section, with theskates resting on the seafloor within 5 - 10 m of the cable. We also noted somewhat higher than normal numbers of the elasmobranchs Parmaturusxaniurus (catsharks) and Hydrolaguscolliei(spotted rat fish) in this general area of the cable route, but off the transect. In 2010, there was no longer a statistically significant difference in the number of skates present over the cable (Figure 6d);the mean number of R. rhinawas9.7 per 100 m2, vs. 6.3 per 100 m2 at the nearby control transects (p=0.90).  The abundance of other elasmobranchs in the area appeared normal.

A number of marine fishes, especially elasmobranchs are known to sense electromagnetic fields using electroreceptors as a method of prey detection (Bullock 1982).  The suspended MARS cable very likely produced a weak electromagnetic field as local ocean currents flow through the Earth’s magnetic field and around the cable (Sanford, 1971).  This is possible even though the cable was not energized during the 2008 video survey. We noted that while the cable was taut and 2-10cm off the seafloor in other areas with topographic highs and lows, no other skate aggregations were seen. The combination of topography (small scarps and sediment depressions unique to this area), natural distribution of the animals, and a mild electrical field may have contributed to the aggregation. This electric field is apparently detectable by R. rhina, which aggregated near the cable.  Electric fields from seabed cables including telecommunications cables and power distribution cables (e.g. coastal windfarms) are expected to have ecological effects due to their effects on the behavior of various species capable of electroreception (Gill 2005).

Regional Effects of Cable Installation - Megafauna

The installation and presence of the MARS cable appears to have mild to benign effects on the structure of the megafaunal assemblages on the scale of kilometers, based on the results of samples from cable and control sites before and after cable installation (Figures 12-15).  Using both multivariate and univariate analyses, fewstatistically significant differences in the densities of megafauna were detected in relation to the installation of the cable.

For these BACI analyses, a significant effect of the installation of the cable would be detected by finding a statistically significant Period x Treatment (PxT) interaction term for the abundance of a particular taxon (for univariate or multivariate tests).  This result would indicate that any change in the abundance of the taxon changed between periods (i.e. Before and After cable installation) at the cable stations was different than changes in abundance at control stations.

At Shelf, Neck, and Slope depths, multivariate comparisons (i.e. comparisons between the entire megafaunal assemblage) indicated no significant Period x Treatment interaction terms (p>0.71, 0.17, 0.60, for Shelf, Neck, and Slope, respectively), indicating that there was little change in the overall megafaunal community that is attributable to the installation of the cable (Table 2).  These tests also indicated that there was significant variation in megafaunal abundance for the main factors of Periods or Treatments, or both.  Overall, these results indicate that most of the variation in the abundance and distribution of megafauna is due to natural variability between stations or periods – in other words, natural variation in megafaunal abundance among control stations was equal or greater than that measured between control and cable stations.  These results were found for multivariate tests using either species-level data or higher taxon-level data.

There were few changes in the abundances of individual species or higher taxa in relation to the installation of the MARS cable, based on univariate tests to examine cable impacts.  Even though there was frequently significant variation in the abundance of species or higher taxa between Periods or Treatments (Table 2), very few significant PxT interaction terms were found, indicating little effect of the MARS cable. Univariate tests were conducted for most major taxa shown in Figure 11-13.  Among these, the PxT interaction term was significant for only 6 of 28tests. Phyllospadixsp. (surf grass) is detritus that was not observed at the Slope cable site during the pre-installation survey, but occurred there during the 2008 and 2010 surveys. It may have accumulated in the trench after cable installation. Kelp and surf grass were not observed collecting near the exposed cable in the Neck region. This is a high-energy region and detritus is subject to local and seasonal bottom currents. Marine snails (Gastropoda)and the actiniaria (anemones) that attach to them(Isoscyonis sp.) may have been affected by the cable installation, as indicated by the significant PxT interactions (Table 2).  Snails were low in abundance at the Slope cable station prior to cable installation (0.3 per 100 m2), but increased by 2008 to a mean of 45 ind. per 100 m2 and to 153 per 100 m2 in 2010. During both periods, snail densities at the control stations averaged 17-22 ind. per 100 m2.  While this may represent natural variability, it is likely that snails aggregated near the cable stations and in the cable trench in association with increased abundance of detritus and other material. The PxT interaction term was also highly significant at the Slope Depth for the deep-sea cucumber, Pannychiamoseleyi. It was initially abundant at the Slope cable station (40 ind. 100 m2), but sparseat Slope control stations (mean = 0.3 ind. 100 m2).  During the 2008 survey, this sea cucumber was not observed at any Slope Depth stations. Because P. moseleyi appears to vary considerably between stations and periods, the observed changes in density may be due simply to natural variability rather than specific cable effects. Large fluctuations in the abundance of this species have been observed in our local benthic studies; this variation is evident at the Neck depth stations (Table 2).We have observed that the density of the sea pens (Pennatulacea) Funiculina sp. and Umbellula lindahlivaries widely on a scale of only 10’s of meters.In 2010, we observed 4.03 Funiculina sp. per 100 m2 at the MARS node station (SLI-1) vs. 0.24-1.07 per 100 m2 at the control site.

Although few effects of cable installation were detectable, significant main effects (Treatment or Period) were detected for some taxa and cable depth regions (Table 2).  Closer examination of these frequently indicated that significant Treatment effects were related to statistically significant variation among control stations as well as differences between control and impact stations.  Thus, natural variability in megafaunal abundance and distribution appears to be as large as differences in abundance related to the cable.

Figure 13. Variation in megafaunal abundance among treatments and periods at Shelf depth stations. BACI analysis indicated no overall significant effect of cable installation (p>0.05) in this region for higher-level taxonomic groups.

Figure 12.  Variation in megafaunal assemblage among periods and treatments at Neck depth stations. BACI analysis indicated no overall significant effect of cable installation (p>0.05) in this region for higher-level taxonomic groups.

Figure 14. Variation in megafaunal abundance among treatments and periods at Slope depth stations. BACI analysis indicated no overall significant effect of cable installation (p>0.05) in this region for higher-level taxonomic groups.

Figure 15. MDS plot based on Bray-Curtis Similarity Index. Regional megafaunal communities remained at least 60 percent similar in a comparison of community structure before the MARS cable was installed, and at 18 and 36 months post-installation.  Stations clustered based on depth, with shelf, neck and slope stations most similar to each other.

Table 2.  Summary of univariate BACI analysis for megafaunal taxa for all cable depths.  P=Period, T= Treatment, PxT = Period x Treatment interaction term.  A significant PxT term suggests an effect of cable installation.  Comments explain patterns of results or propose possible factors influencing differences detected among treatments.

SHELF	NECK	SLOPE

Taxon	P	T	PxT	P	T	PxT	P	T	PxT	Comment

Higher Taxa

Multivariate Tests

All Groups	**	**	ns	*	**	ns	*	**	ns

Univariate Tests

Actinaria (anemones)	ns	ns	ns	ns	**	ns	**	*	*	> after cable inst.

Ceriantharia (tube anemones)	ns	*	ns	ns	*	ns	ns	ns	ns	Sig. station variability

Pennatulacea (sea pens)	ns	ns	ns	ns	ns	ns	**	ns	ns	Sig. station variability

Phyllospadix (surfgrass detritus)	**	*	ns	**	**	*	**	ns	**	> after cable inst.

Asteroidea (sea stars)	ns	ns	ns	ns	**	ns	ns	ns	ns	Sig. Station variability

Echinoidea (sea urchins)	-	-	-	ns	*	ns	ns	ns	ns	Cable > Control

Holothuroidea (sea cucumbers)	*	ns	ns	ns	ns	ns	**	ns	**	Natural variability?	-

Ophiuroidea (brittle stars)	**	ns	ns	-	-	-	ns	ns	ns	Natural variability?

Gastropoda (snails)	ns	ns	ns	ns	ns	ns	**	ns	**	> after cable inst.

Pleuronectiformes (flatfishes)	ns	ns	ns	*	**	**	*	ns	ns	Control > Cable

Species

Multivariate Tests

All Species	**	**	ns	*	**	ns	**	**	*

Univariate Tests

Funiculina sp. (sea pen)	-	-	-	-	-	-	ns	**	ns	Sig. station variation

Rathbunastercalifornicus(sea star)	ns	ns	ns	ns	**	ns	ns	ns	ns	Cable > Control

Stronglylocentrotus fragilis(urchin)	-	-	-	ns	*	ns	ns	ns	ns	Cable > Control

Psolussquamatus (sea cucumber)	-	-	-	ns	ns	ns	-	-	-

Isoscyonis sp. (anemone)	-	-	-	-	-	-	**	*	*	> after cable inst.

Umbellulalindahli (sea pen)	-	-	-	-	-	-	ns	**	ns	Cable > Control

Sebastolobussp. (fish)	-	-	-	ns	ns	ns	ns	*	ns

Pannychiamoseleyi (sea cucumber)	-	-	-	-	-	-	**	**	**	Natural variability?

Actinostolidae (anemone)	ns	ns	ns	ns	*	ns	**	*	*	> after cable inst.

Florometraserratissima	-	-	-	ns	**	ns	-	-	-	> after cable inst.

Figure 9.  Common megafaunal and macrofaunal animals along the MARS Cable route.  A-C: Megafauna:  A. Rathbunastercalifornicus (sea star).  B.  Funiculina sp. (sea pen).  C.  Stronglylocentrotus fragilis, (urchin).  D-F: Macrofauna: D. Cossura sp. (polychaete).  E.  Oligochaeta.  F.  Prionospio sp. (polychaete).  These taxa are some of the most abundant organisms observed in video (megafauna) or collected in sediment cores (macrofauna).

Regional Effects of Cable Installation – Macrofauna

Macrofaunal assemblages (Table 3) along the cable route appear to be largely unaffected by the installation of the cable, as found for the megafaunal assemblages. Multivariate tests indicated that there was no detectable cable effect at the Neck or Slope depths (i.e. PxT = ns), even though there were significant main effects (Period, Treatment) found at one or both depths.  At the Shelf depth, however, a significant PxT interaction term was detected for both the higher taxa and species tests (Table 4). Owing to the overwhelming dominance of polychaete worms in the macrofauna, at the Shelf Depth in particular, this group has a large influence on the outcome of this multivariate test.The abundance of polychaetes, and thus the macrofauna in general, increased at after cable installation, but increased far more at the Shelf cable station than at Shelf control stations (Figure 14). Simultaneously, the abundance of most other macrofaunal taxa at control stations decreased. While it is possible that the installation of the cable affected local conditions favorable to polychaetes in particular (e.g. increased detritus) it seems equally or more likely that other factors (e.g. natural variability) have greater influence on polychaete abundance.

These results indicated few detectable effects of the MARS cable on seabed biology are similar to results reported in other studies.  Kogan et al. (2003) reported that few statistically significant effects of the ATOC submarine cable were detectable.  They noted that the major effect of the cable was on organisms that attached to it, especially anemones, and also reported erosion of the seabed by strumming of the exposed cable at shallow depths.

Table 3.  Mean density of macrofaunal taxa, by group over all samples.  Density is listed as number per core (area = 37.39 cm2).  % indicates the percentage of the taxon of the total macrofaunal abundance.

| Phylum | Group | Mean (#/core) | SE | % |
| --- | --- | --- | --- | --- |
| Annelida |  | 25.572 | 1.664 | 50.705 |
|  | Polychaeta | 23.668 | 1.370 | 46.930 |
|  | Oligochaeta | 1.904 | 0.294 | 3.775 |
| Arthropoda |  | 15.858 | 1.847 | 31.444 |
|  | Amphipoda | 10.084 | 1.023 | 19.996 |
|  | Tanaidacea | 1.939 | 0.311 | 3.845 |
|  | Isopoda | 1.439 | 0.172 | 2.853 |
|  | Ostracoda | 1.146 | 0.179 | 2.273 |
|  | Cumacea | 1.178 | 0.132 | 2.336 |
|  | Mysida | 0.063 | 0.024 | 0.124 |
|  | Decapoda | 0.008 | 0.006 | 0.016 |
|  | Pycnogonida | 0.001 | 0.001 | 0.001 |
| Mollusca |  | 4.909 | 0.558 | 9.733 |
|  | Bivalvia | 3.819 | 0.367 | 7.573 |
|  | Scaphopoda | 0.456 | 0.069 | 0.904 |
|  | Gastropoda | 0.481 | 0.076 | 0.954 |
|  | Aplacophora | 0.136 | 0.037 | 0.270 |
|  | Polyplacophora | 0.016 | 0.009 | 0.032 |
| Echinodermata |  | 2.530 | 0.520 | 5.017 |
|  | Ophiuroidea | 2.480 | 0.494 | 4.918 |
|  | Echinoidea | 0.014 | 0.008 | 0.028 |
|  | Holothuroidea | 0.036 | 0.018 | 0.072 |
| Nemertea |  | 1.122 | 0.146 | 2.224 |
| Cnidaria |  | 0.299 | 0.116 | 0.593 |
|  | Actiniaria | 0.265 | 0.102 | 0.526 |
|  | Hydrozoa | 0.034 | 0.014 | 0.068 |
| Sipuncula |  | 0.052 | 0.016 | 0.103 |
| Echiura |  | 0.040 | 0.017 | 0.079 |
| Enteropneusta |  | 0.034 | 0.024 | 0.068 |
| Phoronida |  | 0.011 | 0.008 | 0.023 |
| Platyhelminthes |  | 0.006 | 0.006 | 0.011 |
| Total |  | 50.432 |  |  |

Table 4.  Summary of Univariate BACI analysis for macrofaunal taxa.  P=Period, T= Treatment, PxT = Period x Treatment interaction term.  A significant PxT term suggests an effect of cable installation.  Comments explain patterns of results or propose possible factors influencing differences detected among treatments. C= Control Stations, I = Cable Stations, B = Before Stations, A = After Stations, * = p<0.05, **= p<0.01, - indicates absent from region

SHELF	NECK	SLOPE

Taxon	P	T	PxT	P	T	PxT	P	T	PxT	Comment

Higher Taxa

Multivariate Tests

All Groups	**	*	*	ns	**	*	*	**	ns

Univariate Tests

Polychaeta (worms)	**	ns	**	ns	ns	ns	ns	ns	ns	> after cable inst.

Amphipoda (crustacea)	ns	ns	ns	ns	**	ns	**	*	*	Impact>control,

Control>Impact all

periods

Bivalvia (clams)	**	ns	ns	ns	**	ns	ns	*	ns	Natural variation

Oligochaeta (worms)	ns	ns	ns	ns	*	ns	*	ns	ns	Impact>control,

>all stations after inst.

Tanaidacea (crustacea)	**	ns	ns	ns	**	ns	ns	ns	ns	> all stations after inst,

Impact>control

Isopoda (crustacea)	ns	ns	ns	ns	ns	*	ns	ns	ns

Ostracoda (crustacea)	ns	ns	ns	ns	**	ns	ns	ns	ns	Impact>control

Ophiuroidea (brittle stars)	**	ns	ns	ns	ns	ns	ns	ns	ns	> after cable inst.

all stations, natural

variation.

Figure 14. Variation in the abundance of macrofaunal taxa (higher groups) at the Shelf cable depth region among treatments and periods.

Figure 15. Variation in the abundance of macrofaunal taxa (higher groups) at the Neck cable region among treatments and periods.

Figure 16. Variation in the abundance of macrofaunal taxa (higher groups) at the Slope cable zone among treatments and periods.

Other factors

Several other factors may have influenced the variability observed in the abundance and distribution of benthic megafauna and macrofauna in relation to the installation of the MARS Cable.  First, the geological and biological sampling program included a few samples collected as early 1999 and 2001, which were included in the ‘Before’ samples collected principally during 2008.  Therefore, estimates of faunal abundance during this extended ‘Before’ sampling period reflect the natural variability of local benthic communities. Considering that the 2007-2008 and 2010 samples were collected over only a few months, they reflect a simpleshort-term “snap shot” the benthic faunal communities.

Second, although the vast majority of sediment samples were collected using the same method (6.9inch diameter tube core), 4 samples (from 1999) were collected using a Smith-MacIntyre Grab (0.1 x 0.1 m).  The abundances of macrofauna derived from these samples were adjusted to 37.39 cm2 (the area of a tube core), but differences in the efficiency of the two collection devices is likely to affect the results.

Third, there were no adjustments of probability levels to account for the large number of statistical tests.  Over 100 statistical tests were performed, using an α of 0.05 – that is the probability of a type 1 error (rejecting a true null hypothesis) is 1 in 20.  Thus, for 100 statistical tests, one would by chance detect a significant effect (e.g. Period x Treatment interaction term indicating an effect of the MARS cable) approximately 5 times.  There are methods of reducing α to further reduce the probability of a type 1 error, but this is generally avoided, since it also increases type II errors (the acceptance of a false null hypothesis) (Cabin and Mitchell 2000).

CONCLUSIONS

Inspection of the MARS cable, coupled with a sampling program to evaluate changes in geological and biological conditions on local and regional scales with respect to the installation of the cable indicate little detectable influence of the cable.  The most conspicuous evidence of cable installation is the cable exposed on the seabed for a short distance where it could not be buried.  Analyses of the geological and biological sampling program indicate the following:

- Over most of its length, the cable remains buried, with little evidence of change since installation

- No detectable changes in sediment characteristics (mean grain size, or percent organic carbon) can be attributed to the MARS cable

- Local variation in benthic megafaunal communities within 50-100 m of the MARS cable is minor or undetectable.

  - The abundances of most animals observed did not differ between the area over the cable route and 50 m away

  - Longnose skates (Raja rhina) were significantly more abundant near regions where the MARS cable is suspended over topography (~300 m depth) in 2008.They are likely to respond to electromagnetic fields generated by the cable, although their numbers were not significantly different in 2010.

- The MARS cable has little effect on the distribution and abundance of macrofaunal and megafaunal assemblages on a regional scale (e.g. kilometers).

  - Megafauna and macrofauna compared before and after cable installation among 3 control stations and 1 cable station at each of 3 depth regions (Shelf - <200 m, Neck – 200-500 m, Slope - >500 m) indicated relatively few potential changes in benthic biological patterns due to the MARS cable.

  - Natural spatial and temporal variation in the abundance and distribution of benthic macrofauna and megafauna appears to be greater than any detectable effects of the MARS cable.

ACKNOWLEDGEMENTS

This study was funded by the National Science Foundation (grants OCE-0222650, OCE-0739828) and the David and Lucille Packard Foundation. We are grateful for the expertise and support of the crews of the R/V Point Lobos, ROV Ventana, ROVDoc Ricketts, and R/VWestern Flyer.

REFERENCES

Anderson, M.J., Robinson, J. 2003.  Generaliseddiscriminant analysis based on distances.  Aust. &New Zealand J. of Statistics, 45: 301-318.

Bullock, T.H. 1982.  Electroreception.  Ann. Rev. of Neuroscience, 5: 121-170.

Cabin, R.J., Mitchell, R.J.  2000.  To Bonferroni or not to Bonferroni:  When and how are the questions.  Bull. Ecol. Soc. Amer., 81(3): 246-248

Gill, A.B. 2005.  Offshore renewable energy: ecological implications of generating electricity in the coastal zone.  J. Applied Ecology 42, 605-615.

Hewitt, J.E., Thrush, S.E., Cummings, V.J. 2001.  Assessing environmental impacts: effects of spatial and temporal variability at likely impact scales.  Ecological Appl. 11(5): 1502-1516.

Kogan, I., Paull, C.K., Kuhnz, L., Burton, E.J., VonThun, S., Greene, H.G., Barry, J.P. 2003.  Environmental impact of the ATOC / Pioneer Seamount submarine cable.Monterey Bay National Marine Sanctuary.

Perneger, T.V. 1998.  What’s wrong with Bonferroniadjustments.  BMJ, 316: 1236-38.

Stewart-Oaten, A., Murdoch, W.W. 1986.  Environmental impact assessment: “pseudoreplication “ in time?  Ecology, 67(4): 929-940.

Underwood, A.J. 1994.  On beyond BACI: sampling designs that might reliably detect environmental disturbances.  Ecol. Appl., 4(1): 4-15.

Appendix 1.  ROV video transect information.  Transect Code (Tr. Code), Station, Cable depth region (Region), Treatment (control or cable location), Period (before or after (date) cable installation), depth in meters, Date, Dive number, transect length (m), and estimated accuracy of navigation used to calculate transect length.

| Tr. Code | Station | Region | Treatment | Period | Latitude | Longitude | Depth (m) | Date | Dive # | Length (m) | Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A-A-2008 | A | All | Cable | 2008 | 36.756899 | -122.188391 | 710 | 12/12/07 | V3139 | 115 | good |
| A-A-2010 | A | All | Cable | 2010 | 36.756645 | -122.188324 | 720 | 1/27/10 | V3500 | 100 | good |
| A-B-2008 | A | All | Control | 2008 | 36.756899 | -122.18771 | 724 | 12/12/07 | V3139 | 114 | good |
| A-B-2010 | A | All | Control | 2010 | 36.757187 | -122.187790 | 713 | 1/27/10 | V3500 | 100 | good |
| B-A-2008 | B | All | Cable | 2008 | 36.79973 | -122.186883 | 435 | 1/29/08 | V3164 | 97 | good |
| B-A-2010 | B | All | Cable | 2010 | 36.800182 | -122.186560 | 431 | 1/29/10 | V3506 | 100 | good |
| B-B-2008 | B | All | Control | 2008 | 36.799712 | -122.18629 | 431 | 1/29/08 | V3164 | 97 | good |
| B-B-2010 | B | All | Control | 2010 | 36.799267 | -122.186540 | 436 | 1/29/10 | V3506 | 100 | good |
| C-A-2008 | C | All | Cable | 2008 | 36.836974 | -122.158251 | 172 | 1/31/08 | V3167 | 83 | good |
| C-A-2010 | C | All | Cable | 2010 | 36.836964 | -122.158460 | 168 | 1/13/10 | V3488 | 100 | good |
| C-B-2008 | C | All | Control | 2008 | 36.83734 | -122.158864 | 162 | 1/31/08 | V3167 | 87 | good |
| C-B-2010 | C | All | Control | 2010 | 36.836964 | -122.157170 | 153 | 1/13/10 | V3488 | 100 | good |
| D-A-2008 | D | All | Cable | 2008 | 36.857358 | -122.10881 | 95 | 2/8/08 | V3169 | 100 | good |
| D-A-2010 | D | All | Cable | 2010 | 36.857227 | -122.109116 | 92 | 1/28/10 | V3503 | 100 | good |
| D-B-2008 | D | All | Control | 2008 | 36.856905 | -122.108491 | 94 | 2/8/08 | V3169 | 100 | good |
| D-B-2010 | D | All | Control | 2010 | 36.857254 | -122.107834 | 92 | 1/28/10 | V3503 | 100 | good |
| E-A-2008 | E | All | Cable | 2008 | 36.879787 | -122.059782 | 72 | 4/1/08 | V3186 | 100 | good |
| E-A-2010 | E | All | Cable | 2010 | 36.879307 | -122.061250 | 73 | 2/25/10 | V3526 | 100 | good |
| E-B-2008 | E | All | Control | 2008 | 36.880117 | -122.060248 | 71 | 4/1/08 | V3186 | 111 | good |
| E-B-2010 | E | All | Control | 2010 | 36.880060 | -122.060875 | 72 | 2/25/10 | V3526 | 100 | good |
| F-A-2008 | F | All | Cable | 2008 | 36.885715 | -122.00602 | 48 | 4/1/08 | V3186 | 100 | good |
| F-A-2010 | F | All | Cable | 2010 | 36.885746 | -122.005420 | 48 | 4/9/10 | V3551 | 100 | good |
| F-B-2008 | F | All | Control | 2008 | 36.885231 | -122.006393 | 49 | 4/1/08 | V3186 | 100 | good |
| F-B-2010 | F | All | Control | 2010 | 36.886240 | -122.006610 | 48 | 4/9/10 | V3551 | 100 | good |
| G-A-2008 | G | All | Cable | 2008 | 36.883813 | -121.949321 | 39 | 1/8/08 | V3149 | 100 | unknown |
| G-A-2010 | G | All | Cable | 2010 | 36.883705 | -121.949730 | 39 | 4/9/10 | V3552 | 100 | good |
| G-B-2008 | G | All | Control | 2008 | 36.883895 | -121.949741 | 40 | 1/8/08 | V3149 | 100 | unknown |
| G-B-2010 | G | All | Control | 2010 | 36.884678 | -121.950645 | 37 | 4/9/10 | V3552 | 100 | good |
| Tr. Code | Station | Region | Treatment | Period | Latitude | Longitude | Depth (m) | Date | Dive # | Length (m) | Accuracy |
| H-A-2008 | H | All | Cable | 2008 | 36.868305 | -121.895886 | 44 | 1/28/08 | V3163 | 100 | poor nav |
| H-A-2010 | H | All | Cable | 2010 | 36.868030 | -121.896835 | 46 | 4/9/10 | V3553 | 100 | good |
| H-B-2008 | H | All | Control | 2008 | 36.867867 | -121.896324 | 44 | 1/28/08 | V3163 | 100 | good |
| H-B-2010 | H | All | Control | 2010 | 36.868860 | -121.897860 | 44 | 4/9/10 | V3553 | 100 | good |
| I-A-2008 | I | All | Cable | 2008 | 36.848015 | -121.845586 | 26 | 1/22/10 | V3173 | 100 | unknown |
| I-A-2010 | I | All | Cable | 2010 | 36.848717 | -121.847100 | 26 | 4/9/10 | V3554 | 100 | good |
| I-B-2008 | I | All | Control | 2008 | 36.847485 | -121.84478 | 26 | 1/22/10 | V3173 | 100 | poor |
| I-B-2010 | I | All | Control | 2010 | 36.849445 | -121.847560 | 26 | 4/9/10 | V3554 | 100 | good |
| J-A-2008 | J | All | Cable | 2008 | 36.815585 | -121.807344 | 20 | 3/31/08 | V3184 | 107 | good |
| J-A-2010 | J | All | Cable | 2010 | 36.816120 | -121.807730 | 20 | 4/9/10 | V3555 | 100 | good |
| J-B-2008 | J | All | Control | 2008 | 36.816038 | -121.806923 | 20 | 3/31/08 | V3184 | 107 | good |
| J-B-2010 | J | All | Control | 2010 | 36.817257 | -121.808020 | 19 | 4/9/10 | V3555 | 100 | good |
| NC1-A-2005 | NC-1 | Neck | Control | Before | 36.789158 | -122.12428 | 401 | 7/13/05 | V2687 | 100 | unknown |
| NC1-A-2008 | NC-1 | Neck | Control | 2008 | 36.788543 | -122.117341 | 399 | 1/30/08 | V3165 | 109 | good |
| NC1-A-2010 | NC-1 | Neck | Control | 2010 | 36.788597 | -122.116370 | 402 | 1/8/10 | V3483 | 100 | good |
| NC1-B-2005 | NC-1 | Neck | Control | Before | 36.789383 | -122.12514 | 400 | 7/13/05 | V2687 | 100 | unknown |
| NC1-B-2008 | NC-1 | Neck | Control | 2008 | 36.789039 | -122.117852 | 394 | 1/30/08 | V3165 | 96 | good |
| NC1-B-2010 | NC-1 | Neck | Control | 2010 | 36.788810 | -122.115770 | 401 | 1/8/10 | V3483 | 100 | good |
| NC1-C-2005 | NC-1 | Neck | Control | Before | 36.788088 | -122.121619 | 400 | 7/13/05 | V2687 | 50 | unknown |
| NC1-C-2008 | NC-1 | Neck | Control | 2008 | 36.789491 | -122.116151 | 386 | 1/30/08 | V3165 | 100 | poor |
| NC1-C-2010 | NC-1 | Neck | Control | 2010 | 36.789043 | -122.116936 | 392 | 1/8/10 | V3483 | 100 | good |
| NC2-A-2005 | NC-2 | Neck | Control | Before | 36.803011 | -122.122515 | 196 | 8/1/05 | V2696 | 100 | unknown |
| NC2-A-2008 | NC-2 | Neck | Control | 2008 | 36.799381 | -122.107564 | 207 | 1/30/08 | V3165 | 100 | good |
| NC2-A-2010 | NC-2 | Neck | Control | 2010 | 36.798930 | -122.107700 | 205 | 1/8/10 | V3483 | 100 | good |
| NC2-B-2005 | NC-2 | Neck | Control | Before | 36.803356 | -122.123501 | 197 | 8/1/05 | V2696 | 100 | unknown |
| NC2-B-2008 | NC-2 | Neck | Control | 2008 | 36.799928 | -122.108486 | 189 | 1/30/08 | V3165 | 100 | good |
| NC2-B-2010 | NC-2 | Neck | Control | 2010 | 36.798664 | -122.106720 | 208 | 1/8/10 | V3483 | 100 | good |
| NC2-C-2005 | NC-2 | Neck | Control | Before | 36.803646 | -122.124485 | 198 | 8/1/05 | V2696 | 100 | unknown |
| NC2-C-2008 | NC-2 | Neck | Control | 2008 | 36.799216 | -122.108455 | 195 | 1/30/08 | V3165 | 100 | good |
| NC2-C-2010 | NC-2 | Neck | Control | 2010 | 36.798023 | -122.107780 | 222 | 1/8/10 | V3483 | 100 | good |
| NC3-A-2006 | NC-3 | Neck | Control | Before | 36.842645 | -122.189108 | 361 | 10/3/06 | V2899 | 100 | unknown |
| NC3-A-2008 | NC-3 | Neck | Control | 2008 | 36.842894 | -122.187867 | 361 | 1/24/08 | V3162 | 100 | good |
| NC3-A-2010 | NC-3 | Neck | Control | 2010 | 36.842316 | -122.187260 | 358 | 1/14/10 | V3490 | 100 | good |
| Tr. Code | Station | Region | Treatment | Period | Latitude | Longitude | Depth (m) | Date | Dive # | Length (m) | Accuracy |
| NC3-B-2006 | NC-3 | Neck | Control | Before | 36.84212 | -122.189979 | 368 | 10/3/06 | V2899 | 100 | unknown |
| NC3-B-2008 | NC-3 | Neck | Control | 2008 | 36.842918 | -122.187669 | 352 | 1/24/08 | V3162 | 100 | good |
| NC3-B-2010 | NC-3 | Neck | Control | 2010 | 36.841690 | -122.186264 | 360 | 1/14/10 | V3490 | 100 | good |
| NC3-C-2008 | NC-3 | Neck | Control | 2008 | 36.842812 | -122.186766 | 356 | 1/24/08 | V3162 | 100 | good |
| NC3-C-2010 | NC-3 | Neck | Control | 2010 | 36.842228 | -122.185660 | 349 | 1/14/10 | V3490 | 100 | good |
| NI1-A-1999 | NI-1 | Neck | Cable | Before | 36.824453 | -122.169606 | 325 | 1999 | MCI-Pref325 | 100 | unknown |
| NI1-A-2008 | NI-1 | Neck | Cable | 2008 | 36.824453 | -122.169606 | 321 | 1/31/08 | V3167 | 100 | good |
| NI1-A-2010 | NI-1 | Neck | Cable | 2010 | 36.824330 | -122.170204 | 323 | 1/13/10 | V3488 | 100 | good |
| NI1-B-1999 | NI-1 | Neck | Cable | Before | 36.824886 | -122.170095 | 325 | 1999 | MCI-Pref325 | 100 | unknown |
| NI1-B-2008 | NI-1 | Neck | Cable | 2008 | 36.824886 | -122.170095 | 318 | 1/31/08 | V3167 | 100 | good |
| NI1-B-2010 | NI-1 | Neck | Cable | 2010 | 36.823880 | -122.170080 | 324 | 1/13/10 | V3488 | 100 | good |
| NI1-C-1999 | NI-1 | Neck | Cable | Before | 36.824287 | -122.170539 | 325 | 1999 | MCI-ACAD325 | 100 | unknown |
| NI1-C-2008 | NI-1 | Neck | Cable | 2008 | 36.824287 | -122.170539 | 323 | 1/31/08 | V3167 | 100 | good |
| NI1-C-2010 | NI-1 | Neck | Cable | 2010 | 36.824795 | -122.169190 | 321 | 1/13/10 | V3488 | 100 | good |
| SC1-A-2006 | SC-1 | Shelf | Control | Before | 36.714458 | -121.909033 | 90 | 9/25/06 | V2891 | 100 | unknown |
| SC1-A-2008 | SC-1 | Shelf | Control | 2008 | 36.714458 | -121.909033 | 89 | 11/30/07 | V3135 | 100 | poor |
| SC1-A-2010 | SC-1 | Shelf | Control | 2010 | 36.715485 | -121.908630 | 88 | 2/18/10 | V3518 | 100 | good |
| SC1-B-2006 | SC-1 | Shelf | Control | Before | 36.714299 | -121.908597 | 90 | 9/25/06 | V2891 | 100 | unknown |
| SC1-B-2008 | SC-1 | Shelf | Control | 2008 | 36.714299 | -121.908597 | 90 | 11/30/07 | V3135 | 100 | poor |
| SC1-B-2010 | SC-1 | Shelf | Control | 2010 | 36.714436 | -121.908620 | 88 | 2/18/10 | V3518 | 100 | good |
| SC1-C-2006 | SC-1 | Shelf | Control | Before | 36.713548 | -121.907717 | 90 | 9/25/06 | V2891 | 100 | unknown |
| SC1-C-2008 | SC-1 | Shelf | Control | 2008 | 36.713548 | -121.907717 | 89 | 11/30/07 | V3135 | 100 | poor |
| SC1-C-2010 | SC-1 | Shelf | Control | 2010 | 36.714790 | -121.907380 | 88 | 2/18/10 | V3518 | 100 | good |
| SC2-A-2006 | SC-2 | Shelf | Control | Before | 36.821804 | -121.9445 | 87 | 10/5/06 | V2905 | 100 | unknown |
| SC2-A-2008 | SC-2 | Shelf | Control | 2008 | 36.822428 | -121.945564 | 87 | 11/30/07 | V3134 | 100 | poor |
| SC2-A-2010 | SC-2 | Shelf | Control | 2010 | 36.821888 | -121.945530 | 88 | 1/29/10 | V3504 | 100 | good |
| SC2-B-2006 | SC-2 | Shelf | Control | Before | 36.822571 | -121.94354 | 86 | 10/5/06 | V2905 | 100 | unknown |
| SC2-B-2008 | SC-2 | Shelf | Control | 2008 | 36.822978 | -121.944894 | 86 | 11/30/07 | V3134 | 100 | poor |
| SC2-B-2010 | SC-2 | Shelf | Control | 2010 | 36.822903 | -121.945540 | 87 | 1/29/10 | V3504 | 100 | good |
| SC2-C-2008 | SC-2 | Shelf | Control | 2008 | 36.823517 | -121.944924 | 86 | 11/30/07 | V3134 | 100 | poor |
| SC2-C-2010 | SC-2 | Shelf | Control | 2010 | 36.823350 | -121.946236 | 88 | 1/29/10 | V3504 | 100 | good |
| SC3-A-2006 | SC-3 | Shelf | Control | Before | 36.878461 | -122.121178 | 89 | 10/5/06 | V2904 | 100 | unknown |
| SC3-A-2008 | SC-3 | Shelf | Control | 2008 | 36.877177 | -122.120826 | 90 | 1/30/08 | V3166 | 100 | good |
| Tr. Code | Station | Region | Treatment | Period | Latitude | Longitude | Depth (m) | Date | Dive # | Length (m) | Accuracy |
| SC3-A-2010 | SC-3 | Shelf | Control | 2010 | 36.878260 | -122.121230 | 90 | 1/28/10 | V3502 | 100 | good |
| SC3-B-2006 | SC-3 | Shelf | Control | Before | 36.878765 | -122.120205 | 89 | 10/5/06 | V2904 | 100 | unknown |
| SC3-B-2008 | SC-3 | Shelf | Control | 2008 | 36.877639 | -122.121503 | 91 | 1/30/08 | V3166 | 100 | good |
| SC3-B-2010 | SC-3 | Shelf | Control | 2010 | 36.878094 | -122.119900 | 90 | 1/28/10 | V3502 | 100 | good |
| SC3-C-2008 | SC-3 | Shelf | Control | 2008 | 36.877494 | -122.120593 | 91 | 1/30/08 | V3166 | 90 | moderate |
| SC3-C-2010 | SC-3 | Shelf | Control | 2010 | 36.876762 | -122.119804 | 90 | 1/28/10 | V3502 | 100 | good |
| SI1-A-1999 | SI-1 | Shelf | Cable | Before | 36.863391 | -122.0969 | 90 | 1999 | MCI-ACAD90 | 100 | unknown |
| SI1-A-2008 | SI-1 | Shelf | Cable | 2008 | 36.863391 | -122.0969 | 91 | 1/23/08 | V3161 | 100 | poor |
| SI1-A-2010 | SI-1 | Shelf | Cable | 2010 | 36.863014 | -122.096540 | 89 | 1/28/10 | V3503 | 100 | good |
| SI1-B-1999 | SI-1 | Shelf | Cable | Before | 36.863186 | -122.096707 | 90 | 1999 | MCI-ACAD90 | 100 | unknown |
| SI1-B-2008 | SI-1 | Shelf | Cable | 2008 | 36.863186 | -122.096707 | 92 | 1/23/08 | V3161 | 100 | poor |
| SI1-B-2010 | SI-1 | Shelf | Cable | 2010 | 36.862130 | -122.096330 | 89 | 1/28/10 | V3503 | 100 | good |
| SI1-C-2008 | SI-1 | Shelf | Cable | 2008 | 36.863471 | -122.096143 | 92 | 1/23/08 | V3161 | 100 | poor |
| SI1-C-2010 | SI-1 | Shelf | Cable | 2010 | 36.862755 | -122.094860 | 89 | 1/28/10 | V3503 | 100 | good |
| Skate1-A-2008 | Skate | Skate | Control | 2008 | 36.82574 | -122.168653 | 313 | 1/31/08 | V3167 | 100 | good |
| Skate1-A-2010 | Skate | Skate | Control | 2010 | 36.82574 | -122.168653 | 313 | 1/13/10 | V3488 | 100 | good |
| Skate1-B-2008 | Skate | Skate | Control | 2008 | 36.826303 | -122.168202 | 310 | 1/31/08 | V3167 | 82 | good |
| Skate1-B-2010 | Skate | Skate | Control | 2010 | 36.826303 | -122.168202 | 310 | 1/13/10 | V3488 | 100 | good |
| Skate1-C-2008 | Skate | Skate | Control | 2008 | 36.827346 | -122.167738 | 309 | 1/31/08 | V3167 | 100 | good |
| Skate1-C-2010 | Skate | Skate | Control | 2010 | 36.827346 | -122.167738 | 309 | 1/13/10 | V3488 | 100 | good |
| Skate2-A-2008 | Skate | Skate | Cable | 2008 | 36.826029 | -122.169319 | 317 | 1/31/08 | V3167 | 100 | good |
| Skate2-A-2010 | Skate | Skate | Cable | 2010 | 36.826029 | -122.169319 | 317 | 1/13/10 | V3488 | 100 | good |
| Skate2-B-2008 | Skate | Skate | Cable | 2008 | 36.826903 | -122.168852 | 311 | 1/31/08 | V3167 | 100 | good |
| Skate2-B-2010 | Skate | Skate | Cable | 2010 | 36.826903 | -122.168852 | 311 | 1/13/10 | V3488 | 100 | good |
| Skate2-C-2008 | Skate | Skate | Cable | 2008 | 36.82757 | -122.168474 | 306 | 1/31/08 | V3167 | 86 | good |
| Skate2-C-2010 | Skate | Skate | Cable | 2010 | 36.82757 | -122.168474 | 306 | 1/13/10 | V3488 | 100 | good |
| SLC1-A-2001 | SLC-1 | Slope | Control | Before | 36.743379 | -122.275459 | 1000 | 10/12/01 | V2083 | 100 | unknown |
| SLC1-A-2008 | SLC-1 | Slope | Control | 2008 | 36.74579 | -122.277049 | 992 | 1/7/08 | V3147 | 100 | good |
| SLC1-A-2010 | SLC-1 | Slope | Control | 2010 | 36.744267 | -122.277030 | 1001 | 3/8/10 | D115 | 100 | good |
| SLC1-B-2001 | SLC-1 | Slope | Control | Before | 36.742199 | -122.274547 | 1000 | 10/12/01 | V2083 | 100 | unknown |
| SLC1-B-2008 | SLC-1 | Slope | Control | 2008 | 36.745207 | -122.277572 | 1001 | 1/7/08 | V3147 | 80 | good |
| SLC1-B-2010 | SLC-1 | Slope | Control | 2010 | 36.745200 | -122.277725 | 1002 | 3/8/10 | D115 | 100 | good |
| SLC1-C-2001 | SLC-1 | Slope | Control | Before | 36.741165 | -122.27301 | 999 | 10/12/01 | V2083 | 100 | unknown |
| Tr. Code | Station | Region | Treatment | Period | Latitude | Longitude | Depth (m) | Date | Dive # | Length (m) | Accuracy |
| SLC1-C-2008 | SLC-1 | Slope | Control | 2008 | 36.745351 | -122.278318 | 1007 | 1/7/08 | V3147 | 100 | good |
| SLC1-C-2010 | SLC-1 | Slope | Control | 2010 | 36.744420 | -122.278070 | 1006 | 3/8/10 | D115 | 100 | good |
| SLC2-A-2005 | SLC-2 | Slope | Control | Before | 36.752527 | -122.20002 | 801 | 6/8/05 | V2674 | 100 | unknown |
| SLC2-A-2008 | SLC-2 | Slope | Control | 2008 | 36.748745 | -122.197382 | 820 | 1/23/08 | V3160 | 100 | good |
| SLC2-A-2010 | SLC-2 | Slope | Control | 2010 | 36.749023 | -122.197830 | 821 | 1/27/10 | V3500 | 100 | good |
| SLC2-B-2005 | SLC-2 | Slope | Control | Before | 36.75264 | -122.200906 | 800 | 6/8/05 | V2674 | 100 | unknown |
| SLC2-B-2008 | SLC-2 | Slope | Control | 2008 | 36.74858 | -122.197578 | 817 | 1/23/08 | V3160 | 100 | good |
| SLC2-B-2010 | SLC-2 | Slope | Control | 2010 | 36.748340 | -122.197580 | 820 | 1/27/10 | V3500 | 100 | good |
| SLC2-C-2005 | SLC-2 | Slope | Control | Before | 36.752558 | -122.202336 | 801 | 6/8/05 | V2674 | 100 | unknown |
| SLC2-C-2008 | SLC-2 | Slope | Control | 2008 | 36.748988 | -122.198433 | 827 | 1/23/08 | V3160 | 100 | good |
| SLC2-C-2010 | SLC-2 | Slope | Control | 2010 | 36.749180 | -122.196240 | 813 | 1/27/10 | V3500 | 100 | good |
| SLC3-A-2006 | SLC-3 | Slope | Control | Before | 36.707094 | -122.167266 | 881 | 10/3/06 | V2898 | 100 | unknown |
| SLC3-A-2008 | SLC-3 | Slope | Control | 2008 | 36.705268 | -122.166423 | 895 | 1/8/08 | V3148 | 100 | good |
| SLC3-A-2010 | SLC-3 | Slope | Control | 2010 | 36.706226 | -122.166200 | 885 | 3/9/10 | D116 | 100 | good |
| SLC3-B-2006 | SLC-3 | Slope | Control | Before | 36.707753 | -122.166954 | 877 | 10/3/06 | V2898 | 100 | unknown |
| SLC3-B-2008 | SLC-3 | Slope | Control | 2008 | 36.706463 | -122.165729 | 884 | 1/8/08 | V3148 | 100 | good |
| SLC3-B-2010 | SLC-3 | Slope | Control | 2010 | 36.705530 | -122.166090 | 892 | 3/9/10 | D116 | 100 | good |
| SLC3-C-2006 | SLC-3 | Slope | Control | Before | 36.70771 | -122.16586 | 874 | 10/3/06 | V2898 | 100 | unknown |
| SLC3-C-2008 | SLC-3 | Slope | Control | 2008 | 36.705928 | -122.164967 | 887 | 1/8/08 | V3148 | 100 | good |
| SLC3-C-2010 | SLC-3 | Slope | Control | 2010 | 36.706146 | -122.167200 | 885 | 3/9/10 | D116 | 100 | good |
| SLI1-A-2003 | SLI-1 | Slope | Cable | Before | 36.711241 | -122.186781 | 885 | 10/13/03 | V2439 | 100 | unknown |
| SLI1-A-2008 | SLI-1 | Slope | Cable | 2008 | 36.712528 | -122.18707 | 877 | 12/6/07 | V3136 | 100 | poor |
| SLI1-A-2010 | SLI-1 | Slope | Cable | 2010 | 36.712750 | -122.187164 | 877 | 3/8/10 | D114 | 100 | good |
| SLI1-B-2003 | SLI-1 | Slope | Cable | Before | 36.711353 | -122.186449 | 883 | 10/13/03 | V2439 | 100 | unknown |
| SLI1-B-2008 | SLI-1 | Slope | Cable | 2008 | 36.71302 | -122.186753 | 877 | 12/6/07 | V3136 | 100 | poor |
| SLI1-B-2010 | SLI-1 | Slope | Cable | 2010 | 36.713116 | -122.186900 | 875 | 3/8/10 | D114 | 100 | good |
| SLI1-C-2003 | SLI-1 | Slope | Cable | Before | 36.711883 | -122.186523 | 882 | 10/13/03 | V2439 | 100 | unknown |
| SLI1-C-2008 | SLI-1 | Slope | Cable | 2008 | 36.712683 | -122.186665 | 874 | 12/6/07 | V3136 | 100 | poor |
| SLI1-C-2010 | SLI-1 | Slope | Cable | 2010 | 36.712590 | -122.186890 | 877 | 3/8/10 | D114 | 100 | good |

Appendix 2.  Mean megafaunal density along the MARS cable route and control sites, with phylum (or subphylum)and group designations.  The density (# 100 m2) of each taxon at regional video transects (n=104) in all years and at all depths were averaged to produce an overall average density.  SD = standard deviation.  Percent indicates the percentage of the total average faunal density (158 megafaunal individuals per 100 m2).

Phylum	Group	Taxa	Mean Density	SD	%

Cnidaria	Actiniaria	Actiniidae sp. 1	1.01	2.61	0.64

Cnidaria	Actiniaria	Actiniaria – unident.	0.64	2.20	0.41

Cnidaria	Actiniaria	Actinostolidae sp. 3	3.54	6.05	2.24

Osteichthyes	Agonidae	Agonidae – unident.	0.10	0.45	0.06

Osteichthyes	Alepocephalidae	Alepocephalustenebrosus	0.01	0.10	0.01

Echinodermata	Echinoidea	Stronglylocentrotusfragilis	11.50	33.75	7.27

Osteichthyes	Anoplopomatidae	Anoplopomafimbria	0.04	0.24	0.02

Cnidaria	Alcyonacea	Anthomastusritteri	0.09	0.45	0.06

Cnidaria	Pennatulacea	Anthoptilumgrandiflorum	0.05	0.26	0.03

Cnidaria	Anthozoa	Anthozoa – unident.	0.03	0.17	0.02

Osteichthyes	Moridae	Antimoramicrolepis	0.08	0.36	0.05

Porifera	Porifera	Asbestoplumasp.	0.00	0.00	0.00

Echinodermata	Asteroidea	Asteroidea – unident.	0.50	1.53	0.32

Echinodermata	Ophiuroidea	Asteronyx sp.	0.16	0.62	0.10

Chondrichthyes	Rajiformes	Bathyrajakincaidii	0.03	0.17	0.02

Echinodermata	Asteroidea	Benthopecten sp.	0.01	0.10	0.01

Osteichthyes	Ophidiidae	Bothrocarabrunneum	0.02	0.20	0.01

Arthropoda	Decapoda	Brachyura – unident.	0.01	0.10	0.01

Echinodermata	Echinoidea	Brisaster sp.	0.11	0.39	0.07

Mollusca	Gastropoda	Calliostoma platinum	0.39	1.60	0.25

Arthropoda	Decapoda	Cancer sp.	0.03	0.22	0.02

Osteichthyes	Liparidae	Careproctusmelanurus	0.04	0.19	0.02

Arthropoda	Caridea	Caridea – unident.	0.05	0.40	0.03

Arthropoda	Caridea	Caridea sp.1	0.09	0.40	0.05

Cnidaria	Ceriantharia	Ceriantharia sp. 1	0.00	0.00	0.00

Cnidaria	Ceriantharia	Ceriantharia sp. 2	0.58	2.22	0.37

Cnidaria	Ceriantharia	Ceriantharia sp. 3	0.13	0.58	0.09

Cnidaria	Ceriantharia	Ceriantharia – unident.	1.21	2.17	0.76

Arthropoda	Decapoda	Chionoecetestanneri	1.92	6.66	1.22

Arthropoda	Decapoda	Chorilialongipes	0.09	0.35	0.06

Cnidaria	Corallimorphidae	Corallimorphuspilatus	0.13	0.52	0.08

Osteichthyes	Macrouridae	Coryphaenoidesacrolepis	0.19	0.90	0.12

Arthropoda	Decapoda	Decapoda – unident.	0.02	0.14	0.01

Arthropoda	Anomura	Diogenidae spp.	0.50	1.74	0.32

Mollusca	Cephalopoda	Doryteuthisopalescens	0.22	1.45	0.14

Mollusca	Cephalopoda	Dosidicusgigas	0.08	0.41	0.05

Cnidaria	Rhodalidae	Dromaliaalexandri	0.04	0.28	0.02

Echiura	Echiura	Echiura- – unident.	0.05	0.26	0.03

Phylum	Group	Taxa	Mean Density	SD	%

Osteichthyes	Pleuronectiformes	Embassichthysbathybius	0.18	0.54	0.12

Vertebrata	Myxinidae	Eptatretusstoutii	0.15	0.55	0.10

Echinodermata	Crinoidea	Florametraserratissima	0.21	1.33	0.13

Cnidaria	Pennatulacea	Funiculina sp.	31.99	79.74	20.22

Mollusca	Gastropoda	Gastropoda – unident.	10.09	29.35	6.38

Mollusca	Gastropoda	Gastropoda sp. 3	0.11	0.70	0.07

Mollusca	Gastropoda	Gastropoda sp. 4	0.01	0.10	0.01

Osteichthyes	Pleuronectiformes	Glyptocephaluszachirus	0.84	1.97	0.53

Cnidaria	Gorgonacea	Gorgonacea – unident.	0.01	0.10	0.01

Echinodermata	Asteroidea	?Henricia sp. 1	0.07	0.38	0.04

Echinodermata	Asteroidea	Myxoderma sp.	0.02	0.14	0.01

Osteichthyes	Hexagrammidae	Hexagrammidae – unident.	0.14	0.58	0.09

Echinodermata	Asteroidea	Hippasteria spp.	0.17	0.63	0.11

Echinodermata	Holothuroidea	Holothuroidea – unident.	0.05	0.32	0.03

Chondrichthyes	Chimaeridae	Hydrolaguscolliei	0.01	0.10	0.01

Detritus	Laminariales	0.48	0.93	0.30

Brachipoda	Brachipoda	Laqueuscalifornianus	0.06	0.59	0.04

Osteichthyes	Liparidae	Liparidae – unident.	0.01	0.10	0.01

Cnidaria	Actiniaria	Liponemabrevicornis	0.06	0.27	0.04

Cnidaria	Hydrozoa	Little Red Jellies Complex	0.07	0.25	0.04

Echinodermata	Asteroidea	Luidiafoliolata	0.31	0.86	0.19

Osteichthyes	Stichaeidae	Lumpenussagitta	0.42	2.11	0.27

Osteichthyes	Zoarcidae	Lycenchelys spp.	1.23	3.07	0.78

Osteichthyes	Zoarcidae	Lycodapus sp.	0.14	0.74	0.09

Osteichthyes	Zoarcidae	Lycodescortezianus	0.23	0.70	0.15

Osteichthyes	Zoarcidae	Lycodesdiapterus	0.05	0.24	0.03

Osteichthyes	Zoarcidae	Lyconemabarbatum	0.01	0.10	0.01

Osteichthyes	Pleuronectiformes	Lyopsettaexilis	0.01	0.10	0.01

Echinodermata	Asteroidea	Mediaster spp.	4.07	10.57	2.57

Urochordata	Urochordata	Megalodicopiahians	0.10	0.80	0.07

Osteichthyes	Merlucciidae	Merlucciusproductus	0.37	1.53	0.24

Cnidaria	Actiniaria	Isosicyonis sp.	11.50	24.48	7.27

Cnidaria	Actiniaria	Metridiumfarcimen	0.36	1.05	0.22

Osteichthyes	Pleuronectiformes	Microstomuspacificus	0.79	1.84	0.50

Arthropoda	Malacostraca	Mysida – unident.	0.07	0.33	0.04

Mollusca	Gastropoda	Neptuneaaminata	1.24	2.80	0.78

Mollusca	Gastropoda	Nudibranchia – unident.	0.02	0.20	0.01

Mollusca	Cephalopoda	Octopus rubescens	0.17	0.67	0.11

Osteichthyes	Ophidiidae	Ophidionscrippsae	0.02	0.15	0.01

Osteichthyes	Hexagrammidae	Ophiodonelongatus	0.00	0.00	0.00

Echinodermata	Ophiuroidea	Ophiuroidea spp.	6.97	32.16	4.41

Osteichthyes	Osmeridae	Osmeridae – unident.	0.15	0.76	0.10

Osteichthyes	Osteichthyes	Osteichthyes – unident.	0.29	1.08	0.18

Phylum	Group	Taxa	Mean Density	SD	%

Arthropoda	Caridea	Pandalusplatyceros	0.03	0.29	0.02

Echinodermata	Holothuroidea	Pannychiamoseleyi	1.18	11.37	0.75

Arthropoda	Anomura	Paralithodes sp.	0.05	0.35	0.03

Echinodermata	Holothuroidea	Parastichopusspp.	0.89	2.29	0.57

Chondrichthyes	Scyliorhinidae	Parmaturusxaniurus	0.05	0.32	0.03

Cnidaria	Pennatulacea	Pennatula spp.	0.71	2.08	0.45

Cnidaria	Pennatulacea	Pennatulacea – unident.	11.43	20.45	7.22

Detritus	Phyllospadix	10.41	17.05	6.58

Mollusca	Gastropoda	Pleurobranchaeacalifornica	0.13	0.46	0.08

Osteichthyes	Pleuronectiformes	Pleuronectiformes – unident.	2.69	6.10	1.70

Porifera	Porifera	Porifera	1.58	5.12	1.00

Echinodermata	Holothuroidea	Psolussquamatus	8.93	51.41	5.64

Cnidaria	Pennatulacea	Ptilosarcusgurneyi	0.11	0.48	0.07

Echinodermata	Asteroidea	Pycnopodiahelianthoides	0.00	0.00	0.00

Chondrichthyes	Rajiformes	Raja binoculata	0.01	0.10	0.01

Chondrichthyes	Rajiformes	Raja rhina	0.16	0.52	0.10

Echinodermata	Asteroidea	Rathbunastercalifornicus	9.65	26.90	6.10

Annelida	Polychaeta	Sabellidae sp.	0.05	0.26	0.03

Osteichthyes	Scorpaenidae	Sebastessp.	0.12	0.63	0.07

Osteichthyes	Scorpaenidae	Sebastes aurora	0.11	0.58	0.07

Osteichthyes	Scorpaenidae	Sebastesdiploproa	2.10	7.51	1.33

Osteichthyes	Scorpaenidae	Sebastesjordani	0.01	0.10	0.01

Osteichthyes	Scorpaenidae	Sebastespaucispinis	0.00	0.00	0.00

Osteichthyes	Scorpaenidae	Sebastesrosenblatti	0.00	0.00	0.00

Osteichthyes	Scorpaenidae	Sebastesrubrivinctus	0.00	0.00	0.00

Osteichthyes	Scorpaenidae	Sebastessaxicola	0.07	0.69	0.04

Osteichthyes	Scorpaenidae	Sebastolobus spp.	3.17	5.27	2.00

Osteichthyes	Scorpaenidae	Sebastomus complex	0.01	0.10	0.01

Echinodermata	Asteroidea	Solaster borealis	0.45	1.16	0.29

Chondrichthyes	Squalidae	Squalussuckleyi	0.03	0.29	0.02

Echinodermata	Asteroidea	Stylasteriasforreri	0.34	1.02	0.22

Cnidaria	Gorgonacea	Swiftia simplex	0.23	1.23	0.15

Osteichthyes	Torpedinidae	Torpedo californica	0.02	0.14	0.01

Mollusca	Gastropoda	Tritoniadiomedea	0.03	0.22	0.02

Cnidaria	Pennatulacea	Umbellulalindahli	6.02	18.78	3.80

Osteichthyes	Agonidae	Xeneretmus sp.	0.35	1.24	0.22

Osteichthyes	Embiotocidae	Zalembiusrosaceus	0.04	0.31	0.02

Osteichthyes	Zaniolepididae	Zaniolepislatipinnis	0.12	0.47	0.07

Osteichthyes	Zoarcidae	Zoarcidae  - unident.	0.03	0.17	0.02

Total, all taxa			158.22		100.00

Appendix 3.  Samples collected during the MARS Cable survey, 2007-2010. Depth indicated in meters.

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3135-C29 | 88 | SC-1 | Macrofauna | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C31 | 88 | SC-1 | Macrofauna | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C33 | 88 | SC-1 | Macrofauna | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C35 | 88 | SC-1 | Macrofauna | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C39 | 88 | SC-1 | Macrofauna | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C30 | 88 | SC-1 | Tracer | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C34 | 88 | SC-1 | Tracer | 11/30/07 | 36.834352 | -122.170323 |
| V3135-C38 | 88 | SC-1 | Tracer | 11/30/07 | 36.834352 | -122.170323 |
| V3134-C04 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C23 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C24 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C26 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C27 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C32 | 86 | SC-2 | Macrofauna | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C02 | 86 | SC-2 | Tracer | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C25 | 86 | SC-2 | Tracer | 11/30/07 | 36.714237 | -121.909300 |
| V3134-C28 | 86 | SC-2 | Tracer | 11/30/07 | 36.714237 | -121.909300 |
| V3136-C21 | 878 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C22 | 873 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C26 | 874 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C29 | 877 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C33 | 875 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C34 | 876 | SLI-1 | Macrofauna | 12/6/07 | 36.798840 | -122.107930 |
| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| V3136-C25 | 878 | SLI-1 | Tracer | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C30 | 874 | SLI-1 | Tracer | 12/6/07 | 36.798840 | -122.107930 |
| V3136-C37 | 875 | SLI-1 | Tracer | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A07 | 878 | SLI-1 | Actiniidae sp. 1 | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A03 | 878 | SLI-1 | Gastropoda | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A02 | 878 | SLI-1 | Mesomyaria | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A06 | 878 | SLI-1 | Funiculina | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A04 | 878 | SLI-1 | Funiculina | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A01 | 879 | SLI-1 | Solaster | 12/6/07 | 36.798840 | -122.107930 |
| V3136-A05 | 878 | SLI-1 | Actinostolidae | 12/6/07 | 36.798840 | -122.107930 |
| V3147-C11 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C14 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C17 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C23 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C24 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C27 | 1000 | SLC-1 | Macrofauna | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C13 | 1000 | SLC-1 | Tracer | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C15 | 1000 | SLC-1 | Tracer | 1/7/08 | 36.749540 | -122.197410 |
| V3147-C28 | 1000 | SLC-1 | Tracer | 1/7/08 | 36.749540 | -122.197410 |
| V3148-C02 | 893 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C04 | 885 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C05 | 891 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C07 | 884 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C08 | 891 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C09 | 896 | SLC-3 | Macrofauna | 1/8/08 | 36.788670 | -122.116560 |
| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| V3148-C03 | 891 | SLC-3 | Tracer | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C06 | 891 | SLC-3 | Tracer | 1/8/08 | 36.788670 | -122.116560 |
| V3148-C12 | 891 | SLC-3 | Tracer | 1/8/08 | 36.788670 | -122.116560 |
| V3161-C02 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C05 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C08 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C09 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C11 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C15 | 89 | SI-1 | Macrofauna | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C04 | 89 | SI-1 | Tracer | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C24 | 89 | SI-1 | Tracer | 1/23/08 | 36.744930 | -122.277850 |
| V3161-C40 | 89 | SI-1 | Tracer | 1/23/08 | 36.744930 | -122.277850 |
| V3160-C06 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C13 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C17 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C18 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C28 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C35 | 820 | SLC-2 | Macrofauna | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C12 | 820 | SLC-2 | Tracer | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C14 | 820 | SLC-2 | Tracer | 1/23/08 | 36.788670 | -122.116560 |
| V3160-C23 | 820 | SLC-2 | Tracer | 1/23/08 | 36.788670 | -122.116560 |
| V3162-C16 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C20 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C22 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C26 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3162-C32 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C36 | 360 | NC-3 | Macrofauna | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C19 | 360 | NC-3 | Tracer | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C25 | 360 | NC-3 | Tracer | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C27 | 360 | NC-3 | Tracer | 1/24/08 | 36.712337 | -122.186830 |
| V3162-C19 | 360 | NC-3 | Enteropneusta? | 1/24/08 | 36.712337 | -122.186830 |
| V3165-C02 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C17 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C23 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C24 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C28 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C40 | 400 | NC-1 | Macrofauna | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C08 | 400 | NC-1 | Tracer | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C15 | 400 | NC-1 | Tracer | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C35 | 400 | NC-1 | Tracer | 1/30/08 | 36.706360 | -122.166440 |
| V3165-C10 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C31 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C33 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C34 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C37 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C39 | 200 | NC-2 | Macrofauna | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C03 | 200 | NC-2 | Tracer | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C29 | 200 | NC-2 | Tracer | 1/30/08 | 36.712260 | -122.186870 |
| V3165-C30 | 200 | NC-2 | Tracer | 1/30/08 | 36.712260 | -122.186870 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3166-C11 | 89 | SC-3 | Macrofauna | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C12 | 89 | SC-3 | Macrofauna | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C14 | 89 | SC-3 | Macrofauna | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C18 | 89 | SC-3 | Macrofauna | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C38 | 89 | SC-3 | Macrofauna | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C04 | 89 | SC-3 | Tracer | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C06 | 89 | SC-3 | Tracer | 1/30/08 | 36.744930 | -122.277850 |
| V3166-C07 | 89 | SC-3 | Tracer | 1/30/08 | 36.744930 | -122.277850 |
| V3167-C09 | 324 | NI-1 | Macrofauna | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C16 | 324 | NI-1 | Macrofauna | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C21 | 324 | NI-1 | Macrofauna | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C25 | 324 | NI-1 | Macrofauna | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C32 | 324 | NI-1 | Macrofauna | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C20 | 324 | NI-1 | Tracer | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C22 | 324 | NI-1 | Tracer | 1/31/08 | 36.712856 | -122.186360 |
| V3167-C36 | 324 | NI-1 | Tracer | 1/31/08 | 36.712856 | -122.186360 |
| V3483-PC03 | 400 | NC-1 | Macrofauna | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC21 | 400 | NC-1 | Macrofauna | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC02 | 400 | NC-1 | Tracer | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC04 | 400 | NC-1 | Macrofauna | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC05 | 400 | NC-1 | Tracer | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC06 | 400 | NC-1 | Macrofauna | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC07 | 400 | NC-1 | Macrofauna | 1/8/10 | 36.706360 | -122.166440 |
| V3483-PC08 | 400 | NC-1 | Tracer | 1/8/10 | 36.706360 | -122.166440 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3483-PC12 | 200 | NC-2 | Macrofauna | 1/8/10 | 36.712260 | -122.186870 |
| V3483-PC13 | 200 | NC-2 | Tracer | 1/8/10 | 36.712260 | -122.186870 |
| V3483-PC14 | 200 | NC-2 | Macrofauna | 1/8/10 | 36.712260 | -122.186870 |
| V3483-PC15 | 200 | NC-2 | Macrofauna | 1/8/10 | 36.712260 | -122.186870 |
| V3483-PC16 | 200 | NC-2 | Tracer | 1/8/10 | 36.712260 | -122.186870 |
| V3483-PC18 | 200 | NC-2 | Macrofauna | 1/8/10 | 36.712260 | -122.186870 |
| V3488-PC03 | 324 | NI-1 | Macrofauna | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC04 | 324 | NI-1 | Macrofauna | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC05 | 324 | NI-1 | Macrofauna | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC07 | 324 | NI-1 | Macrofauna | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC21 | 324 | NI-1 | Macrofauna | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC02 | 324 | NI-1 | Tracer | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC06 | 324 | NI-1 | Tracer | 1/13/10 | 36.712856 | -122.186360 |
| V3488-PC08 | 324 | NI-1 | Tracer | 1/13/10 | 36.712856 | -122.186360 |
| V3490-PC09 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC21 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC23 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC24 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC26 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC27 | 357 | NC-3 | Macrofauna | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC22 | 357 | NC-3 | Tracer | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC25 | 357 | NC-3 | Tracer | 1/14/10 | 36.712337 | -122.186830 |
| V3490-PC28 | 357 | NC-3 | Tracer | 1/14/10 | 36.712337 | -122.186830 |
| V3501-PC10 | 89 | SC-2 | Macrofauna | 1/27/10 | 36.714237 | -121.909300 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3501-PC13 | 89 | SC-2 | Tracer | 1/27/10 | 36.714237 | -121.909300 |
| V3500-PC03 | 800 | SLC-2 | Macrofauna | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC04 | 800 | SLC-2 | Macrofauna | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC06 | 800 | SLC-2 | Macrofauna | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC07 | 800 | SLC-2 | Macrofauna | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC09 | 800 | SLC-2 | Macrofauna | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC05 | 800 | SLC-2 | Tracer | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC08 | 800 | SLC-2 | Tracer | 1/27/10 | 36.788670 | -122.116560 |
| V3500-PC22 | 800 | SLC-2 | Tracer | 1/27/10 | 36.788670 | -122.116560 |
| V3502-PC22 | 89 | SC-3 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC24 | 89 | SC-3 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC25 | 89 | SC-3 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC27 | 89 | SC-3 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC28 | 89 | SC-3 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC23 | 89 | SC-3 | Tracer | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC26 | 89 | SC-3 | Tracer | 1/28/10 | 36.744930 | -122.277850 |
| V3502-PC29 | 89 | SC-3 | Tracer | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC15 | 89 | SI-1 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC16 | 89 | SI-1 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC31 | 89 | SI-1 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC33 | 89 | SI-1 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC34 | 89 | SI-1 | Macrofauna | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC14 | 89 | SI-1 | Tracer | 1/28/10 | 36.744930 | -122.277850 |
| V3503-PC17 | 89 | SI-1 | Tracer | 1/28/10 | 36.744930 | -122.277850 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| V3504-PC02 | 89 | SC-2 | Macrofauna | 1/29/10 | 36.714237 | -121.909300 |
| V3504-PC03 | 89 | SC-2 | Tracer | 1/29/10 | 36.714237 | -121.909300 |
| V3504-PC04 | 89 | SC-2 | Macrofauna | 1/29/10 | 36.714237 | -121.909300 |
| V3504-PC05 | 89 | SC-2 | Macrofauna | 1/29/10 | 36.714237 | -121.909300 |
| V3504-PC06 | 89 | SC-2 | Tracer | 1/29/10 | 36.714237 | -121.909300 |
| V3518-PC02 | 89 | SC-1 | Macrofauna | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC04 | 89 | SC-1 | Macrofauna | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC05 | 89 | SC-1 | Macrofauna | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC07 | 89 | SC-1 | Macrofauna | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC08 | 89 | SC-1 | Macrofauna | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC03 | 89 | SC-1 | Tracer | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC06 | 89 | SC-1 | Tracer | 2/18/10 | 36.834352 | -122.170323 |
| V3518-PC09 | 89 | SC-1 | Tracer | 2/18/10 | 36.834352 | -122.170323 |
| D115-PC79 | 1000 | SLC-1 | Macrofauna | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC78 | 1000 | SLC-1 | Macrofauna | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC64 | 1000 | SLC-1 | Macrofauna | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC45 | 1000 | SLC-1 | Macrofauna | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC69 | 1000 | SLC-1 | Macrofauna | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC44 | 1000 | SLC-1 | Tracer | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC66 | 1000 | SLC-1 | Tracer | 3/8/10 | 36.749540 | -122.197410 |
| D115-PC57 | 1000 | SLC-1 | Tracer | 3/8/10 | 36.749540 | -122.197410 |
| D114-PC79 | 890 | SLI-1 | Macrofauna | 3/8/10 | 36.798840 | -122.107930 |
| D114-PC69 | 890 | SLI-1 | Macrofauna | 3/8/10 | 36.798840 | -122.107930 |
| D114-PC48 | 890 | SLI-1 | Macrofauna | 3/8/10 | 36.798840 | -122.107930 |

| Sample | Depth | Station | Type | Date | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| D114-PC67 | 890 | SLI-1 | Macrofauna | 3/8/10 | 36.798840 | -122.107930 |
| D114-PC76 | 890 | SLI-1 | Tracer | 3/8/10 | 36.798840 | -122.107930 |
| D114-PC68 | 890 | SLI-1 | Tracer | 3/8/10 | 36.798840 | -122.107930 |
| D114-PC53 | 890 | SLI-1 | Tracer | 3/8/10 | 36.798840 | -122.107930 |
| D116-PC45 | 888 | SLC-3 | Macrofauna | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC68 | 888 | SLC-3 | Macrofauna | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC48 | 888 | SLC-3 | Macrofauna | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC67 | 888 | SLC-3 | Macrofauna | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC80 | 888 | SLC-3 | Macrofauna | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC51 | 888 | SLC-3 | Tracer | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC52 | 888 | SLC-3 | Tracer | 3/9/10 | 36.788670 | -122.116560 |
| D116-PC53 | 888 | SLC-3 | Tracer | 3/9/10 | 36.788670 | -122.116560 |