Benthic Rover Respirometer Chamber Volume

- Chamber volume

  - Inside diameter of chambers: .303m (11.93”)

  - Distance from top plate to bottom of acrylic: .121m (4.75”)

    - Each mm of exposed knife adds: 72 cc

  - 8.7x10-3 M3 , 8700.9 cc or 530 cubic inches

  - Add volume below valves

    - Front valve:  48cc, or 2.65 in3

    - Side valves (2): 25.2 cc, 1.54 in3

  - Add volume around stirrer: ~ 5cc, .3 in3

  - 8700.9+48+25.2+25.2+ 5= 8804.2 cc

- Minus the volume of components inside the chamber

  - Optode:  57.1 cc, or 3.48 in3

  - Stir rods:  28.6 cc or 1.74 in3

- Total volume 8718 cc

  - Don’t forget to add added/reduced volume based on level of sediment.

- If you really want to know the precise volume, then you should remove the rack/chamber, and carefully fill it with water and measure the actual volume