April 9, 2015

Canon MarkIII Energy tests, done using Yokogawa WT310

V= 28.2 V

Images collected = 39 images

Test durations = 3.25 Hours

Offset = 0.1mA

Total energy = 822.64 mWHr

Energy per image 	= (Total energy – offset)/number of images

= (822.64 mWHr – (0.1 mA*28.2V*3.25 Hr))/39 images

= 21 mWHr/image

Energy for camera for one year	= (Energy per image * 365 days * 24 Hr/day * 1 image/Hr)

= 21 mWHr *365 days/year *24 Hr/day *1 image/Hr  = 184 WHr/year

It is possible this could be lower as I did not have the display screen disabled.