# Snappy Test and Deploy Functional Specs

March 30, 2016: Original document (Henthorn)

All actions assume 25V 5A external power and serial connection at 9600,8,N,1 through deck cable.

- Test – choose Unit Tests from the main menu

  - Functions - Everything on the current controller menu

- Pre-Deployment - choose Pre-deployment form from main menu

  - Set the current time

  - Image interval in minutes

  - Strobe charge time in seconds

  - Delay before starting deployment in minutes

  - Save options

- Deployment - choose Deploy from main menu and remove external power

  - Delay for delay-time

  - Sleep for image-interval

  - Take image

    - Power on and charge strobes for maximum of charge-time seconds

    - Power on and boot the cameras

    - Trigger the camera shutters

    - Power off cameras and strobes

  - Log before and after strobe voltages w/timestamp

  - Back to sleep for image-interval

- Post-Deployment – return external power and choose Post-deployment form from main manu

  - Stop deployment

  - Request log data to be displayed