| 30 October Sept 2019 |  |
| --- | --- |
| Pulse 72 debrief | ADCP download data- remove battery Paul focusing on laser measure, writing code Lasers need to be taken off vehicle. SES- timing Larval sampler needs postmordem to see if water got into pump or index motor. Something blew up pump controller, main controller, main controller power supply. Leaking filteration Red tagged mark I camera. Put a mark I back in.- Xif in late dec to see if camera had been used before. SCOC insertion depths All but SES images on pulse/project library Fresh coin cell in mark I but not dualie. Update technical orders. When johnny turnedaround dualie- reused release anodes from previous deployment. Once you crush the copper balls they can’t be reused. Need to do a better job of cleaning isolator hardware How will macro fit into this cruise?Give Diana a chance to do a replicate? Prioritize |
| SES timing | Wakey. There’s a limit to how many times you can write to the flash 2 minute safety was in effect. Went through its scheduled 5000 sets. Will Rover has an issue with this? Not likely. |
| Optodes | Recalibrate and swap out. Look for dip in rover reference during salp blooms Check offset between reference optode readings Rover |
| Year end spending | Camera Connectors Spare for macro Anodes, chain isolators We have 1 spare release Optode Recalibrate and swap out Check offset between reference optode readings Rover |
| Snappy delay | Doesn’t work…working to figure it out. Mark III and IV issue. |
| Lasers | Hang up is mount. If we have angles, Johnny can make the mount. Working on getting the laser mount done. Need to calibrate Tueday on shore. Ship gets back today 1900 |
| Upside down sediment trap | Deployed. |
| Larvae pump | Need to post mortem. |
| Macro system (aka shrimpoteuthis) | Don’t ask till after cruise Sidelined until the other aspects are done. Keep up momemtum and finish after cruise. 50 mm lens, higher contrast. Cut housing in half Will we have a pointer laser ready? Lid’s coming in the mail. Johnny working on endcap. Test in dark In 2020 Change to easier to use menu? Engineering logging? To measure angle off horizontal- ~30 degrees. Move to 45. Mount battery at mud level. No purge port, use alkalines for lasers Pressure test? Pump helium in to test weld? |
| Solo camera height, angles | Not coming back. New mooring line will go on Solo camera and two inverted traps. |
| 30 year volume | Foram paper has been published |
| Olmo | Revising draft. Resolve field of view discrepancy |
| Personnel schedules | Paul in texas- back Tuesday Alana here |