Pelagic-Benthic Instrumentation SitRep

20Oct21

Rover

Data downloaded – yes

Data backed up – yes

Data reviewed – yes

Issues- Change out with new where possible.

- LF modem can’t talk to surface although it does make sounds on deck. Visible corrosion on LF transducer. Put new ducer on

- MF modem can’t talk to surface and makes no sounds on deck. Housing hasn’t been opened and should be treated as if it were internally pressurized. Tag modem- could be under pressure.

- Stbd drive motor quit turning during deployment. Computer can’t talk to motor controller. Johnny to open up- get to controller, inspect

- Fluorescence camera stopped capturing images after 9 pics. Unknown.

- Oil leak in hose on port rack. Hose carries signals for motor and Hall switches, and port rack was throwing errors during deployment. All oil-comp hoses show wear and should be replaced. Not trivial. Giant job. Port rack had started to throw errors. Fix what we have to fix to make it run for a year.

- Strobe air bubble has grown and pieces of rubber seal are floating inside. Seal needs to be replaced. Transit camera strobe worked for half a year or so. Swap it out.

- Still needs regular turn-around servicing and hasn’t been touched yet.

Dual Camera Tripod

Data downloaded – yes for images, yes for current meter

Data backed up – yes for images, no for current meter- check to make sure current meter data are on pulse

Data reviewed – yes for images, no for current meter

Issues

Images on VARS- almost ready for annotation

- NF camera is vignetting. John is working on mechanical solution but the cause of the problem isn’t obvious. Set up with live view

- Still needs regular turn-around servicing.

- What was condition of mooring hardware? Most was good. A couple of bad thimbles. Issue looks to be solved.

SES- unless we have to get inside, 2 weeks. Johnny’s preference to work on first. Change batteries once we have a ship date. Put in container

Data downloaded – yes

Data backed up – yes

Data reviewed – no

Issues

Tiffs converted to JPGs, currently reviewing- track down image spots

- Still needs regular turn-around servicing and hasn’t been touched yet.

- What was condition of mooring hardware?

Leave Dualie and Rover in high bay until people complain. Johnny to fend off space as best possible, take stuff to Castroville.

Sediment Trap Mooring

Data downloaded – Yes (controller data)

Data backed up – Yes (controller data)

Data reviewed – Comparing to SES data to see if we can figure out when the clog happened.

Issues

- Some mooring hardware was corroded and needs to be replaced.

- Squirty mods need to be completed. Mounted and need to put battery can on one, plug it in.

Workhorse ADCP

Data downloaded – Yes

Data backed up – Yes

Data reviewed – Viewed plot in their program, and looks good superficially

Issues

- Still needs regular turn-around servicing but has been opened by John.