Tethys Deployment 63 Hot Wash

30 March 2012

- March 7th lost, 24th recovered: 17 days + 3 days

- Tethys_Location_Seach_Recovery document

  - Captures what happened

- Should we have done the 300m depth mission in 700m of water

- Should we have dove straight to 300m – step down?

- OK to do deep dive that ended early in the morning

- Should we have been on site?

  - Actively track

  - Vehicle isn’t set up for local operations, e.g. no a-modem

- Current no-comms plan:

  - Drop weight and rely on backup (Argos)

- We did have a partial conversation with a SV (?), but it didn’t result in an email to us from GSS; are there background comms in iridium that we don’t know, e.g. like cellphones

- Spiral down, spiral up – don’t run a track, that increases the search area

- Spend more time planning search

  - Boxed drift

  - Use direct HF radar – like Sergey

  - GPS track plot for aircraft

  - Extreme limits of possible drift range

  - Had a radio and GPS

  - Had binoculars

  - RF

- Dye marker released with weight is dropped?  RF is better

- Coast guard has very large search swath – they know how to do this

- Red Herring of other transponder

  - 3 min update was telling us whatever we were tracking had a low battery

  - 180 db transponder is not very loud, and Benthos has limited channels

  - Acoustic results were very bad

  - Didn’t consider the transponder wasn’t ours.

  - We considered Charlie’s but they were too old, but didn’t consider USGS or somebody else.

- What worked?

  - 280m dive was a success

  - Vehicle tracked its position

  - Emergency battery pack

  - Detected low-side ground fault

- Argos

  - Modify transmission schedule

  - Procedure to watch transmissions

  - Update procedure

    - Argos

  - Modify checklist

    - Estimated life of argos

Conclusions:

- Abort operations if there are problems with the argos

- More cautious with “risky” dives;

- Needed another RF locator

Action items

- Check with iridium to see if there were transmissions from Tethys when it was on the surface after the dive?

- Investigate fluorescent paints for the

- Are there RF beacons we can easily add?

- Are there any other transponders available that would be unique?

- Investigate default mission changes