LRAUV Deployment Checklist

V1.3b

This page not part of checklist

Checklist goals

- As short as possible, as long as necessary

- Basic instructions included

- Checkboxes/easy completion, minimize writing

- Automatic value checking

- Visual indication of complete/not complete

- Available on mobile platforms

- Not: a user manual, validation tool, foolproof

LRAUV Checklist

Part I: Pre-Deployment Checklist

Vehicle name: Daphne

Date: 10/26/2012

Completed by: Jordan/Thomas

- Vehicle configuration files

  - ___	Closeout checklist completed. Date:    [2012-10-08]

  - ___	Vehicle configuration unchanged from last closeout

Notes:

In water ballast/trim results

  - ___	Neutral buoyancy setting: 450 cc  at density: 1024.739 kg/m3 (normal range: 200cc – 600cc)

  - ___	Mass default setting: 0.2 cm (normal range +/- 1 cm)

  - ___	Mass limits setting:  20 mm fwd / -30 mm aft

Limit within physical stop positions

Configuration, simulation, and vehicle load

  - ___	Enable installed sensors/components

  - ___	Adjust logger.cfg to return shore data  (Reference document TBD)

  - ___	Adjust stop depth and abort depth 125, 150

  - ___	Missions simulated and placed into deployment folder

  - ___	Tag software for release if necessary.

From command line, drill to workspace and execute:

“svn cp . svn+ssh://tethyscode.shore.mbari.org/svn/repos/lrauv/tags/Tethys_YYYY_MM_DD”

From eclipse: right click on LRAUV and select team->Branch/Tag

  - ___	Build and load software.

No application running. From workspace: “make clean;make distclean;make lpc3250”. Then “make copyTo<Vehicle>” (e.g. makeCopyToDaphne)

- Prior to tether disconnect

  - ___ sync and remove logs from vehicle

as tethys@tellum: ~/workspace/LRAUV $ ./Tools/sync-remove-data.sh tethys

  - ___	Run e2fsck

cd /, killall toggleHeartbeat.sh, umount /dev/mmcblk0, e2fsck –f /dev/mmcblk0
reboot to start application

  - ___	Verify no immediate failures after application start

  - ___	Verify revision log output matches revision tagged 9999

e.g. 2012-09-10T04:23:18.911Z,1347250998.911 [SBIT](IMPORTANT): Tethys CM Info: $Rev: 9842

as tethys@tellum: ~/workspace/LRAUV $ svn info

as tethys@tellum: ~/workspace/LRAUV $ ssh tethys “grep -A 1 'CM Info' /mnt/mmc/LRAUV/Logs/latest/syslog”

  - ___	Verify burnwire voltage ~24V

From application prompt: “burn 5”

turn off shore power

  - ___	Verify battery voltage > 16V (15V for primaries) 16.321377 V

on primaries:

maintain control SpeedControl.propOmegaAction 300 rpm

get ThrusterServo.component_voltage

“get platform_battery_voltage”  (“get RudderServo.component_voltage”)

  - ___	Shut down application and “safe” vehicle

as tethys@tellum: ~/workspace/LRAUV $ ./Tools/on-tether.sh tethys

LRAUV Checklist

Part II: Deployment Checklist

- Tether disconnected. Vehicle ready for launch. No application running.

  - ___	Set date/time to GMT from GMT source (e.g. http://wwp.greenwichmeantime.com/ )

“date MMDDhhmmCCYY[.ss]”

  - ___	Set hwclock

“hwclock --systohc”

  - ___	Reboot vehicle to start application

“reboot”

  - ___	SBIT pass

  - ___	No failed components

“failc”

  - ___	IBIT passes outdoors (“ibit”)

Failed due to lack of battery data

From IBIT log output:

    - ___	Confirm stop and abort depths match configuration

    - ___	Confirm battery capacity and voltage thresholds N/A

    - ___	Confirm buoyancy neutral value

    - ___	Confirm battery status adequate for mission

  - ____	Iridium transmission received

  - ____	Argos transmission received

- Vehicle in water.

  - ___	Restart application (“restart application”)

  - ___	No ground fault detected

- Deployment specific configuration changes:

Abnormal changes necessary for deployment success can be listed/checked here (e.g. buoyancyDefault value not 945 cc)

LRAUV Checklist

Part III: Post-Deployment Checklist

- Vehicle recovered

  - ___	Restart logs (“restart logs”)

- Vehicle on tether

  - ___	Safe vehicle and copy logs

From tethys@tellum run: ” $~/workspace/LRAUV/Tools/on-tether.sh <vehicle name>”

  - ___	Once all logs are copied to tellum:/tmp/Logs, copy all at sea (i.e. deployed) logs to deployment folder