Project Methods Outline – LISST-HOLO/SHA Project

Fulton-Bennett, Bellingham, Harvey

LISST-HOLO

Controls:

Run background measurements of MilliQ, FSW

Samples:

Isolate x # copepods (Tigriopus californicus) on mesh filter, place into 50mL FSW

x = 1, 5, 10, 15, 20, 30, 50

For samples with Pseudo-nitzschia australis – add 5mL of known concentration to 45 mL of copepods, for low, medium, and high natural water concentrations

Fill LISST-HOLO chamber with sample

Run LISST-HOLO sampling 0.2 Hz, for 3-15 (TBD) min

Back pipet every few seconds to maintain water movement (no bubbles!)

Filter sample on 60 µm nylon mesh filter

Rinse chamber two times with 50 mL FSW to remove all copepods

Check for escapees on funnel, edge of filter base, stopper, and bench top and re-imprison with tweezers or pipet tip

Place filter in 2mL cryovial, label w/ date, sample #, initials

Freeze sample in liquid nitrogen until later processing

Record storage dewar and colour

Sandwich Hybridization Assay

Build plates

A 	Substrate	1 step Ultra TMB Elisa		fridge

B	Wash E		Wash buffer E			above bench

C	Wash E		Wash buffer E			above bench

D	Conjugate	(note lot #)			fridge

E	Wash E		Wash buffer E			above bench

F	Signal Solution	zoop (0.5 M) or phyto (2 M)	bench top

G	Captures	Pod1951 or auD1		fridge

H	Lysate		Sample (control - lysis buffer)	buffer above bench

Signals

Zoop: 519, 915, 1194

Phyto: PsDs TRD

Probes Pod1951, auD1

Lysis of sample - 0.25 mL of lysate needed for each well (1.9 mL added to each tube, 0.2 mL added to syringe)

Defrost slightly from liquid nitrogen dewar

Turn on hot plate; fill middle four wells on right side with water, heat until ~85°C

Add lysis buffer to tubes (~1.8 mL), 5 min lysis at ~85°C in water bath, vortex for 8 sec before and at 2.5 min (keep lid tight to prevent leakage and lysate loss)

Pour buffer from tube to syringe, add another ~0.4 mL to syringe

Filter into pipet well

Add to wells H w/ multichannel pipet – control wells have lysis buffer

Load prongs onto microarray – record batch number

Run microarray (1 hr 3 min) - set plates to the back and right

Read at 650 nm

Add H2SO4 to wells A, mix by back pipetting (~15 times)

Read at 450 nm

Save and print results w/ notes, data tables, txt conversion