# LRAUV Deployment plan for DAT Estimation test

## Chief Scientists or Engineers

Brian Kieft, Software Engineer, MBARI 
bkieft@mbari.org 
+1-831-775-2070

M Jordan Stanway, Postdoctoral Fellow, MBARI 
mjstanway@mbari.org 
+1-831-775-1960

## Vehicles

### Daphne

- all operations

## Other resources

### Small Boat

- LRAUV launch & recovery operations

### Acoustic Modem

- Standalone Benthos MF acoustic modem with battery pack and compass.

- Rigged to rise 10 meters altitude from an anchor, marked by a Norway float on the surface. Surface expression location is here: http://share.findmespot.com/shared/faces/viewspots.jsp?glId=0ZmngEMkPrkqqqoQ4JmuqyKh9ryqi84jW

- The modem location is:  36.8151, -121.8257

## Preparation

### Daphne

- docking long-nose configuration

  - DAT acoustic head and driving electronics installed

  - single SADM-1 acoustic baffle installed

  - docking connector assembly installed

  - PAR sensor tucked out of way (physical interference with SADM baffle)

- ISUS optional (for supplementary science)

## Location

- On-shelf -- continuous bottom lock for comparison

- near MBARI -- minimal transit time

## Mission

## Deployment instructions

- Prep vehicle for cart launch and transit via boat and launch within 2km of 36.815, -121.826

- Run Maintenance/dat_on.xml and confirm AGC levels and range brought back to shore. Troubleshoot from shipboard DAT if not.

- Transit to deeper water for ballast and trim if necessary (36.81, -121.83)

- run Maintenance/ballast_and_trim.xml

  - Adjust buoyancy and mass parameters:

  - configset VerticalControl.buoyancyNeutral XXX cc persist

  - configset VerticalControl.massDefault Y.YYY cm persist

- run Engineering/DAT_line_with_circles.xml

- NOTE: simulations are showing the possibility of not enough mission time to complete the circles around the modem at the end of the mission. If this occurs, set MissionTimeout to 9 hours instead of 4 when running the mission. Additionally, the variable Repeat can be increased from 3/lap if operators would prefer the mission run longer during the night. Be sure to adjust the MissionTimeout as well if you increase the lap repeat number.

- Mission finished? Run again.

## Post-processing and evaluation

Make sure range and bearing estimates are realistic -- observe AGC and LVL.

Estimate stationary acoustic modem position using ranges only.

Compare angular errors using phase outputs with and without general linear calibration applied.

Consider implementing a look-up table for post-processing angles in an even more general fashion.

## Conclusion

Decide whether to send DAT and noes assembly to Benthos for precise calibration, or to drop efforts on DAT integration indefinitely.