# LRAUV Deployment plan for Compact Models Test

## Chief Scientists or Engineers

M Jordan Stanway, Postdoctoral Fellow, MBARI 
mjstanway@mbari.org 
+1-831-775-1960

## Vehicles

### Daphne

- all operations

## Other resources

### Small Boat

- LRAUV launch & recovery operations

## Preparation

### Daphne

- expansion coefficient (EC) timeseries for recent past in Data/HFRadarModel on vehicle

- set up an hourly cron job from tellum

## Location

- On-shelf -- continuous bottom lock for comparison

- 36.830, -121.870

- 36.875,-121.967

## Mission

## Deployment instructions

- Prep vehicle for cart launch and transit via boat and launch within 0.5km of 36.83,-121.87

- Load Engineering/SurfaceCurrentCompactModel.xml and set NeedCommsTime to 20 minutes.

- Run the mission

- After first navigation using HF currents, note any dangerous navigation mistakes that could ground the vehicle over the next hour. If noted, issue a stop.

- Assuming safe navigation, set NeedCommsTime to 60 minutes and run the mission.

- After at least one lap and at a convenient time for operators, transit to 36.83,-121.87 and run Maintenance/ballast_and_trim.xml

  - Adjust buoyancy and mass parameters:

  - configset VerticalControl.buoyancyNeutral XXX cc persist

  - configset VerticalControl.massDefault Y.YYY cm persist

- Continue running SurfaceCurrentCompactModel mission

## Post-processing and evaluation

## Conclusion