# LRAUV Deployment plan for Long Body Test

## Chief Scientists or Engineers

Rob McEwen
rob@mbari.org 
+1-831-775-1863

## Vehicles

### Tethys

- all operations

## Other resources

### Small Boat

- LRAUV launch & recovery operations

## Preparation

### Tethys

- Fitted with long body expansion

## Location

- 36.150, -121.869

- 36.829, -121.886

- 36.816, -121.883

## Mission

## Deployment instructions

- Prep vehicle for cart launch and transit via boat and launch at 36.805,-121.825

- Run transit to 36.835,-121.830 with timeout 30 minutes

- Verify navigation to the North and continue to waypoint above.

- Run transit to 36.835,-121.872 with comms timeout 30 minutes

- Run ballast and trim at 35 meters

- Run Engineering/triangle_control_speed_compare.xml

Note: Summary for the mission

This is a long mission that does four laps around a triangle contained

within the four pre-existing waypoings BoxWpt.  The entire mission takes

about 10 hours.

The objective is to determine power versus range for various

configurations.  A secondary objective is to test control loop behavior and

interaction.

Each lap has three speeds; 1.0, .75 and .60 m/s.  The triangle is shaped so

that each leg lasts approximately 40 minutes.

Lap1: Mass fixed; Buoyancy fixed; Elevator deadband zeroed.

Lap2: Mass fixed; Buoyancy fixed; Elevator deadband 0.5 deg.

Lap3: Mass fixed; Buoyancy control on; Elevator deadband zeroed.

Lap4: Mass fixed; Buoyancy control on; Elevator deadband 0.5 deg.

## Post-processing and evaluation

## Conclusion