# LRAUV Deployment plan for Compact Models Test

## Chief Scientists or Engineers

M Jordan Stanway, Postdoctoral Fellow, MBARI 
mjstanway@mbari.org 
+1-831-775-1960

## Vehicles

### Daphne

- all operations

## Other resources

### Small Boat

- LRAUV launch & recovery operations

### Acoustic Modem

- Standalone Benthos MF acoustic modem with battery pack and compass.

- Rigged to rise 10 meters altitude from an anchor, marked by a Norway float on the surface. Surface expression location is here: http://share.findmespot.com/shared/faces/viewspots.jsp?glId=0ZmngEMkPrkqqqoQ4JmuqyKh9ryqi84jW

- The modem location is:  36.81535, -121.82619

## Preparation

### Daphne

- docking long-nose configuration

  - DAT acoustic head and driving electronics installed

## Location

- Within 1km of modem location

## Mission

## Deployment instructions

- Prep vehicle for cart launch and transit via boat and launch at least 1 km from shore.

- Run transit mission to 36.814, -121.8277 Run the mission

- Run Maintenance/dat_on.xml and verify range data comes back to shore

- Run Maintenance/ballast_and_trim.xml. Move further West if deeper water is needed.

  - Adjust buoyancy and mass parameters:

  - configset VerticalControl.buoyancyNeutral XXX cc persist

  - configset VerticalControl.massDefault Y.YYY cm persist

- Run Engineering/Homing_pursuit.xml. The default timeouts are very short

- Run Engineering/DAT_line_with_circles.xml with default.

- Be prepared to run Engineering/Homing_pursuit with a longer WaitDuration and a higher lap count.

## Post-processing and evaluation

## Conclusion