# LRAUV Deployment plan for Compact Models Test

## Chief Scientists or Engineers

Yanwu Zhang

## Vehicles

### Tethys

- all operations

## Other resources

### Small Boat

- LRAUV launch & recovery operations

## Preparation

### Tethys

- Short nose standard configuration with Eco Puck and CTD

## Location

- Within the box bounded by:

  - Latitudes: 36.88 and 36.7

  - Longitudes: -122.3 and to within 5km of shore to the East

## Mission

- Science/canon_2Dfront.xml

## Deployment instructions

- Launch vehicle at least 1 km from shore.

- Run transit mission to 36.82, -121.855

- Run Maintenance/ballast_and_trim.xml at 50m depth

  - Adjust buoyancy and mass parameters:

  - configset VerticalControl.buoyancyNeutral XXX cc persist

  - configset VerticalControl.massDefault Y.YYY cm persist

- Run Science/canon_2Dfront.xml

- If we have an opportunity to run keepstation in deep water, it would be a good time to test the new buoyancy system at 100 m depth. Be sure to temporarily adjust stop/abort depth settings prior to running.

## Post-processing and evaluation

## Conclusion