# LRAUV Deployment plan for Homing Pursuit Wave Glider deployment

## Chief Scientists or Engineers

Brett Hobson

## Vehicles

### Daphne

- all operations

## Other resources

- Paragon for launch & recovery

- MF modem, deck box, and long cable

- GPS position logging to laptop

- Power supply for GPS

## Preparation

### Daphne

- Long nose with DAT

## Location

  - Initially in the vicinity of 36 47.61, -121 50.74

  - Then tracking a circle around the Paragon’s motion to the West

## Mission

- Science/CANON_LST_Track.xml

## Deployment instructions

- Launch vehicle near 36 47.61, -121 50.74 (previous LST test site)

- Deploy topside modem into water at least 15m deep. Make sure address is configured to match ContactLabel

- Activate GPS logging

- Run mission and wait for active tracking (will hear on the topside speaker)

- Move boat slowly or drift allowing AUV to follow

## Post-processing and evaluation

- Compare boat position with vehicle position.

## Conclusion