2014 LRAUV Configurations for CANON

- Daphne

  - Mission: Circles Makai, or CPF using USBL collecting contextual bio/geo data.  Position tracked by Tiny (WaveGlider), responds to a-comms command to continue mission

  - Instruments:

    - Linquest DVL Serial #013245, LCB B5 (5A)

    - Benthos DAT Serial # XXXX, LCB B6

    - Wetlabs Puck Serial # XXXX, LCB B3

    - NBOSI CTD Serial # XXXX, LCB B4

    - Turner Cyclops Rhodamine , Serial # XXXX ,LCB B0 (A/D)

    - Sparton Compass, Serial # XXXX , LCB B7

    - Aanderra Optode Serial # XXXX, not used

    - MBARI ISUS Serial # XXXX, LCB B1

    - MBARI SCIPI, Serial # XXXX, LCB B2

    - OPEN LCB:  none

  - Comms

    - Freewave, Serial # XXXX  LCB A2

    - NAL 9602, Serial # XXXX,  LCB A1

    - Applied Acoustic transducer, Serial 2093573, standalone with 9v batteries, HPR3 Channel 6 (24.038 interrogate, 27.173 kHz reply)

    - Desert Star Argos, # 804, ID 136622, standalone battery powered

    - Telonics VHF beacon, Serial #678951A,159.65 MHz, Standalone battery powered

  - Notes:

- Tethys

  - Mission: Patch tracking with rhodamine sensor, area recon, high resolution data upload (including ADCP?) through hotspot

  - Instruments:

    - Rowe ADCP/DVL Serial #013245, LCB B5 (5A)

    - Wetlabs Puck Serial # XXXX, LCB B3

    - Turner Cyclops Rhodamine, Serial # XXXXXX, LCB B0, using PAR port and wiring

    - NBOSI CTD Serial #XXXXX, LCB B4

    - PNI Compass (in Rowe), Serial #XXXXX, LCB B7

    - Aanderra 4830 Optode Serial # 240, LCB B2

    - MBARI ISUS Serial #XXXX , LCB B1

    - OPEN LCB: B6

  - Comms

    - CDMA radio, Serial #XXX LCB A2, IP 166.248.105.035, 831-710-7016

    - NAL 9602, Serial #XXX, IMEI 30023401186620, LCB A1

    - Benthos transponder, Serial # 49897, 26/29 kHz

    - Desert Star Argos, Serial # 1022, ID 136621

    - Telonics VHF beacon, Serial #678950A, 159.55 MHz

  - Notes:

- Makai

  - Mission: Sampler/ Patch center marker Vehicle, holding at density 1025.5, or temperature

  - Instruments

    - Linquest DVL Serial #,10089, LCB B5 (5A)

    - Benthos modem Serial #XXX, LCB B6

    - Wetlabs Puck Serial #XXX, LCB B3

    - NBOSI CTD Serial #XXX, LCB B4

    - Turner Cyclops Rhodamine, Serial #XXX, LCB B0

    - Sparton Compass, Serial #XXX LCB B7

    - Aanderra Optode Serial #XXX, LCB B2

    - OPEN LCB: B1

  - Comms

    - Cradlepoint radio, Serial #XXX LCB A2

    - NAL 9602, Serial #XXX, LCB A1

    - Benthos transponder Serial #XXX, 26/29 kHz, standalone battery

    - Desert Star Argos Serial # 939, Argos ID 136623, standalone battery

    - Telonics VHF beacon, Serial #XXX, XXX MHz, standalone battery

  - Notes: Using MB 2.0, old LCB backplane, new battery pack, new umbilical/charger box, though no shore-power