2015 LRAUV Configurations for CANON

- Daphne

  - Mission: Circles drifting AUV using DAT while collecting contextual bio/geo data.  Position tracked by Tiny (WaveGlider), responds to a-comms command to continue mission.  Can also drift while being tracked by WaveGlider, but no other vehicles can circle

  - Instruments: C, T, DO, Chl, OBS, PAR, Nitrate

    - NBOSI CTD Serial # 212/292 LCB A4 36-232-5-

    - Wetlabs Puck Serial # 935, LCB A6 39-232-12-

    - PAR , Serial # 8455, LCB A0 50-0-0-A/D

    - Aanderra Optode Serial # 240 LCB A2 30-232-12

    - MBARI ISUS Serial # XXXX, LCB A1 19-232-Batt-Discrete

    - Linquest DVL Serial #013245, LCB A5 46-232-24V-4A

    - Sparton Compass, Serial # XXXX , LCB A7 34-TTL-3.3-

  - Comms

    - Cradlepoint Cell/WiFi # XXXX  LCB B2 – 37-OFFBOARD-12V

    - NAL 9602, Serial # XXXX,  LCB B1 35-232-5-

    - Applied Acoustic transducer, Serial 2093573, standalone with 9v batteries, HPR3 Channel 6 (24.038 interrogate, 27.173 kHz reply) HPR3 Channel 6 (24.038 interrogate, 27.173 kHz reply)

    - Desert Star Argos, # 804, ID 136622 standalone battery powered

    - Telonic VHF beacon#678951A,159.65 MHz, Standalone battery powered

    - Benthos DAT Serial #43987, LCB A6 45-232-24-4A

  - Notes:

    - Using MB 2.5, old LCB backplane, new battery pack

- Tethys

  - Mission: Carries CANON Sampler and can drift while being tracked by the WaveGlider and circled by Daphne

  - Instruments: C, T, Chl, OBS, PAR and CANON Sampler,  DO, Vemco tag receiver

    - Wetlabs Puck Serial # 634, LCB B3 16-232-12, front port 2

    - NBOSI CTD Serial #159/199, LCB B4 22-232-5, , front port 3

    - Sparton Compass, Serial, LCB B7 20-TTL-3V, shares front port 7 with DVL

    - Aanderra Optode Serial # 14, LCB B2 23-232-12, , front port 6

    - Vemco fish tag receiver VR2C-69 kHzSerial # 450149 , share power with Puck (B3) Comms TTY/TX1/VRRT2/ J6, ISUS tray, front port 1

    - Linquest DVL Serial #,10089, B5 29-232-24-5V, shares front port 7 with compass

    - PAR #8454, LCB: B0 48-0-0-A/D, , front port 4

    - CANON Sampler LCB: B6 38/232/12 , front port 8

  - Comms

    - Cradlepoint radio (cell/WiFi), Serial #MM140253200389, LCB A2 51-Offb oard-12v

    - NAL 9602, Serial #300234011866620/D04VR3 LCB A1 13-232-5v

    - Benthos transponder 26/29 kHz , Serial # 49897, 26/29 kHz

    - Desert Star Argos, Serial # 1022, ID 136621

    - Telonics VHF beacon, Serial #678950A, 159.55 MHz

    - Benthos modem Serial #64093 PCB, LCB B1 83/232/24/5A,  front port 5

    - MCV MUX #0003

  - Notes:  Using MB 2.5, old LCB backplane, new battery pack, new

- Makai

  - Mission: Can run front/patch tracking and be a drifter tracked by the Wave Glider

  - Instruments: CT, DO, Chl, OBS, PAR, Rowe Current Profiling

    - NBOSI CTD Serial #100/54 LCB B1 69-232-5-

    - PNI Compass (in Rowe), Serial #XXXXX #XXX LCB B2 74-TTL-3.3-

    - NAL 9602, 300234060751590, LCB B3 67-232-5-

    - Wetlabs Puck Serial #1073, LCB B4 70-232-12-

    - Aanderaa Optode Serial #241, LCB B5  71-232-12-

    - Keller Depth Sensor, Serial 51, A0 47-analog

    - Rowe ADCP/DVL Serial #013245, LCB A1 73-232-24-5A

    - PAR #8594, LCB B0 70 Analog

  - Comms

    - Benthos Acoustic Transponder Serial #49901, 26/29 kHz

    - Cradlepoint radio, Serial #130489900096 LCB B6 75--12-

    - Desert Star Argos Serial # 939, Argos ID 136623

    - Telonics VHF beacon, Serial #678952, 159.75 MHz

    - Benthos Acoustic modem, Serial #64093 PCB, LCB A2 72-232-24-

    - MCV mux serial number 005

  - Notes: Using MB 2.5, old LCB backplane, new battery pack, new

  - 970g trim weight in nose and 120g in tail at thruster motor